3GPP TSG-RAN WG4 Meeting #113	R4-2418044

Orlando, Florida, US, 18th – 22nd November 2024

Source:	Nokia

Title:	Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements

Agenda item:	7.14.2

Document for:	Information

Introduction

During RAN#104 the study item on Spatial Channel Model was agreed [1] and it started in RAN4#112.
This informative contribution is intended to help other interested parties to implement a TS 38.827 based CDL channel model and enable alignment with our version. Furthermore, this tdoc serves as the companion contribution to our FS_NR_demod_SCM discussion [2] and contains the complete context and extended results for the simulation results used in the discussion.
Previous results can be found in [3] and [4].

In summary, this contribution contains the following parts:

- Full set of updated CDL/TDL/TDL+antCorr simulation results.

- TP for 38.827 based CDL channel model (“CDL”) updated with agreements from RAN4#112 and RAN4#112bis.

- Implementation notes (continuation from prior meeting).

No observations nor proposals are made in this simulation contribution.

Simulation results

Simulation setup

All simulation results in this tdoc and our companion discussion tdoc [2] use the configuration detailed in Table 1, unless explicitly stated otherwise.

Table 1: Proposed simulation configuration for channel model alignment in study (8T/8R/8-layer reference case).

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Simulated Channel |  |  | PDSCH |
| Duplex mode |  |  | FDD |
| FR / Carrier frequency |  |  | FR1 / 3.5 GHz |
| UE receiver type |  |  | Simple SU MMSE |
| Antenna configuration (BB branches) |  |  | 8 Tx / 8 Rx |
| Waveform |  |  | CP-OFDM with normal CP |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| MCS |  |  | 17 (64 QAM table) |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static (N/A) |
|  | PRB bundling size |  | 2 (N/A) |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 (N/A) |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
|  | Transmission Rank |  | 8 for 2CW |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 (for rank > 4) |
|  | DMRS ports indexes |  | {1000-1007} for Rank8 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | RI Restriction |  | 10000000 for rank 8 |
| PDSCH & PDSCH DMRS Precoding | Common parameters |  | Wideband (but shouldn’t matter) |
|  | Fixed |  | i11=2, i12=0, i2=1 (i12 is fixed 0 in ULA with this configuration. All indices counting from “0”) |
|  | Random |  | Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with wideband granularity |
| NZP CSI-RS for CSI acquisition (Only used for 4Tx PMI results. No TRS, TOE based on DMRS.) | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 4, (0) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (13) |
|  | CSI-RS periodicity and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 4 |
| Analog, RF, fixed point impairments |  |  | None. Ideal results only. (No TO, no FO, no PA model, no TxEVM, no RxEVM, no IQ imbalance, etc.) |
| Number of HARQ Processes |  |  | 4 |
| Maximum HARQ transmissions |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 1 (Change w.r.t. initial contribtuions) |
| Channels |  |  | CDL-C UMa NLOS (“CDLC”) 3/30kph UE velocity, 365ns RMS DS, in the TR 38.827 based version detailed in Section 3. I.e., - Reiver uses n/2x1 cross-pol ULA with lambda/2 distance, i.e., [++…+], with omni AE radiation pattern. - Cosine/38.901 AE pattern on transmitter side. - CDL ray paring and polarization scalar initial phases fixed. TDLC300-100 (ULA) Low, (30kph DUT speed) TDLC300-100 (ULA) MedA, (30kph DUT speed) Further TDL antenna correlation matrices as defined in 38.101-4 |

Notes on SNR/SINR definitions: 	
Operating points and performance KPIs are usually defined in terms of “SNR”. We use the RAN4 demodulation definition of “SNR” (see for example TS 38.101-4, section 4.4.2). It can be described as the average signal over noise power ratio as measured at the BB input branches of the equalizer/demodulator, averaged over each demodulator input branch and RE, where the signal exists, i.e., it is the pre-equalizer SNR per Rx branch.	
RAN4 demod also uses a similar definition of pre-equalizer SINR per Rx branch (see for example TS 38.101-4 section 4.4.5), with the interference power added to the noise power. However, we will refrain from using SINR in this sense and opt to call all pre-EQ S(I)NR relationships simply “SNR”.

CDL based SCM

Precoder selection and DoA for 827 CDL

8Tx8Rx, Rank 1 AoAs

| typeI PMI | Port selection precoder |
| --- | --- |

Figure 4: TR 38.827 based CDLC Uma Bartlett DoA analysis vs. precoding vector/layer (x-axis is AoA).
4x1 Xpol ULA receiver assumption, 8Tx8Rx, Rank 1 AoAs.

8Tx8Rx, Rank 8 AoAs

Figure 18: TR 38.827 based CDLC Uma Bartlett DoA analysis vs. precoding vector/layer (x-axis is AoA).
4x1 Xpol ULA receiver assumption, 8Tx8Rx, Rank 8 AoAs.

Convergence speed/testing time

CDLC @ 22dB (52% TPUT) / TDLClow @ 19dB (50% TPUT) / TDLCmedA @ 50dB/MCS7 (32% TPUT)

Figure 5: Simulation convergence trials: 8T8R 2CW (MCS17 both) full rank, SU-MMSE receiver, Random precoding vs. simulated slots (at 50% TPUT operating point target).

Numerical stability for 827 CDL

Different starting seeds

Figure 6: CDLC 365-100 differences between randomness seeds.

Redrawing scalar random initial phase terms

| Seed 1 | Seed 2 |
| --- | --- |

Figure 7: CDLC 365-100 differences from redrawing scalar random initial phase terms.

Implementing all clusters as weak clusters (no sub-clusters)

Figure 6: Impact of 827 sub-cluster rules (using 827 AAV).

CDL comparisons and observations

SU PDSCH: 2CW

827 CDL vs. legacy

|  | Cannot achieve meaningful TPUT |  |
| --- | --- | --- |
| TDLC300-100 low (MCS17) | TDLC300-100 MedA (MCS17) | 827 CDLC UMa (MCS17) |
|  | Cannot achieve 70% TPUT |  |
| TDLC300-100 low (MCS13) (Fixed PMI from histogram max) | TDLC300-100 XP high (MCS13) |  |
|  | TDLC300-100 MedA (MCS7) | 827 CDLC UMa (MCS7) |

Figure 12: 2CW Legacy vs. 827 CDL: (MCS17 top, MCS13 mid, MCS7 bottom) full rank relative TPUT, SU-MMSE receiver, Fixed and random precoding vs. pre-EQ SNR.

CDL AAV variants

| 70% TPUT Fixed PMI CW0 16.7 dB Fixed PMI CW1 24.0 dB Random PMI CW0 11.1 dB Random PMI CW1 20.2 dB | 70% TP Fixed PMI CW0 30.9 dB Fixed PMI CW1 15.3 dB Random PMI CW0 23.4 dB Random PMI CW1 32.0 dB | 70% TPUT Fixed PMI CW0 26.5 dB Fixed PMI CW1 15.2 dB Random PMI CW0 20.6 dB Random PMI CW1 26.8 dB |
| --- | --- | --- |
| 827 CDLC UMa | 827 CDLC UMa (downtilt AAV) | 827 CDLC UMa (no AAV) |

Figure 13: CDL Candidates: (MCS13) full rank relative TPUT, SU-MMSE receiver, Fixed and random precoding vs. pre-EQ SNR.

TPUT scaling and precoder effects vs. AE correlation in CDLC

| 827 CDLC UMa ULA lambda/2 | 827 CDLC UMa ULA lambda/4 | 827 CDLC UMa ULA lambda/8 |
| --- | --- | --- |

Figure 4: 8T8R, 2CW (MCS17 both) full rank relative TPUT, SU-MMSE receiver, Fixed and random precoding vs. pre-EQ SNR, for increasing CDL ULA correlation (AE distances reducing).

TPUT scaling and precoder effects vs. speed in different channel models

| TDLC300-100 low (30/50kph) | 827 CDLC UMa (30/50kph) |
| --- | --- |

Figure 4: 8T8R, 2CW (MCS17 both) full rank relative TPUT, SU-MMSE receiver, Fixed and random precoding vs. pre-EQ SNR for differing speeds and channel models. FOC off.

SU PDSCH: Sensitivity to receiver implementations

| TDLC300-100 low | TDLC300-100 MedA (MCS 7) |
| --- | --- |
| 827 CDLC UMa ULA |  |

Figure 15: 4T4R, 1CW (MCS17) full rank, random precoding, Differing SDM receiver implementation. 
Relative TPUT vs. pre-EQ SNR.

SU PMI: PMI profiles

4Tx4Rx 4Layers

| TDLC300-10 low SNR=15dB => CW Throughput = 96.07% | TDLC300-10 XP high SNR=25dB => CW Throughput = 91.38% |
| --- | --- |
| 827 CDLC UMa, DS 365ns, 10 Hz Doppler, ULA Rx SNR=14dB => CW Throughput = 92.35% |  |

Figure 16: Histograms of PMI feedback choices under different channel models (2000 CSI reports).
4T4R, 1CW, MCS13, full rank, PMI chosen based on (non-precoded) CSI-RS, N1=2, N2=1, O1=4, O2=1.

8Tx8Rx 8Layers

| TDLC300-10 low SNR=16dB => Throughput = CW0 90.3% CW1 90.3% | TDLC300-10 XP high SNR=40dB => Throughput = CW0 2%, CW1 73.6% |
| --- | --- |
| 827 CDLC UMa ULA, 10Hz SNR=22dB => Throughput = CW0 74.9%, CW1 78.15% | 827 CDLC UMa ULA, 10Hz SNR=25dB => Throughput = CW0 87.4%, CW1 98.5 |

Figure 17: Histograms of PMI feedback choices under different channel models (2000 CSI reports).
8T8R, 1CW, MCS13, full rank, PMI chosen based on (non-precoded) CSI-RS, N1=4, N2=1, O1=4, O2=1.

SU PMI: 4Rx and 2/4 layer

CDL AAV variations

| Follow PMI 90% TP 5.3dB Random PMI TP @5.3 dB 76.6% Γ = 1.18 | Follow PMI 90% TP 4.82dB Random PMI TP @4.81 dB 58.3% Γ = 1.54 | Follow PMI 90% TP 4.81dB Random PMI TP @4.81 dB 55.8% Γ = 1.61 |
| --- | --- | --- |
| 827 CDLC UMa (rank 2) | 827 CDLC UMa (downtilt AAV, rnk2) | 827 CDLC UMa (no AAV, rank 2) |
| Follow PMI 90% TP 13.24dB Random PMI TP @13.24 dB 66.6% Γ = 1.35 | Follow PMI 90% TP 13.92dB Random PMI TP @13.39 dB 52.6% Γ = 1.711 | Follow PMI 90% TP 13.39dB Random PMI TP @13.39 dB 53.2% Γ = 1.69 |
| 827 CDLC UMa (rank 4) | 827 CDLC UMa (downtilt AAV, rnk4) | 827 CDLC UMa (no AAV, rank 4) |

Figure 18: Follow vs. random PMI in AAV alternatives.
8T4R, 1CW, MCS13, 10Hz/3kph, rank2 (top) and rank 4 (bottom), typeI CB with N1=4, N2=1, O1=4, O2=1.

Legacy channel models

| Follow PMI 90% TP 5.33dB Random PMI TP @5.33 dB 68.8% Γ = 1.31 | Follow PMI 90% TP 7.08dB Random PMI TP @7.08 dB 26.9% Γ = 3.35 |
| --- | --- |
| TDLC300-10 low (rank 2) | TDLC300-10 XP high (rank 2) |
| Follow PMI 90% TP 13.73dB Random PMI TP @13.73 dB 67.3% Γ = 1.34 | Follow PMI 90% TP 31.36dB Random PMI TP @31.36 dB 52.2% Γ = 1.72 |
| TDLC300-10 low (rank 4) | TDLC300-10 XP high (rank 4) |

Figure 18: Follow vs. random PMI in legacy channels.
8T4R, 1CW, MCS13, 10Hz/3kph, rank2 (top) and rank 4 (bottom), typeI CB with N1=4, N2=1, O1=4, O2=1.

MU PDSCH: 2+2 layers with IRC, E-IRC, and R-ML

| TDLC300-100 low | TDLC300-100 XP Med |
| --- | --- |
| 827 CDLC UMa ULA |  |

Figure 20: Differing advanced receivers (R-ML, E-IRC, IRC) in presence of co-scheduled UE.
4T4R, 1CW (MCS13+MCS4), rank 2+2, MCS4 interference, orthogonal and random MU precoding.

TP for 38.827 based CDL channel model (SU version)

This section gives a description of a TR38.827 based SU CDL channel model (often referred to as “CDL” or “827 CDL”) in the form of a potential TP for 38.101-4, which was used to derive all our results in our contributions. Note that explanatory parts of TR38.827, that are not needed for implementation, have been removed, and the extension for conducted/virtual cable test setup have been added.	
Minor modifications based on discussions and agreements in RAN4#112/112bis have been included, where appropriate

Interested contributors are encouraged to implement 827 CDL based on the following TP and exchange with Nokia to achieve alignment.

<Start of TP>

## B.2.X1	Cluster Delay Line Channel Model

### B.2.X1.Y1	General

The channel model is based on the description and clarifications given in TR 38.827.

For NR FR1 and FR2 MIMO testing, the number of samples for sequence length at each testing point is FFS.

### B.2.X1.Y1	Channel Models

This section describes amendments to the stepwise procedure of the CDL subclause 7.7.1 in TR38.901 for generating fast fading radio channel realizations. This channel model methodology considers non-Jakes spectrum with the multi-path fading propagation conditions between the gNB emulator and DUT modelled based on Clustered Delay Line (CDL) methodology.

The tables provided in section B.2.X1.Y1 are to be used directly in the procedure in TR38.901 for generating fast fading radio channel realizations. The given coefficients have been scaled to achieve the following target delay spread values

Table 7.2-1. Target delay spread values.

| Frequency | Scenario | DSdesired |
| --- | --- | --- |
| FR1 | UMi | 100 ns |
| FR1 | UMa | 365 ns |
| FR2 | UMi | 60 ns |
| FR2 | InO | 30 ns |

Similarly, the cluster-wise rms azimuth spread of arrival angles (cluster ASA, cASA), and similar spreads in the tables, are scaled to achieve the following desired AS, for the given cluster AOA (n,AOA) and similar azimuth/zenith angles (Tables 7.7.1.1 – 7.7.1.5 of TR 38.901).

Table 7.2-4: Desired AS for UMi and UMa at 3.5 GHz (FR1)

| Model | ASdesiredASdesiredASdesired [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| UMi NLOS (CDL-A, B, C) | 23.9751 | 57.2457 | 0.7762 | 7.8320 |
| UMi LOS (CDL-D, E) | 15.0432 | 47.6149 | 0.6166 | 4.6204 |
| UMa NLOS (CDL-A, B, C) | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| UMa LOS (CDL-D, E) | 14.0180 | 64.5654 | 3.4674 | 8.9125 |
| Note: For UMa frequency fc = 6 as stated in [2], and other parameters hUMa = 25, hUMi = 10, hUT = 1.5, and D2D = 100. |  |  |  |  |

Once the channel realization procedure arrives at the step of coupling the AOD angles to AOA angles within a cluster n (step 2). Instead of random procedure, the coupling shall be performed using the fixed coupling pattern specified in Table 7.2-6. The same fixed coupling pattern is applied for all clusters n.

Table 7.2-6: Fixed coupling pattern of ray angles to be applied for each cluster

|  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 12 | 5 | 10 | 8 | 11 | 16 | 14 | 18 | 9 | 20 | 4 | 2 | 15 | 7 | 13 | 19 | 17 | 3 | 1 |
|  | 20 | 9 | 12 | 1 | 13 | 18 | 10 | 4 | 8 | 2 | 6 | 14 | 11 | 19 | 7 | 3 | 17 | 5 | 15 | 16 |
|  | 2 | 16 | 3 | 11 | 18 | 9 | 5 | 17 | 4 | 19 | 15 | 20 | 13 | 7 | 10 | 1 | 8 | 12 | 6 | 14 |
|  | 15 | 18 | 13 | 1 | 12 | 9 | 6 | 7 | 5 | 3 | 2 | 8 | 14 | 17 | 19 | 16 | 11 | 20 | 10 | 4 |

In the next steps, the linear cross polarization power ratios (XPR) are calculated for each ray m of each cluster n as

,	(7.2-7)

where X is the per-cluster XPR in dB from the tables provided in section B.2.X1.Y1.

The gNB beam pattern including the assumptions for gNB antenna for definitions and symbols of subclause 7.3 of TR38.901 for FR1 and FR2 are summarized in Table 7.2-7.

Table 7.2-7: BS Antenna Parameters

| Parameter description | Symbol | Parameter value |  |  |
| --- | --- | --- | --- | --- |
|  |  | FR1 ≤2.5GHz | FR1 >2.5GHz | FR2 |
| Antenna panels in vertical dimension | Mg | 1 | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 | 1 |
| Elements per panel in vertical dimension | Me | 4 | 8 | 8 |
| Elements per panel in horizontal dimension | Ne | 8 | 8 | 16 |
| Number of polarizations per panel | P | 2 | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 | 0.5 |

Antenna element radiation patterns, including orientation of the element main polarization components as well as orientation of the antenna array for both FR1 and FR2 are as in the example pattern in Table 7.3-1 of TR38.901. The antenna element has ±45 polarization components and the radiation pattern parameters are 3dB = 65, 3dB = 65, Amax = 30dB, SLAv = 30dB, GE,max =8 dBi.

Note this chosen AE pattern may impact the RMS delay spread scaling and necessitates coordinate transformations as defined in TR 38.901 section 7.3.2, but this has not been taken into account for deriving the tables provided in section B.2.X1.Y1.

The BS antenna panel is oriented to have 10 of mechanical tilt with respect to the horizon (10 rotation around the y-axis).

It is assumed the co-polarized elements of the array are combined to a single RF port, i.e. they compose an antenna array that can form beams by setting certain weights per element. Weight vector for the first polarization and for the second polarization is

α1,1…αMe,Ne=1MeNeexp-j2πrtx,max T ∙ dtx,me,neλ0∈C1×MeNe,	(7.2-8)

where dtx,me,ne is the location vector of transmit antenna element me=1,…,Me and ne=1,…,Ne, and rtx,max is a spherical unit vector denoting the target beam direction. Note that the target beam directions, directions of departure (DoD) angles of the transmitter array, i.e., ZoD and AoD, and the locations of antenna elements, are referenced to the global coordinate system (GCS). Determination of beam directions rtx,max is done as follows:

-	For FR1: A grid of 60 fixed beams is constructed to a grid of five elevation angles from –20 to +20 with 10 steps and 12 azimuth angles from –80 to +80 with ~15 steps；

For FR1 4x4, the two strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model. These beams should have different azimuth directions and can provide the highest receive power for UE.

For FR1 8x8, the four strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model. These beams should have different azimuth directions and can provide the highest receive power for UE.

In detail, beam directions for channels model given in Section B.2.X1.Y1 are

-	For UMa CDL-C, the beam directions in the global coordinate system are:

-	Strongest beam: AoD: -7.27°, ZoD: 100°

-	2nd strongest beam: AoD: -21.82°, ZoD: 100°

-	3rd strongest beam: AoD: -36.36°, ZoD: 100°

-	4th strongest beam: AoD: -50.91°, ZoD: 100°

For the UE it is assumed that RF ports map directly to receive antenna elements, where drx,u is the location vector of receive antenna elements, which are configured as a ULA with Cross-pol (1, n/2, 2), i.e. [+ + + +] for n=8, antenna elements with omni radiation pattern, 0.5 spacing, and the AE slant is rotated 45deg to the transmitter polarizations. Note that “n” is the number of Rx elements, and the directions of arrival (DoA) angles of the receiver array, i.e., ZoA and AoA, are referenced to the GCS.

The UE antenna panel is oriented to point towards the front of the BS panel without any mechanical tilt with respect to the horizon (no rotation around the y-axis).

The random initial phases  are not used for the different polarization combinations (θθ, θϕ, ϕθ, ϕϕ). Instead, a fixed and pre-defined set of initial phases Φmθθ,Φmθϕ,Φmϕθ,Φmϕϕ of Table 7.2-8 and a scalar random initial phase term Φn,m~Uni(-π,π) is used for each ray m of each cluster n.

Table 7.2-8: Fixed initial phases for 2x2 polarization matrices. These values are drawn from uniform distribution

| m | Φmθθ [rad] | Φmϕθ [rad] | Φmθϕ [rad] | Φmϕϕ [rad] |
| --- | --- | --- | --- | --- |
| 1 | 1.7609 | -0.6928 | -1.6230 | -0.6037 |
| 2 | -2.5356 | -2.3124 | 2.7775 | 2.8660 |
| 3 | 0.4725 | -2.7660 | -1.6664 | -0.9226 |
| 4 | 2.0181 | -3.0448 | -2.8713 | -2.0798 |
| 5 | 0.9369 | 1.4560 | 0.9283 | -0.3084 |
| 6 | 0.2954 | -1.2798 | 1.5375 | -1.9544 |
| 7 | 1.1735 | -1.9886 | -0.8263 | 0.7893 |
| 8 | 1.7607 | -2.6319 | 2.6979 | 1.7324 |
| 9 | -0.0830 | -0.4030 | -0.3344 | -1.2167 |
| 10 | 0.0535 | 0.0677 | 1.9957 | 1.8525 |
| 11 | 0.9068 | -0.7627 | 1.9577 | 0.2062 |
| 12 | -0.9379 | 2.7583 | 2.3621 | 0.3151 |
| 13 | 0.7695 | 0.5469 | -1.8363 | -1.2488 |
| 14 | -0.1827 | -1.6934 | 2.1634 | -1.9179 |
| 15 | -1.7221 | -2.0690 | -1.7111 | -0.4040 |
| 16 | -1.1869 | 2.6602 | -0.4385 | -1.9804 |
| 17 | 2.5439 | 3.0143 | -0.3841 | -2.4434 |
| 18 | -1.5201 | -0.5735 | 0.5962 | -1.4941 |
| 19 | 0.6462 | 1.3271 | -1.7483 | -2.4038 |
| 20 | -1.2775 | -1.1386 | -0.4765 | 0.0494 |

To determine the channel all clusters are treated as "weaker cluster", i.e., no further sub-clusters in delay should be generated. The BS beamforming weights αme,neαme,nedefined in Equation 7.2-8 for antenna elements are used and the BS antenna signals are summed for BS beamforming. The BS transmits downlink signals with S beams. Index s=1,…,S denotes the formed beam index. Each beam may have different and thus the beamforming weight of eq. (7.2-8) becomes specific for index s as αs,me,ne; it should be noted though that there are always two orthogonally polarized beams to the same direction. Here, the random initial phases Φn,m are used for sub-paths, but not for the different polarization combinations (θθ, θϕ, ϕθ, ϕϕ). The channel coefficient for time instant t, Rx antenna/beam u, Tx beam s, and cluster n is defined by the following equations. They apply for the NLOS clusters:

Hu,s,nNLOSt=PnMm=1Mne=1Neme=1MeFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕFtx,s,me,ne,θθn,m,ZOD,φn,m,AODFtx,s,me,ne,φθn,m,ZOD,φn,m,AOD

αs,me,neexpjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,s,me,neλ0expj2πrrx,n,mT.vλ0t,	(7.2-9)

where Ftx,s,me,ne,θ, Ftx,s,me,ne,φ, and dtx,s,me,ne are the theta and phi polarized radiation patterns and the position vector of the BS antenna element me,ne of sub-array s, respectively. Symbols Frx,u,θ , Frx,u,ϕ, , , and , are determined as in TR 38.901, which includes the generation of AOA (ϕn,m,AOA), AOD (), ZOA (), and ZOD () to derive  and . The velocity vector is determined as

(7.2-11)

UE velocity v is defined as follows: 30km/h for FR1 vs 3 km/h (Indoor Office) and 12 km/h (UMi) for FR2. The UE travelling direction (v, v) are as follows for FR1:

-	(135°,90o) for UMi CDL A channel model

-	(127.05°],90o) for UMi CDL C channel model

-	(182.17°,90o) for UMa CDL A channel model

-	(65°,90o) for UMa CDL C channel model

The UE travelling direction (v, v) are as follows for FR2:

-	(112.51°,90°) for InO CDL-A channel model

-	(74.11°,90°) for UMi CDL-C channel model

Note: the FR2 cases may be mixed up in 38.827, though this has no impact in the CDL-C UMa case we are focusing on.

### B.2.X1.Y1	Channel Model Parameters for FR1

The Channel model parameter tables for CDL-A, B, C, D, and E for UMa and UMi at 3.5 GHz are presented in this subclause without the effect of base station antenna filtering.

For FR1, the baseline emulated propagation environment is 2D without elevation modelling, i.e., all ZOA are set 90° and ZSA is 0° in the following tables.

Tables 7.2.1-6—7.2.1-10 tabulate channel model parameters for UMa CDL-A—CDL-E models at 3.5 GHz, respectively.

In the determination of desired angle spreads (ASdesired), frequency is set 6 GHz as stated in Table 7.5-6 Part-1 of TR38.901.

For the determination of desired zenith spread of departure (ZSDdesired) from table 7.5-7 of TR38.901, the following parameters are used hUT = 1.5 m, and d2d = 100 m.

Table 7.2.1-8: Channel model parameters for UMa CDL-C at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | -96.4031 | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |

<End of TP>

Implementation notes

During the implementation of the channel model from section 3, a number of questions and remarks came up repeatedly. To aid other interested contributors with their implementation, we summarize our understanding of some less evident aspects of the implementation in the following.

Everything in this section is our interpretation of sometimes unclear wording or concepts in TS 38.827, hence all notes are open to re-interpretation during the study. See also the previous contributions [3] and [4] with earlier implementation notes.

Setup in GCS and LCS

The TP is intended to describe the following antenna coordinates, angles, and other parameters for gNB and UE, from a global coordinate system perspective (GCS). The LCS transformations are given to highlight the configured mechanical pointing direction of the tx and rx antenna panels.

Figure 8: Antenna coordinates, angles, and other parameters for gNB and UE

Note:

- It is our understanding that the r_{tx,max} from 38.827, and the DoDs/DoAs of the tables in 38.827/901 are in spherical angles and, thus, referenced to GCS.

- The Rx side UE ULA with omnidirectional AE field patterns is effectively “tilt-invariant”, but the parameter is given for completeness.

TR 38.827 Fully connected TX virtualizer + directional steering

In TR 38.827 a fully connected virtualizer (per polarization) is defined via the formulation

α1,1…αMe,Ne=1MeNeexp-j2πrtx,max T ∙ dtx,me,neλ0∈C1×MeNe.

This is applied to each BB/RF port, with different rtx,max  per port pair. Hence, each port pair has a separate directional steering (direction of departure, DoD) using the full antenna array. The virtualizer is not changed dynamically during the channel emulation, and rather set to match DoDs that result in maximized received power at the receiver in the given large-scale environment.	
The connection framework can be quickly visualized in Figure 1.

Figure 1: TR 38.827 fully connected TX virtualizer (+ directional steering).

It is currently unclear, why 38.827 has chosen this specific virtualizer. It remains unknown if this constitutes a limitation in the MPAC minimum test setup assumption.

TR38.901 Partially connected (subarray partition) TX virtualizer

In TR 38.901 it is most common to assume subarray partition virtualizers with directional steering/tilting on a per antenna array column basis. As we aim to represent in Figure 2, each BB/RF port is only connected to a subarray of the TX panel(s) and less weights are chosen. Often the weights are found

Using the matrix representation of the fully connected virtualizer it is straightforward to apply the same framework and judicious use of “0” weights, albeit that an implementation would not use zero weights, but rather not process these parts of the time domain channel coefficient generation.

Figure 2: Partially connected (subarray partition) TX virtualizer.

Conclusion

Within this contribution we have provided simulation results and CDL implementation reference for use in the Spatial Channel Model for Demodulation Performance Requirements study, the results will be used in our companion discussion contribution [2].

- References

- RP-241610; “New SID: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc et al, RAN#104, Shanghai, June 2024.

- R4-2418043, “Discussion on Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#113.

- R4-2411301, “Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#112.

- R4-2415283, “Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#112bis.