3GPP TSG-RAN4 Meeting #113	R4-2418050

# Orlando, US, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Duty cycle solutions and MSD rules for HPUE inter-band UL CA and EN-DC

Agenda item:	7.1.1.1.1

Document for:	Discussion

Introduction

In this paper, we provide our views on duty cycle solutions and MSD rules for HPUE inter-band UL CA and EN-DC.

Discussion (duty cycle solutions)

In last RAN4 meeting, the following WF was approved [1].

[1] 4.2 Duty cycle solution for SAR compliance - For intra-band CA and inter-band CA PC1.5, duty cycle solution for SAR will be discussed. - For EN-DC PC1.5 and PC2, duty cycle solution for SAR will discussed for the scenario without new IE introduction.

Table 2-0 summarizes the cases which can be discussed in Rel-19 for HPUE NR inter-band CA and EN-DC.

Table 2-0: Example case for HPUE NR inter-band CA and EN-DC

|  | Number of Tx | NR | NR | Whether to support or not in Rel-19 |
| --- | --- | --- | --- | --- |
| PC1.5 CA | 2Tx/3Tx | PC2 TDD (1Tx) | PC2 TDD (1Tx/2Tx) | O |
|  | 3Tx | PC3 TDD (1Tx) | PC1.5 TDD (2Tx) | O |
|  | 3Tx | PC2 TDD (1Tx) | PC1.5 TDD (2Tx) | O |
|  | Number of Tx | E-UTRA | NR |  |
| PC2 EN-DC | 2Tx/3Tx | PC3 FDD (1Tx) | PC3 FDD (1Tx/2Tx) | O |
|  | 2Tx/3Tx | PC2 TDD (1Tx) | PC3 TDD (1Tx/2Tx) | O |
| PC1.5 EN-DC | 2Tx/3Tx | PC2 TDD (1Tx) | PC2 TDD (1Tx/2Tx) | O |
|  | 3Tx | PC3 FDD | PC1.5 TDD (2Tx) | O |
|  | 3Tx | PC3 TDD | PC1.5 TDD (2Tx) | O |
|  | 3Tx | PC2 TDD | PC1.5 TDD (2Tx) | O |

PC1.5 UE for NR inter-band uplink CA with 2Tx and/or 3Tx

For 2Tx,

PC2+PC2 for 2-band NR bands is new power combination.

- Case 1 : PC2 NR FDD + PC2 NR TDD

- Case 2 : PC2 NR TDD + PC2 NR TDD

Based on WF [2], case 1 can be treated after example combo(s) is added in the WID. however, case 2 can be treated in RAN4. For case2, the existing PC2 duty cycle solution should be modified.

For example,

If a UE supports power class 1.5 for the band combination and the supported power class enables the higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and - if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or - if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; - shall apply all requirements for the default power class and set the configured transmitted power as specified in in clause 6.2A.4.1.3; - if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or - if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is between 23dBm and 26dBm; - shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2A.4.1.3; - else; - shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 50% ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331. For NR Band x or NR Band y, – if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

Proposal 1: Update the existing PC2 duty cycle solution if PC2 TDD + PC2 TDD with 2Tx is introduced.

For PC1.5 UE supporting NR inter-band UL CA with 3Tx,

In Rel-18, the following combination of PC for NR bands is supported.

- Case 1 : PC3 NR FDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

Whether to support the following combinations in Rel-19 need to be decided. Based on WF [2], case 2, and case 3 can be treated after example combo(s) is added in the WID.

- Case 2 : PC2 NR FDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 3 : PC2 NR FDD + PC2 NR TDD (UL-MIMO or Tx Diversity)

- Case 4 : PC3 NR TDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 5 : PC2 NR TDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 6 : PC2 NR TDD + PC2 NR TDD (UL-MIMO or Tx Diversity)

Proposal 2: For PC1.5 UE supporting NR inter-band UL CA with 3Tx,  the existing duty-cycle solution can be applied if PC3 NR TDD + PC1.5 NR TDD (2Tx), and PC2 NR TDD + PC1.5 NR TDD (2Tx) are introduced.

PC1.5 and PC2 for EN-DC with 2Tx and/or 3Tx

For PC2 UE supporting two band EN-DC with 2Tx,

In Rel-18, the following combination of PC for E-UTRA band and PC for NR band is supported.

- Case 1 : PC3 E-UTRA FDD + PC3 NR TDD

- Case 2 : PC3 E-UTRA FDD + PC2 NR TDD

- Case 3 : PC3 E-UTRA TDD + PC3 NR TDD

- Case 4 : PC3 E-UTRA TDD + PC2 NR TDD

Whether to support the following combinations in Rel-19 need to be decided. Based on WF [2], case 6 can be treated after example combo(s) is added in the WID.

- Case 5 : PC3 E-UTRA FDD + PC3 NR FDD

- Case 6 : PC3 E-UTRA FDD + PC2 NR FDD

- Case 7 : PC2 E-UTRA TDD + PC3 NR TDD

For Case 5 Case 6, the existing E-UTRA FDD + NR TDD duty cycle solution can be reused with a new capability, for example, maxUplinkDutyCycle-interBandENDC-FDD-PC2 which comprises of maxUplinkDutyCycle-FDD-EN-DC1 and  maxUplinkDutyCycle-FDD-EN-DC2,  instead of maxUplinkDutyCycle-FDD-TDD-EN-DC1 and  maxUplinkDutyCycle-FDD-TDD-EN-DC2 which are applicable only for inter-band FDD+TDD EN-DC.

Or,

the existing E-UTRA FDD + NR TDD duty cycle solution can be reused with same capability, maxUplinkDutyCycle-FDD-TDD-EN-DC1 and  maxUplinkDutyCycle-FDD-TDD-EN-DC2, which need to be updated  for inter-band FDD+FDD EN-DC.

For Case 7, the existing E-UTRA TDD + NR TDD duty cycle solution needs to be modified, because it is based on PC3 E-UTRA TDD. PC2 E-UTRA TDD based duty cycle solution is needed.

For example, Table 2.1 shows the percentage of maximum E-UTRA/NR uplink transmission for Case 7.

Table 2.1: Percentage of maximum E-UTRA/NR uplink transmission in PC2 EN-DC (Case 7)

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D:threshold) |  |
| --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (23dBm) |  |  |
|  | Y=0~50 | Z=0~100, D=100-2*Y | Z <= D | Z>D |
| 0 | 70 | N/A | PC2 | PC3 |
| 1 | 50 | N/A |  |  |
| 2 | 30 | 40 |  |  |
| 3 | 40 | 20 |  |  |
| 4 | 30 | 40 |  |  |
| 5 | 20 | 60 |  |  |
| 6 | 60 | N/A |  |  |

- X : E-UTRA TDD Uplink-downlink Configuration

- Y : maximum percentage of E-UTRA uplink transmission during any evaluation period of time

- Z : actual percentage of NR uplink transmission during any evaluation period of time

- D : threshold of NR uplink transmission during any evaluation period of time

Here, E-UTRA TDD uplink-downlink configuration ‘0’, ‘1’ and ‘6’ are not applicable for EN-DC uplink transmission with maximum output power for SAR compliance. A minimum percentage of 20% for maximum NR uplink transmission during evaluation period can be assumed as threshold when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent.

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 3 and threshold of NR uplink transmission = 20,

- Z > 20,        PC3 EN-DC is applied

- Z ≤ 20,      PC2 EN-DC is applied

Table 2.2 shows the percentage of maximum E-UTRA/NR uplink transmission for Case 3. Threshold of 30% was specified with the minimum percentage of maximum NR uplink transmission for PC3 E-UTRA TDD+PC3 NR TDD

when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent.

Table 2.2: Percentage of maximum E-UTRA/NR uplink transmission in PC2 EN-DC (Case 3)

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D:threshold) |  |
| --- | --- | --- | --- | --- |
|  | E-UTRA (23dBm) | NR (23dBm) |  |  |
|  | Y=0~100 | Z=0~100, D=100-Y | Z <= D | Z>D |
| 0 | 70 | 30 | PC2 | PC3 |
| 1 | 50 | 50 |  |  |
| 2 | 30 | 70 |  |  |
| 3 | 40 | 60 |  |  |
| 4 | 30 | 70 |  |  |
| 5 | 20 | 80 |  |  |
| 6 | 60 | 40 |  |  |

Proposal 3: For PC2 UE supporting two band EN-DC with 2Tx, consider a new capability if PC3 E-UTRA FDD + PC3 NR FDD is introduced. For example,

- maxUplinkDutyCycle-interBandENDC-FDD-PC2

  - comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 4: For PC2 UE supporting two band EN-DC with 2Tx, consider a minimum percentage of 20% as threshold for maximum NR uplink transmission if PC2 E-UTRA TDD + PC3 NR TDD is introduced.

For PC1.5 UE supporting two band EN-DC with 2Tx,

In Rel-18, it is not supported.

Whether to support the following combination in Rel-19 needs to be decided.

- Case 1 : PC2 E-UTRA TDD + PC2 NR TDD

For Case 1, the existing E-UTRA TDD + NR TDD duty cycle solution needs to be modified, because it is based on PC3 E-UTRA TDD. PC2 E-UTRA TDD based duty cycle solution is needed.

For example, Table 2.3 shows the percentage of maximum E-UTRA/NR uplink transmission for Case 1.

Table 2.3: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (26dBm(E-UTRA TDD)+26dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (26dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~50 | Z1=0~50, D1=50-Y | Z2=0~50, D2=25-Y = D1-25 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | N/A | N/A | N/A | N/A | N/A |
| 1 | 50 | N/A | N/A | N/A | N/A | N/A |
| 2 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 3 | 40 | 10 | N/A | PC3 | PC2 | N/A |
| 4 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 5 | 20 | 30 | 5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | N/A | N/A | N/A | N/A | N/A |

Here, E-UTRA TDD uplink-downlink configuration ‘0’, ‘1’ and ‘6’ are not applicable for EN-DC uplink transmission with maximum output power for SAR compliance.

- X : E-UTRA TDD Uplink-downlink Configuration

- Y : maximum percentage of E-UTRA uplink transmission during any evaluation period of time

- Z : actual percentage of NR uplink transmission during any evaluation period of time

- D : threshold of NR uplink transmission during any evaluation period of time

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 30 (D1=30, D2=5),

- Z > 30,        PC3 EN-DC is applied

- 5 < Z ≤ 30, PC2 EN-DC is applied

- Z ≤ 5,         PC1.5 EN-DC is applied

Proposal 5: For PC1.5 UE supporting two band EN-DC with 2Tx,  define new duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission if PC2 E-UTRA TDD + PC2 NR TDD is introduced. For example,

- E-UTRA TDD Uplink-downlink Configuration (X) = 5

- minimum percentage of 30% and 5% as 2 thresholds for maximum NR uplink transmission

For PC2 UE supporting two band EN-DC with 3Tx,

In Rel-18, the following combination of PC for E-UTRA band and PC for NR band is supported.

- Case 1 : PC3 E-UTRA FDD + PC3 NR TDD (UL-MIMO or Tx diversity)

- Case 2 : PC3 E-UTRA FDD + PC2 NR TDD (UL-MIMO or Tx diversity)

- Case 3 : PC3 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

- Case 4 : PC3 E-UTRA TDD + PC2 NR TDD (UL-MIMO or Tx diversity)

Whether to support the following combinations in Rel-19 need to be decided. Based on WF [1], case 6 can be treated after example combo(s) is added in the WID.

- Case 5 : PC3 E-UTRA FDD + PC3 NR FDD (UL-MIMO or Tx diversity)

- Case 6 : PC3 E-UTRA FDD + PC2 NR FDD (UL-MIMO or Tx diversity)

- Case 7 : PC2 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

For Case 5, the existing E-UTRA FDD + NR TDD duty cycle solution can be reused with a new capability, for example, maxUplinkDutyCycle-interBandENDC-FDD-PC2 which comprises of maxUplinkDutyCycle-FDD-EN-DC1 and  maxUplinkDutyCycle-FDD-EN-DC2,  instead of maxUplinkDutyCycle-FDD-TDD-EN-DC1 and  maxUplinkDutyCycle-FDD-TDD-EN-DC2 which are applicable only for inter-band FDD+TDD EN-DC.

For Case 7 , the existing E-UTRA TDD + NR TDD duty cycle solution needs to be modified, because it is based on PC3 E-UTRA TDD. PC2 E-UTRA TDD based duty cycle solution is needed. Table 2.1 can be considered.

Proposal 6: For PC2 UE supporting two band EN-DC with 3Tx,  consider  a new capability if PC3 E-UTRA FDD + PC3 NR FDD is introduced. For example,

- maxUplinkDutyCycle-interBandENDC-FDD-PC2

  - comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 7: For PC2 UE supporting two band EN-DC with 3Tx, consider a minimum percentage of 20% as threshold for maximum NR uplink transmission if PC2 E-UTRA TDD + PC3 NR TDD is introduced.

For PC1.5 UE supporting two band EN-DC with 3Tx,

In Rel-18, it is not supported.

Whether to support the following combinations in Rel-19 needs to be decided.

- Case 1 : PC3 E-UTRA FDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 2 : PC3 E-UTRA TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 3 : PC2 E-UTRA TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 4 : PC2 E-UTRA TDD + PC2 NR TDD (UL-MIMO or Tx diversity)

For Case 1, the following formula can be assumed between the LTE UL duty cycle, NR UL duty cycle and the overall duty threshold.

-   DutyLTE x( PLTE/ P29) + DutyNR x(PNR/ P29)  ≤ Duty threshold

-   0.25xDutyLTE + DutyNR  ≤ Duty threshold

DutyLTE is from 0 to 100%.

DutyNR is from 0 to 25%.

PLTE, PNR, P29 represent the maximum linear power (mW) of LTE, NR, and EN-DC PC1.5 respectively; DutyLTE, DutyNR represent the maximum uplink percentage of LTE, NR respectively. Duty threshold represents the maximum UL duty cycle that can maintain the PC1.5 power class for FDD+TDD EN-DC.

The LTE reference configuration can be DutyLTE=70%. The default value of NR UL duty capability can be 7.5% i.e. "DutyLTE=70% and PLTE=23dBm", corresponding to default DutyNR=7.5%, PNR=29dBm.

For better flexibility, two LTE configurations are supported with LTE reference configuration of DutyLTE=70% and 40%.

The LTE reference configuration can be DutyLTE=40%. The default value of NR UL duty capability can be 15% i.e. "DutyLTE=40% and PLTE=23dBm", corresponding to default DutyNR=15%, PNR=29dBm.

It means that Duty threshold is scaled with 0.25.

For Case 2, the corresponding duty cycle solution needs to be specified.

Table 2.4 shows this example for the percentage of maximum E-UTRA/NR UL transmission. The possible E-UTRA TDD uplink-downlink configurations and the percentage need to be considered when specifying duty cycle solution.

Table 2.4: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (23dBm(E-UTRA TDD)+2x26dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (23dBm) | NR (29dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~100 | Z1=0~25, D1=(100-Y)/4 =25-Y/4 | Z2=0~25 D2=(50-Y)/4 =D1-12.5 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | 7.5 | N/A | N/A | N/A | N/A |
| 1 | 50 | 12.5 | N/A | N/A | N/A | N/A |
| 2 | 30 | 17.5 | 5 | PC3 | PC2 | N/A |
| 3 | 40 | 15 | 2.5 | PC3 | PC2 | N/A |
| 4 | 30 | 17.5 | 5 | PC3 | PC2 | N/A |
| 5 | 20 | 20 | 7.5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | 10 | N/A | N/A | N/A | N/A |

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 20 (D1=20, D2=7.5),

- Z > 20,            PC3 EN-DC is applied

- 7.5 < Z ≤ 20, PC2 EN-DC is applied

- Z ≤ 7.5,         PC1.5 EN-DC is applied

For Case 3, the corresponding duty cycle solution needs to be specified.

Table 2.5 shows this example for the percentage of maximum E-UTRA/NR UL transmission. The possible E-UTRA TDD uplink-downlink configurations and the percentage need to be considered when specifying SAR solution.

Table 2.5: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (26dBm(E-UTRA TDD)+2x26dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (29dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~50 | Z1=0~25, D1=(50-Y)/2 | Z2=0~25 D2=(25-Y)/2 =Z1-12.5 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | N/A | N/A | N/A | N/A | N/A |
| 1 | 50 | N/A | N/A | N/A | N/A | N/A |
| 2 | 30 | 10 | N/A | PC3 | PC2 | N/A |
| 3 | 40 | 5 | N/A | PC3 | PC2 | N/A |
| 4 | 30 | 10 | N/A | PC3 | PC2 | N/A |
| 5 | 20 | 15 | 2.5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | N/A | N/A | N/A | N/A | N/A |

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 15 (D1=15, D2=2.5),

- Z > 15,           PC3 EN-DC is applied

- 2.5 < Z ≤ 15, PC2 EN-DC is applied

- Z ≤ 2.5,         PC1.5 EN-DC is applied

For Case 4, the corresponding duty cycle solution needs to be specified.

Table 2.6 shows this example for the percentage of maximum E-UTRA/NR UL transmission. The possible E-UTRA TDD uplink-downlink configurations and the percentage need to be considered when specifying duty cycle solution.

Table 2.6: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (26dBm(E-UTRA TDD)+2x23dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (26dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~50 | Z1=0~50, D1=50-Y | Z2=0~50 D2=25-Y = D1-25 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | N/A | N/A | N/A | N/A | N/A |
| 1 | 50 | N/A | N/A | N/A | N/A | N/A |
| 2 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 3 | 40 | 10 | N/A | PC3 | PC2 | N/A |
| 4 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 5 | 20 | 30 | 5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | N/A | N/A | N/A | N/A | N/A |

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 30 (D1=30, D2=5),

- Z > 30,        PC3 EN-DC is applied

- 5 < Z ≤ 30, PC2 EN-DC is applied

- Z ≤ 5,         PC1.5 EN-DC is applied

Proposal 8: For PC1.5 UE supporting two band EN-DC with 3Tx, define new duty cycle solutios considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission if the following band combinations are introduce.

- PC3 E-UTRA FDD/TDD + PC1.5 NR TDD

- PC2 E-UTRA TDD + PC2/PC1.5 NR TDD

For example of PC2 E-UTRA TDD + PC2 NR TDD,

- E-UTRA TDD Uplink-downlink Configuration (X) = 5

- minimum percentage of 30% and 5% as 2 thresholds for maximum NR uplink transmission

Discussion (MSD rules)

In last RAN4 meeting, the following WF was approved [1].

[1] 4.1 MSD rules - To clarify the Equations/LUT/Simplified guidelines mentioned in Option 1 and Option 3 - Some examples are provided in the Annex of this WF - Other options such as Option2 are not precluded Option 1: - RAN4 agrees on the following directions in this meeting and continues on details in subsequent meetings. - RAN 4 defines the default power class inter-band CA/DC and intra-band CA/DC MSD requirements, and RAN4 specifies equations/simplified guidelines/lookup tables “LUT” that define the relationship between the default power class MSD and the HPUE MSD requirements to replace the existing HPUE MSD tables: - Equations/LUT/Simplified guidelines identifying the HPUE MSD requirements are expected to be captured in RAN4 spec as normative requirements, in Rel-19 if possible, the details are FFS - FFS the expected spec changes and procedure changes (i.e., continue/stop providing TP’s for each combination, remove/keep all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19 - Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus] - [Agree to capture just single worst case MSD in TS for HPUE PC which is agnostic to number of TX, for each kind of MSD] NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done. NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow. Option 2: - Current working procedure of HPUE CA/DC introduction are kept unchanged - HPUE MSD tables are kept - Unified Excel tool of calculating HPUE MSD are being developed with following details to be addressed - Companies to evaluate if the interferer level for PC2/PC1.5 can be scaled by less than current 100% to achieve more meaningful PC2/PC1.5 IMD4 and IMD5. - [Agree to capture just single worst case MSD value in TS for each PC which is agnostic to number of TX]. - Agree to use the following rules in deriving PC2/PC1.5 MSD for cases where PC3 MSD is 0dB: - If one of the rules above is met, PC2 1TX MSD=0.4dB, PC2 2TX MSD=0.5dB, PC1.5 2TX MSD=0.8dB. - If both rules above are met, PC2 1TX MSD=0.8dB, PC2 2TX MSD=0.9dB, PC1.5 2TX MSD=1.5dB. - In case it is discovered that the PC3 MSD is omitted or is illogical compared to respective similar band combinations, then naturally the PC3 MSD can be further evaluated before PC2/PC1.5 assessment. - Agree that the assumed PRX/DRX difference is 6dB, and additional noise into PRX is 1dB due to 2 TX case for PC2/PC1.5 NR CA and EN-DC. - The tool is published in 3GPP in Excel format if all proposals are agreeable. NOTE: Freedom for any company to provide PC3 MSD evaluations using companies’ calculation methods and other assumptions fully remains because PC3 is the baseline to derive PC2/PC1.5 MSD. It is not precluded to provide evaluations for PC2/PC1.5. Option 3: - PC2/PC1.5 MSD requirements are not specified. UE only need to be verified under the Rx requirements of default power class, with default power class transmission - Add the Equations/LUT/Simplified guidelines into the Annex of TS to derive the HPUE MSD for information, in Rel-19 if possible, the details are FFS - FFS the expected spec changes and procedure changes (i.e., continue/stop providing TP’s for each combination, remove/keep all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19 -Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus] NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done. NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow. Option 4: - PC2/PC1.5 MSD requirements are not specified, MSD for PC2 and PC1.5 are only specified in TR or Annes of TS which is not mandate, UE shall meet the Rx requirements of default power class, with default power class transmission.

General understanding on MSD rules

In the last RAN4#112bis meeting, there was a lot of discussion about the MSD rule regarding HPUE, and the relevant WF was presented. Option 1, option 3, and Option 4 propose new MSD rules that differ from the existing MSD rule, while Option 2 suggests a rule similar to the existing one. Each option has its pros and cons, and particularly Option 1, 3, and 4 could significantly reduce the workload related to the HPUE basket and also considerably decrease the amount of the MSD table for HPUE in the current specifications. However, a common equation for HPUE MSD is needed, and it seems challenging to address all HPUE MSDs with a single equation. On the other hand, Option 2, which maintains the existing approach, can accurately define the MSD by reflecting the characteristics of HPUE in specific band combinations, but it has a significant workload issue that exists in the current basket work, and large tables for HPUE remain in the specifications, which could make the specifications increasingly complex.

Observation 1: Option 1, 3, and 4 could significantly reduce the workload related to the HPUE basket and also considerably decrease the amount of the MSD table for HPUE in the current specifications. However, a common equation for HPUE MSD is needed, and it seems challenging to address all HPUE MSDs with a single equation.

Observation 2: Option 2, which maintains the existing approach, can accurately define the MSD by reflecting the characteristics of HPUE in specific band combinations, but it has a significant workload issue that existes in the current basket work, and large tables for HPUE remain in the specifications, which could make the specifications increasingly complex.

Currently, there are three types of MSD(Maximum Sensitivity Degradation) in the specifications for CA/DC, and the MSD types are categorized based on the types of UL interference. The types of interference include UL harmonic, intermodulation, and cross-band isolation, and the MSD value can be calculated based on the difference between the power level of the interference leaking into the DL and the reference sensitivity of the victim band. HPUE MSD can also be derived in the same way as default UE, and the value can be obtained by adjusting the UL aggressor power. Therefore if the size of the UL interference for HPUE increases linearly, the MSD value for HPUE can be easily derived based on the default MSD value.

Observation 3: If UL interference for HPUE increases linearly, the MSD value for HPUE can be easily derived based on the default MSD value.

The following presents our company’s perspective on whether a simple equation can be applied to the MSD of HPUE.

Harmonic & Harmonic mixing

The MSD values of Harmonic and harmonic mixing can be determined by the harmonic components of the UL aggressor. Table 3.1 and Table 3.2 show the MSD requirements for Harmonic and harmonic mixing as defined in the current specification, along with the differences in MSD values according to power class.

Table 3.1 MSD values of harmonic comparison between PC3 and PC2

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD |  | MSD difference between PC3 and PC2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | PC2 | PC3 |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 27.1 | 23.9 | 3.2 |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 16.6 | 13.8 | 2.8 |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 26.9 | 23.9 | 3 |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 3.1 | 1.1 | 2 |
| n25 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 16.8 | 13.8 | 3 |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 5 | 12.8 | 10 | 2.8 |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 40 | 3.5 | 2.1 | 1.4 |
| n71 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 13.6 | 10.8 | 2.8 |
| n71 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 2.5 | 1.4 | 1.1 |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 26.9 | 23.9 | 3 |
| n66 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 16.7 | 13.8 | 2.9 |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 2.0 | 1.1 | 0.9 |
| n71 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 13.2 | 10.4 | 2.8 |

Table 3.2 MSD values of harmonic mixing comparison among PC1.5, PC2 and PC3

| UL band | DL band | MSD |  |  | MSD difference between PC3 and PC2 | MSD difference between PC2 and PC1.5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PC1.5 | PC2 | PC3 |  |  |
| n25 | n41 | N/A | 2.2 | 1.3 | 0.9 | N/A |
| n25 | n713 | N/A | 29.5 | 26.5 | 3 | N/A |
| n25 | n713 | N/A | 18.2 | 15.3 | 2.9 | N/A |
| n39 | n41 | N/A | 10.6 | 8.1 | 2.5 | N/A |
| n39 | n41 | N/A | 4.9 | 3.3 | 1.6 | N/A |
| n40 | n28 | 43.8 | 40.8 | 37.8 | 3 | 3 |
| n40 | n28 | 36.3 | 33.3 | 30.3 | 3 | 3 |
| n41 | n186 | N/A | 27.3 | 24.3 | 3 | N/A |
| n41 | n186 | N/A | 27.3 | 24.3 | 3 | N/A |
| n41 | n39 | 10.5 | 6.1 | 4.3 | 1.8 | 4.4 |
| n41 | n39 | 3 | 1.5 | 0.8 | 0.7 | 1.5 |
| n77 | n2 | 11.8 | 9.1 | 6.7 | 2.4 | 2.7 |
| n77 | n2 | 9.2 | 6.7 | 3,7 | 3 | 2.5 |
| n77 | n3 | 11.1 | 8.1 | N/A | N/A | 3 |
| n77 | n3 | 4 | 1 | N/A | N/A | 3 |
| n77 | n5 | 10.7 | 8.1 | [5.7] | 2.4 | 2.6 |
| n77 | n5 | 6.4 | 4.3 | [2.7] | 1.6 | 2.1 |

Observation 4: As shown in Table 3.1 and Table 3.2, when the power of the UL aggressor is increased by 3 dB in harmonic and harmonic mixing, MSD is increased by approximately 0.7 to 4.4 dB, 3 dB in many cases. This indicates that the increase in UL aggressor power is similar to the increase in UL harmonic interference power.

Proposal 9: The simple approaches (LUT, equation  ...) can be applied for harmonic & harmonic mixing MSD requirements of HPUE.

Inter-modulation

Inter-modulation MSD refers to the case where interference signals generated by two UL signals passing through a nonlinear device affect the DL band.

Observation 5: The IMD MSD requirement is more complex than other types of MSD requirement. The reasons why it is difficult to derive the IMD MSD value using a simple equation are shown below.

- Numerous band combinations are defined in the specification, and the RF characteristics of the UE vary depending on the band combination

- IMD MSDs are defined for IMD2 to IMD9 in the current specification, and as the order of the influencing IMD increases, the number of case for IMD products also increases. Even for MSDs of the same IMD order, the MSD values can vary significantly depending on the IMD product. For example, the products of IMD4 can be (3,1) and (2,2) …, where the case of (3,1) can have a much larger MSD value than (2,2).

- Since MSD values are calculated using various methods such as measurement, simulation, and hand calculation with different RF assumption, it may be difficult to express them with a simple equation.

Proposal 10: Do not apply the simple approaches for IMD MSD requirements of HPUE.

Cross band isolation

MSD requirement for cross band isolation is that applies the impact of UL aggressor emissions on victim DL REFSENS. Therefore, the MSD value is determined by how well filtering is performed between the UL and DL bands and how much UL emission occurs. Additionally, the closer the spacing between the UL aggressor band and the DL victim band, the larger the MSD value tends to be specified. Below is a representation of some of the cross band isolation MSD requirement in the current specification, showing how much the MSD differs between each power class for PC1.5, PC2, and PC3.

Table 3.3 MSD values of cross band isolation comparison among PC1.5, PC2 and PC3

| UL band | DL band | DL Fc | MSD |  |  | MSD difference between PC3 and PC2 | MSD difference between PC2 and PC1.5 | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | PC1.5(dB) | PC2 (dB) | PC3 (dB) | (dB) | (dB) |  |
| n2 | n66 | 2112.5 | N/A | 0.7 | N/A | N/A | N/A | >ACLR2 |
| n3 | n1 | 2112.5 | N/A | 0.8 | N/A | N/A | N/A | >ALCR2 |
| n3 | n7 | 2622.5 | N/A | 0.5 | N/A | N/A | N/A | >ALCR2 |
| n7 | n1 | 2167.5 | N/A | 0.8 | N/A | N/A | N/A | >ALCR2 |
| n7 | n3 | 1877.5 | N/A | 0.4 | N/A | N/A | N/A | >ALCR2 |
| n25 | n41 | 2501 | N/A | 0.8 | N/A | N/A | N/A | >ACLR2 |
| n25 | n66 | 2112.5 | N/A | 0.7 | N/A | N/A | N/A | >ACLR2 |
| n39 | n41 | 2501 | N/A | 4.7 | 3.3 | 1.4 | N/A | >ACLR2 |
| n40 | n3 | 1877.5 | 1.2 | 0.6 | N/A | N/A | 0.6 | >ACLR2 |
| n40 | n41 | 2501 | 37.9 | 31.1 | 28.1 | 3 | 6.8 | ACLR2 |
| n41 | n1 | 2167.5 | 23.5 | 20.8 | 18.1 | 2.7 | 2.7 | >ACLR2 |
| n41 | n3 | 1877.5 | 3.9 | 2.3 | 0.6 | 1.7 | 1.6 | >ACLR2 |
| n41 | n25 | 1992.5 | 2.8 | 1.6 | 0.6 | 1 | 1.2 | >ACLR2 |
| n41 | n39 | 1917.5 | 6.7 | 2.7 | 1.6 | 1.1 | 4 | >ACLR2 |
| n41 | n40 | 2397.5 | 41.2 | 34.4 | 31.4 | 3 | 6.8 | ACLR2 |
| n41 | n66 | 2197.5 | 7.7 | 5.4 | 10.5 | -5.1 | 2.3 | >ACLR2 |
| n41 | n77 | 3305 | 13.3 | 10.5 | 8.3 | 2.2 | 2.8 | >ACLR2 |
| n41 | n79 | 4405 | N/A | 3.1 | N/A | N/A | N/A | >ACLR2 |
| n66 | n2 | 1932.5 | N/A | 1.9 | 1.2 | 0.7 | N/A | >ACLR2 |
| n66 | n25 | 1932.5 | N/A | 2.2 | 1.4 | 0.8 | N/A | >ACLR2 |
| n66 | n41 | 2501 | N/A | 0.8 | 0.4 | 0.4 | N/A | >ACLR2 |
| n71 | n85 | 730.5 | N/A | 10.94 | 23 | -12.06 | N/A | ACLR2 |
| n71 | n85 | 730.5 | N/A | 26 | 23 | 3 | N/A | ACLR1 |
| n77 | n2 | 1987.5 | 1.8 | 1 | N/A | N/A | 0.8 | >ACLR2 |
| n77 | n7 | 2687.5 | 9 | 6.5 | 4.5 | 2 | 2.5 | >ACLR2 |
| n77 | n25 | 1992.5 | 1.8 | 1 | N/A | N/A | 0.8 | >ACLR2 |
| n77 | n30 | 2357.5 | 1.8 | 1 | N/A | N/A | 0.8 | >ACLR2 |
| n77 | n401 | 2397.5 | 9 | 6.5 | 4.5 | 2 | 2.5 | >ACLR2 |
| n77 | n401 | 2350 | 2.2 | 6.5 | 4.5 | 2 | -4.3 | >ACLR2 |
| n77 | n411 | 2685 | 9 | 6.5 | 4.5 | 2 | 2.5 | >ACLR2 |
| n77 | n411 | 2640 | 9 | 6.5 | 4.5 | 2 | 2.5 | >ACLR2 |
| n77 | n66 | 2197.5 | 1.8 | 1 | N/A | N/A | 0.8 | >ACLR2 |
| n78 | n7 | 2687.5 | N/A | 6.5 | 4.5 | 2 | N/A | >ACLR2 |
| n78 | n401 | 2395 | 9 | 6.5 | 4.5 | 2 | 2.5 | >ACLR2 |
| n78 | n401 | 2350 | 2.2 | 1.2 | 4.5 | -3.3 | 1 | >ACLR2 |
| n783 | n79 | 4405 | 8 | 5 | 2 | 3 | 3 | >ACLR2 |
| n783 | n79 | 4420 | 8 | 5 | 2 | 3 | 3 | >ACLR2 |
| n783 | n79 | 4450 | 8 | 5 | N/A | N/A | 3 | >ACLR2 |
| n79 | n41 | 2685 | N/A | 3.5 | N/A | N/A | N/A | >ACLR2 |
| n79 | n783 | 3795 | 8,6 | 5.6 | 2.6 | 3 | N/A | >ACLR2 |
| n79 | n783 | 3750 | 8.6 | 5.6 | 2.6 | 3 | 3 | >ACLR2 |

Observation 6: The cross band isolation MSD values exhibit the following trends in current specification.

- The MSD difference between each power class is 0.4~3 dB.

- There are many MSDs that are not defined in the default power class (PC3) but are defined only in HPUE (PC2, PC1.5).

Proposal 11: Need study whether the simple approaches can be applied for cross band isolation MSD requirement of HPUE.

Conclusion

In this contribution, we proposed our view on the duty cycle solutions and MSD rules for HPUE  inter-band CA and EN-DC. The proposals are as follows.

Duty cycle solutions

For PC1.5 UE supporting NR inter-band UL CA with 2Tx,

Proposal 1: Modify the existing PC2 duty cycle solution if PC2 TDD + PC2 TDD with 2Tx is introduced.

For example,

If a UE supports power class 1.5 for the band combination and the supported power class enables the higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and - if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or - if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; - shall apply all requirements for the default power class and set the configured transmitted power as specified in in clause 6.2A.4.1.3; - if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or - if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is between 23dBm and 26dBm; - shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2A.4.1.3; - else; - shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 50% ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331. For NR Band x or NR Band y, – if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

For PC1.5 UE supporting NR inter-band UL CA with 3Tx,

Proposal 2: For PC1.5 UE supporting NR inter-band UL CA with 3Tx,  the existing duty-cycle solution can be applied if PC3 NR TDD + PC1.5 NR TDD (2Tx), and PC2 NR TDD + PC1.5 NR TDD (2Tx) are introduced.

For PC2 UE supporting two band EN-DC with 2Tx,

Proposal 3: For PC2 UE supporting two band EN-DC with 2Tx,  consider  a new capability if PC3 E-UTRA FDD + PC3 NR FDD is introduced. For example,

- maxUplinkDutyCycle-interBandENDC-FDD-PC2

  - comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 4: For PC2 UE supporting two band EN-DC with 2Tx, consider a minimum percentage of 20% as threshold for maximum NR uplink transmission if PC2 E-UTRA TDD + PC3 NR TDD is introduced.

For PC1.5 UE supporting two band EN-DC with 2Tx,

Proposal 5: For PC1.5 UE supporting two band EN-DC with 2Tx,  define new duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission if PC2 E-UTRA TDD + PC2 NR TDD is introduced. For example,

- E-UTRA TDD Uplink-downlink Configuration (X) = 5

- minimum percentage of 30% and 5% as 2 thresholds for maximum NR uplink transmission

For PC2 UE supporting two band EN-DC with 3Tx,

Proposal 6: For PC2 UE supporting two band EN-DC with 3Tx,  consider  a new capability if PC3 E-UTRA FDD + PC3 NR FDD is introduced. For example,

- maxUplinkDutyCycle-interBandENDC-FDD-PC2

  - comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 7: For PC2 UE supporting two band EN-DC with 3Tx, consider a minimum percentage of 20% as threshold for maximum NR uplink transmission if PC2 E-UTRA TDD + PC3 NR TDD is introduced.

For PC1.5 UE supporting two band EN-DC with 3Tx,

Proposal 8: For PC1.5 UE supporting two band EN-DC with 3Tx, define new duty cycle solutios considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission if the following band combinations are introduce.

- PC3 E-UTRA FDD/TDD + PC1.5 NR TDD

- PC2 E-UTRA TDD + PC2/PC1.5 NR TDD

For example of PC2 E-UTRA TDD + PC2 NR TDD,

- E-UTRA TDD Uplink-downlink Configuration (X) = 5

- minimum percentage of 30% and 5% as 2 thresholds for maximum NR uplink transmission

MSD rules

General understanding on MSD rules

Observation 1: Option 1, 3, and 4 could significantly reduce the workload related to the HPUE basket and also considerably decrease the amount of the MSD table for HPUE in the current specifications. However, a common equation for HPUE MSD is needed, and it seems challenging to address all HPUE MSDs with a single equation.

Observation 2: Option 2, which maintains the existing approach, can accurately define the MSD by reflecting the chacteristics of HPUE in specific band combinations, but it has a significant workload issue that existes in the current basket work, and large tables for HPUE remain in the specifications, which could make the specifications increasingly complex.

Observation 3: If UL interference for HPUE increases linearly, the MSD value for HPUE can be easily derived based on the default MSD value.

Harmonic & Harmonic mixing

Observation 4: As shown in Table 3.1 and Table 3.2, when the power of the UL aggressor is increased by 3 dB in harmonic and harmonic mixing, MSD is increased by approximately 0.7 to 4.4 dB, 3 dB in many cases. This indicates that the increase in UL aggressor power is similar to the increase in UL harmonic interference power.

Proposal 9: The simple approaches (LUT, equation  ...) can be applied for harmonic & harmonic mixing MSD requirements of HPUE.

Inter-modulation

Observation 5: The IMD MSD requirement is more complex than other types of MSD requirement. The reasons why it is difficult to derive the IMD MSD value using a simple equation are shown below.

- Numerous band combinations are defined in the specification, and the RF characteristics of the UE vary depending on the band combination

- IMD MSDs are defined for IMD2 to IMD9 in the current specification, and as the order of the influencing IMD increases, the number of case for IMD products also increases. Even for MSDs of the same IMD order, the MSD values can vary significantly depending on the IMD product. For example, the products of IMD4 can be (3,1) and (2,2) …, where the case of (3,1) can have a much larger MSD value than (2,2).

- Since MSD values are calculated using various methods such as measurement, simulation, and hand calculation with different RF assumption, it may be difficult to express them with a simple equation.

Proposal 10: Do not apply the simple approaches for IMD MSD requirements of HPUE.

Cross band isolation

Observation 6: The cross band isolation MSD values exhibit the following trends in current specification.

- The MSD difference between each power class is 0.4~3 dB.

- There are many MSDs that are not defined in the default power class (PC3) but are defined only in HPUE (PC2, PC1.5).

Proposal 11: Need study whether the simple approaches can be applied for cross band isolation MSD requirement of HPUE.

- References

[1] R4-2417098, “WF on HPUE RF requirements”, Samsung, Skyworks, Nokia

[2] R4-2414277, “WF on HPUE for CA in TN”, Samsung

[3] R4-2410750, “WF on higher power UE”, Samsung

[4] RP-240828, “New WID : UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, ATT