3GPP TSG-RAN4 Meeting #113	R4-2418055

# Orlando, US, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	AdditionalSpectrumEmission in NR SL Pre-configuration

Agenda item:	5.20.1

Document for:	Approval

Introduction

In this paper, we provide our view on NS value for NR sidelink which has been missing in NR SL Pre-configuration [1].

Discussion

For signalling of NS value, as of now, NS_33 and NS_52 are specified for NR V2X in RAN4 specification as follows.

[TS38.101-1] 6.2E.3 UE additional maximum output power reduction for V2X 6.2E.3.1 General For the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in clause 6.2E.2 and A-MPR requirements specified in current clause. Additional emission requirements can be indicated by the network or pre-configured radio parameters. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field [additionalSpectrumEmission]. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR V2X frequency band number of the applicable operating band, the IE field [freqBandIndicatorNR] and an associated value of [additionalSpectrumEmission] in the relevant RRC information elements [7].

However, the field ‘AdditionalSpectrumEmission’ has been missing in Pre-configurable parameters in NR sidelink [1] even though it was specified in E-UTRA V2X [2]. It makes problematic situation for NR sidelink in out of coverage. Therefore, it should be informed to RAN2. If possible, the field ‘AdditionalSpectrumEmission’ in Pre-configurable parameters should be supported from Rel-16.

Proposal 1: Send LS to RAN2 to introduce ’AdditionlSpectrumEmission’ in NR sidelink pre-configured parameters to align with the existing RAN4 requirement.

[TS 36.331] 9.3.2 Pre-configurable parameters This ASN.1 segment is the start of the E-UTRA definitions of pre-configured sidelink parameters. NOTE 1: Upper layers are assumed to provide a set of pre-configured parameters that are valid at the current UE location if any, see TS 24.334 [69], clause 10.2. -- ASN1START EUTRA-Sidelink-Preconf DEFINITIONS AUTOMATIC TAGS ::= BEGIN IMPORTS AdditionalSpectrumEmission, AdditionalSpectrumEmission-v10l0, ARFCN-ValueEUTRA-r9, FilterCoefficient, maxCBR-Level-r14, maxCBR-Level-1-r14, maxFreq, maxFreqV2X-r14, maxSL-TxPool-r12, maxSL-CommRxPoolPreconf-v1310, maxSL-CommTxPoolPreconf-v1310, maxSL-DiscRxPoolPreconf-r13, maxSL-DiscTxPoolPreconf-r13, maxSL-V2X-CBRConfig2-r14, maxSL-V2X-CBRConfig2-1-r14, maxSL-V2X-RxPoolPreconf-r14, maxSL-V2X-TxConfig2-r14, maxSL-V2X-TxConfig2-1-r14, maxSL-V2X-TxPoolPreconf-r14, MCS-PSSCH-Range-r15, P-Max, ReselectionInfoRelay-r13, SL-AnchorCarrierFreqList-V2X-r14, SL-CBR-Levels-Config-r14, SL-CBR-PSSCH-TxConfig-r14, SL-CommTxPoolSensingConfig-r14, SL-CP-Len-r12, SL-HoppingConfigComm-r12, SL-NR-AnchorCarrierFreqList-r16, SL-OffsetIndicator-r12, SL-OffsetIndicatorSync-r12, SL-OffsetIndicatorSync-v1430, SL-PeriodComm-r12, RSRP-RangeSL3-r12, SL-MinT2ValueList-r15, SL-PriorityList-r13, SL-TF-ResourceConfig-r12, SL-TRPT-Subset-r12, SL-TxParameters-r12, SL-ZoneConfig-r14, P0-SL-r12, TDD-ConfigSL-r12, SubframeBitmapSL-r14, SL-P2X-ResourceSelectionConfig-r14, SL-RestrictResourceReservationPeriodList-r14, SL-SyncAllowed-r14, SL-OffsetIndicatorSync-r14, SL-Priority-r13, SL-V2X-FreqSelectionConfigList-r15, SL-V2X-PacketDuplicationConfig-r15, SL-V2X-SyncFreqList-r15 FROM EUTRA-RRC-Definitions; -- ASN1STOP – SL-Preconfiguration The IE SL-Preconfiguration includes the sidelink pre-configured parameters. SL-Preconfiguration information elements -- ASN1START SL-Preconfiguration-r12 ::= SEQUENCE { preconfigGeneral-r12 SL-PreconfigGeneral-r12, preconfigSync-r12 SL-PreconfigSync-r12, preconfigComm-r12 SL-PreconfigCommPoolList4-r12, ..., [[ preconfigComm-v1310 SEQUENCE { commRxPoolList-r13 SL-PreconfigCommRxPoolList-r13, commTxPoolList-r13 SL-PreconfigCommTxPoolList-r13 OPTIONAL } OPTIONAL, preconfigDisc-r13 SEQUENCE { discRxPoolList-r13 SL-PreconfigDiscRxPoolList-r13, discTxPoolList-r13 SL-PreconfigDiscTxPoolList-r13 OPTIONAL } OPTIONAL, preconfigRelay-r13 SL-PreconfigRelay-r13 OPTIONAL ]] } SL-PreconfigGeneral-r12 ::= SEQUENCE { -- PDCP configuration rohc-Profiles-r12 SEQUENCE { profile0x0001-r12 BOOLEAN, profile0x0002-r12 BOOLEAN, profile0x0004-r12 BOOLEAN, profile0x0006-r12 BOOLEAN, profile0x0101-r12 BOOLEAN, profile0x0102-r12 BOOLEAN, profile0x0104-r12 BOOLEAN }, -- Physical configuration carrierFreq-r12 ARFCN-ValueEUTRA-r9, maxTxPower-r12 P-Max, additionalSpectrumEmission-r12 AdditionalSpectrumEmission, sl-bandwidth-r12 ENUMERATED {n6, n15, n25, n50, n75, n100}, tdd-ConfigSL-r12 TDD-ConfigSL-r12, reserved-r12 BIT STRING (SIZE (19)), ..., [[ additionalSpectrumEmission-v1440 AdditionalSpectrumEmission-v10l0 OPTIONAL ]] } SL-PreconfigSync-r12 ::= SEQUENCE { syncCP-Len-r12 SL-CP-Len-r12, syncOffsetIndicator1-r12 SL-OffsetIndicatorSync-r12, syncOffsetIndicator2-r12 SL-OffsetIndicatorSync-r12, syncTxParameters-r12 P0-SL-r12, syncTxThreshOoC-r12 RSRP-RangeSL3-r12, filterCoefficient-r12 FilterCoefficient, syncRefMinHyst-r12 ENUMERATED {dB0, dB3, dB6, dB9, dB12}, syncRefDiffHyst-r12 ENUMERATED {dB0, dB3, dB6, dB9, dB12, dBinf}, ..., [[ syncTxPeriodic-r13 ENUMERATED {true} OPTIONAL ]] } SL-PreconfigCommPoolList4-r12 ::= SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-PreconfigCommPool-r12 SL-PreconfigCommRxPoolList-r13 ::= SEQUENCE (SIZE (1..maxSL-CommRxPoolPreconf-v1310)) OF SL-PreconfigCommPool-r12 SL-PreconfigCommTxPoolList-r13 ::= SEQUENCE (SIZE (1..maxSL-CommTxPoolPreconf-v1310)) OF SL-PreconfigCommPool-r12 SL-PreconfigCommPool-r12 ::= SEQUENCE { -- This IE is same as SL-CommResourcePool with rxParametersNCell absent sc-CP-Len-r12 SL-CP-Len-r12, sc-Period-r12 SL-PeriodComm-r12, sc-TF-ResourceConfig-r12 SL-TF-ResourceConfig-r12, sc-TxParameters-r12 P0-SL-r12, data-CP-Len-r12 SL-CP-Len-r12, data-TF-ResourceConfig-r12 SL-TF-ResourceConfig-r12, dataHoppingConfig-r12 SL-HoppingConfigComm-r12, dataTxParameters-r12 P0-SL-r12, trpt-Subset-r12 SL-TRPT-Subset-r12, ..., [[ priorityList-r13 SL-PriorityList-r13 OPTIONAL -- For Tx ]] } SL-Preconfiguration field descriptions. additionalSpectrumEmission The UE requirements related to IE AdditionalSpectrumEmission are defined in TS 36.101 [42], clause 6.2.4. If additionalSpectrumEmissionExt-r14 is configured, the UE only considers additionalSpectrumEmissionExt-r14 (and ignores additionalSpectrumEmission-r12)..

Conclusion

We proposed as follows.

Proposal 1: Send LS to RAN2 to introduce ’AdditionlSpectrumEmission’ in NR sidelink pre-configured parameters to align with the existing RAN4 requirement.

- References

[1] TS 38.331

[2] TS 36.331