3GPP TSG RAN WG4 #113	R4-2418069

Orlando, US, November 18 – 22, 2024

Agenda item:		7.20.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SBFD operation

Document for:		Discussion

# Introduction

At RAN4#112bis meeting, the impacts on SSB based measurement and CSI-RS based measurement due to SBFD operation were identified in the WF [1]. In this contribution, we are discussing the open issues and providing our initial views on the RRM impacts due to SBFD operation based on the progress in other WGs.

# Discussion

SBFD is being specified in Rel-19 as a new mode of operation in unpaired spectrum where separate set of RBs within a TDD carrier are used for UL and DL transmission simultaneously. RRM requirements for SBFD operation have been discussed in RAN4#112 and RAN1#112bis, where multiple agreements have been reached. In the following subsections, we discuss some issues as listed in the Way forward (WF) from RAN4#112bis.

## Requirements for SSB based measurement

The following was agreed in RAN4#112:

Issue 2-3-2: Requirements for SSB based measurement Agreements (from online session) - For SBFD-aware UE, existing requirements apply for SSB-based serving cell measurement. Further discuss for SSB based neighbour cell measurement in RAN4. - Note: The serving cell is SBFD cell, and the neighbour cell is SBFD or non-SBFD cell.

Furthermore, RAN4 agreed the following in RAN4#112bis:

Issue 1-2-1: Requirements for SSB based measurement Agreements (from online session) - Down-select one of the two options: - Option 1: - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements. - Option 2: - For inter-frequency measurement without gap, UL transmission is prioritized. - Follow option 1 in other cases.

As noted above, RAN4 initially agreed that existing requirements shall be applied for SSB-based measurements in the serving cell; this was motivated by the RAN1 agreement in which the SSB reception is prioritized over UL transmissions. For the SSB-based neighbour cell measurement, SSB transmission is also prioritized in the neighbour cell and will be measured as in legacy. The UE is expected to measure all the cells on the MO including the SBFD neighbour cell. Hence we think existing measurement requirements shall be applied to SSB based neighbour cell measurement as well irrespective if it is an SBFD cell or not, i.e. Option 1 above.

Considering option 2, we see two aspects to be discussed. The first one is related to the existing scheduling availability specified in TS 38.133, clause 9.3.9. Currently, the scheduling availability during inter-frequency measurements without measurement gaps is clarified in two cases: when the SSB is completely contained in the active BWP of the UE and when the SSB is not completely contained in the active BWP of the UE. In both cases, the UE is not expected to transmit UL during a subset of symbols that overlap with SSBs to be measured. For example, for the case in which the SSB is completely contained in the active BWP of the UE, the restricted symbols are SSB symbols to be measured, RSSI measurement symbols and 1 data symbol before/after these symbols. We do not expect any optimization for SBFD.

The second aspect to be discussed about option 2, is related to the existing measurement period. If there are some UL transmissions prioritized over SSB measurements during inter-frequency measurements without gaps, this would impact the existing requirements on the measurement periods, which are currently specified as a function of the SMTC periodicity. If the UL transmissions are prioritized, the measurement period will need to be revisited.

For SSB based neighbor cell measurements, adopt Option 1:

- As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission.

- This can be revisited if it proves to be conflicting with further RAN1 agreements.

## Requirements for CSI-RS based measurement

The following way forward was agreed in RAN4#112bis regarding this topic:

Issue 1-2-2: Requirements for CSI-RS based measurement - Agreements (from online session) - RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations. - CSI-RS colliding with dynamically scheduled UL transmission - CSI-RS puncturing in frequency domain in SBFD symbols - CSI-RS distributed over SBFD and non-SBFD symbols - Align the understanding whether above SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI. - RAN4 to discuss whether and how L3 CSI-RS measurement requirements would be impacted by SBFD operations, and further RAN1/2 conclusions can be considered.

RAN1 has reached quite a few agreements regarding CSI-RS transmission on SBFD symbols which are listed in the following:

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

Essentially, this means that CSI-RS signals are configured according to the legacy way (start and end RB, density, etc.), but only the CSI-RS resources in the DL subband are measured by the UE and used to derive the report. The CSI-RS resources in the UL subband and guardbands are assumed not to be present, i.e. “punctured”. This principle is illustrated in Figure 1, where CSI-RS signals are transmitted in both non-SBFD and SBFD symbols, but the latter one is non-contiguous in frequency due to the puncturing operation.

As a first step in RAN4, it should be agreed that the measurement of CSI-RS signals according to RAN4 specifications should be aligned with the RAN1 agreements: i.e. SBFD-aware UE measures only CSI-RS frequency resources within DL usable PRBs.

According to RAN1 agreement, only CSI-RS resources within DL subbands are measured by the UE and used to derive the report.

For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs.

Figure 1: Puncturing of CSI-RS due to SBFD DUD operation

On whether SBFD CSI-RS operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI:

The above agreements from RAN1 refer to the transmission of a CSI-RS resource in general, and do not explicitly specify whether the CSI-RS is used for the purposes of CSI determination, L1-RSRP or for e.g. RLM or BFR purposes. Meanwhile the CSI-RS resources used for RLM, BFR and L1-RSRP are all configured in CSI-MeasConfig. From this point of view, we think RAN4 should assume that SBFD CSI-RS operations as per RAN1 agreements are also applicable to CSI-RS for RLM, BFR and L1-RSRP, and this could be revisited if needed depending on further RAN1 progress:

The following three SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP:

- CSI-RS colliding with dynamically scheduled UL transmission

- CSI-RS puncturing in frequency domain in SBFD symbols

- CSI-RS distributed over SBFD and non-SBFD symbols

CSI-RS distributed over SBFD and non-SBFD symbols:

RAN4 TS 38.133 specify the measurement delay requirements related to CSI-RS based measurements and reporting. In particular, the measurement delay defines how long time it should take for the UE to derive a qualified CSI-RS based measurement result and is counted by a number of measurement samples. This includes CSI-RS based L3 measurements for mobility, CSI-RS based L1-RSRP measurement, and CSI-RS based RLM/BFD measurement etc.

When SBFD operation is considered, it may happen a periodic CSI-RS signal takes place in both SBFD and non-SBFD symbol types. For the CSI-RS resource in SBFD symbols, if it is overlapping with UL subband, UE is expected to measure a less number of PRBs due to puncturing behaviour hence the number of PRBs i.e. the CSI-RS physical resources to be measured for the configured CSI-RS resource may be different on SBFD and non-SBFD symbols.

RAN4 shall discuss if the measurement samples over SBFD and non-SBFD symbols can be combined when defining the CSI-RS based measurement requirement.

The number of PRBs i.e. the CSI-RS physical resources measured for the configured CSI-RS resource may be different on SBFD and non-SBFD symbols.

RAN4 shall discuss if the measurement samples over SBFD and non-SBFD symbols can be combined when defining the CSI-RS based measurement requirement.

CSI-RS puncturing in frequency domain in SBFD symbols:

On the other hand, there are measurement accuracy requirements specifying the absolute and/or relative accuracy that CSI-RS based measurement reporting shall meet. In TS38.133 clause 10, “the bandwidth of CSI-RS is 48 PRBs and the density is 3” has been assumed. With respect to these RRM requirements, the following issues are foreseen due to SBFD operation:

- The “residual” amount of CSI-RS PRBs on SBFD symbols after puncturing operation may be smaller than 48 PRBs, which is a prerequisite for fulfilling many of the RAN4 RRM requirements.

- Even if the “residual” amount of CSI-RS PRBs is larger or equal than 48 PRBs, the CSI-RS resources may be split across two disjoint DL subbands (i.e. the CSI-RS is not contiguous in frequency).

In our view, if the CSI-RS resource is within one of the DL subbands or there are at least 48 consecutive CSI-RS PRBs in one of the DL subbands, RAN4 should agree that the existing requirements shall apply. Note that this is a reasonable scenario since SBFD is primarily suitable for large carrier bandwidths (e.g. 50-100 MHz in FR1).

The “residual” amount of CSI-RS PRBs on SBFD symbols after puncturing operation may be smaller than 48 PRBs, which is a prerequisite for fulfilling many of the RAN4 RRM requirements.

For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (i.e. within one of the DL subbands) is no less than 48 PRBs.

Now, if the CSI-RS PRBs are split across two DL subbands, RAN4 should discuss whether or not existing measurement period and accuracy requirements can still be applied. If the residual CSI-RS RB bandwidth is below 48 PRBs, we think existing requirements should be either modified (relaxed) or not be applicable at all. If the residual CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, we think the existing requirements can be applied but we are also open to hear if additional side conditions need to be defined.

For CSI-RS on SBFD symbols, existing accuracy requirements do not apply if the residual CSI-RS bandwidth (number of PRBs) after applying the puncturing operation is below 48 PRBs.

For CSI-RS on SBFD symbols, if the residual CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, RAN4 to discuss if existing accuracy requirements for CSI-RS based measurements can apply and whether additional side conditions need to be specified.

CSI-RS colliding with dynamically scheduled UL transmission:

As discussed in details in our previous contribution R4-2415573, RAN1 has agreed that in SBFD symbols the UE would prioritize a dynamically-scheduled UL transmission (i.e. triggered by DCI) over a semi-statically configured DL signal, e.g. periodic CSI-RS (see Figure 2). This does not happen in static TDD operation since symbols are exclusively used for one link direction (either UL or DL) and this link direction is fixed well in advance. Similar as how the measurement delay is impacted by the overlapping of CSI-RS with measurement gaps, we think similar principles shall be applied for this collision case.

According to RAN1 agreements, UE should prioritize a dynamically-scheduled UL transmission over a semi-statically configured DL signal, e.g. periodic CSI-RS.

RAN4 to discuss the impact on CSI-RS measurement requirements if a CSI-RS resource is not received by the UE due to overlapping dynamic-UL transmission.

Figure 2: Prioritization of dynamic UL transmission over a semi-static DL reception

Whether L3 CSI-RS measurement requirements are impacted by SBFD operation:

As noted above, RAN1 has agreed that CSI-RS are configured in a contiguous manner (i.e. as legacy) but only the CSI-RS resources in the DL usable RBs are assumed to be valid, while other resources are ‘punctured’. In our view, the RAN1 agreements apply to any CSI-RS transmission (at least) from the serving cell, regardless of whether the CSI-RS is configured for the purpose of L1 or L3 measurement.

However, how to perform CSI-RS measurements from neighbour cells (e.g. whether the puncturing is to be applied or not) is unclear considering the neighbour cells may or may not operate in SBFD mode. In order to measure CSI-RS from neighbor cell, UE needs to be aware of the SBFD configuration from neighbour which we understood not yet agreed in RAN1/2. RAN4 can wait for RAN1/2 discussion on the feasibility before discussing the RRM requirements for L3 CSI-RS based measurements.

RAN1 agreements on CSI-RS transmission in SBFD symbols are applicable to any CSI-RS transmission from the serving cell, regardless of whether the CSI-RS is configured for the purpose of L1 or L3 measurement. Whether L3 CSI-RS neighbour cell measurements are impacted by SBFD requires further clarification from RAN2 and/or RAN1.

RAN4 to wait for RAN1/2 discussion on the CSI-RS resources for mobility before discussing the RRM requirements for L3 CSI-RS based measurements.

# Conclusion

In this contribution, our views on the RRM requirements are presented. The following proposals were listed.

- For SSB based neighbor cell measurements, adopt Option 1:

- As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission.

- This can be revisited if it proves to be conflicting with further RAN1 agreements.

- According to RAN1 agreement, only CSI-RS resources within DL subbands are measured by the UE and used to derive the report.

For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs.

The following three SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP:

- CSI-RS colliding with dynamically scheduled UL transmission

- CSI-RS puncturing in frequency domain in SBFD symbols

- CSI-RS distributed over SBFD and non-SBFD symbols

The number of PRBs i.e. the CSI-RS physical resources measured for the configured CSI-RS resource may be different on SBFD and non-SBFD symbols.

RAN4 shall discuss if the measurement samples over SBFD and non-SBFD symbols can be combined when defining the CSI-RS based measurement requirement.

The “residual” amount of CSI-RS PRBs on SBFD symbols after puncturing operation may be smaller than 48 PRBs, which is a prerequisite for fulfilling many of the RAN4 RRM requirements.

For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (i.e. within one of the DL subbands) is no less than 48 PRBs.

For CSI-RS on SBFD symbols, existing accuracy requirements do not apply if the residual CSI-RS bandwidth (number of PRBs) after applying the puncturing operation is below 48 PRBs.

For CSI-RS on SBFD symbols, if the residual CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, RAN4 to discuss if existing accuracy requirements for CSI-RS based measurements can apply and whether additional side conditions need to be specified.

According to RAN1 agreements, UE should prioritize a dynamically-scheduled UL transmission over a semi-statically configured DL signal, e.g. periodic CSI-RS.

RAN4 to discuss the impact on CSI-RS measurement requirements if a CSI-RS resource is not received by the UE due to overlapping dynamic-UL transmission.

RAN1 agreements on CSI-RS transmission in SBFD symbols are applicable to any CSI-RS transmission from the serving cell, regardless of whether the CSI-RS is configured for the purpose of L1 or L3 measurement. Whether L3 CSI-RS neighbour cell measurements are impacted by SBFD requires further clarification from RAN2 and/or RAN1.

RAN4 to wait for RAN1/2 discussion on the CSI-RS resources for mobility before discussing the RRM requirements for L3 CSI-RS based measurements.

# References

- R4-2416858, WF on SBFD RRM, Huawei