3GPP TSG-RAN WG4 Meeting #113	R4-2418070

Orlando, US, November 18 – 22, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	FR2-1 SSB based L3 measurement delay reduction with CSSF optimization

Agenda item:	7.15.2.2

Document for:	Discussion

Introduction

The discussion about RRM phase 5 enhancements for L3 measurement delay reduction started in the RAN4#110bis meeting. In this contribution we continue the discussion on CSSF optimization based on the agreed way forward [1].

Discussion

WID clarification

At RAN#105 meeting, the RRM Phase 5 WID was revised to clarify the applicability of CSSF optimization [2]. It is now indicated that CSSF optimization is defined assuming a UE is not operating multi-Rx based L3 measurements. While this makes clear the dependency on multi-Rx “L3” measurements, we still see some ambiguity on the expected UE behaviour when CSSF optimization is enabled.

#### 4.1 Objective of SI or Core part WI or Testing part WI - FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, iItif feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

In our understanding, UEs are already allowed and may perform L3 measurements using multiple panels in parallel in their implementation. Such implementation is not prohibited but the UE requirements are based on the assumption of only requiring the UE using one panel. As the UE implementation behavior is unknown to network, network does not know if the UE is operating using multi-Rx for performing L3 measurements, and hence is not able to determine if the optimized CSSF would apply. In our view, the UE implementation or support of multi-Rx based L3 measurement (if available) shall not impact the applicability of CSSF optimization. In other words, a UE is assumed not operating with multi-Rx based L3 measurement unless it is known by network.

Network does not know if the UE is operating multi-Rx based L3 measurement in its implementation, hence network is not able to determine if the optimized CSSF would apply.

A UE is assumed not operating with multi-Rx based L3 measurement unless it is known by network.

In addition, a UE is understood to be able to operate with the multi-Rx L3 measurement under certain conditions. According to the revised WID, the UE may have to adjust the measurement behavior due to applying CSSF optimization or not based on if the multi-Rx L3 measurement is applicable (triggered) or not. To simplify the discussion, if a UE is configured with CA/DC, it can be assumed not operating with the multi-Rx based L3 measurement. Since the two topics are being discussed separately, the dependency in-between shall not impact the discussion on CSSF optimization solutions.

A UE may have to adjust the measurement behavior due to applicability of CSSF optimization based on if the multi-Rx L3 measurement is applied or not.

However, currently it is not clear when UE is required to apply mRx for L3 measurements and hence it is open when UE shall apply the CSSF optimization. If RAN4 can agree (which we believe was the initial intention) that if the UE is configured with CA or DC, the UE configured with CSSF optimization, applies CSSF optimization. When CSSF optimization is applied, the UE is not required to use mRx L3 measurements.

If a UE is configured with CSSF optimization (CA or DC), the UE is not required to operate with the multi-Rx based L3 measurement.

The independency between CSSF optimization and multi-Rx L3 measurement shall be clear. This would allow the discussion on the CSSF optimization solutions to proceed without impact from the multi-Rx L3 measurement discussion.

Measurement procedures

In last meeting, all the measurement procedures were agreed to be discussed. As the CSSF optimization implies new UE measurement behavior, it would be easier to move forward focusing on very basic measurement procedures. We prefer starting from SSB-based intra-frequency and inter-frequency measurements without MG to simplify the discussion. The applicability to other measurement procedures can be revisited after the baseline solution is concluded.

Issue 2-2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

To start from SSB-based intra-frequency and inter-frequency measurements without MG to simplify the CSSF optimization discussion.

- Prioritization of EN-DC/SA scenarios

At RAN4#112 meeting, it was agreed to consider the CSSF optimization for all the scenarios. There was further proposal to prioritize some of the scenarios as below. In our view, we can focus the discussion on the CSSF optimization solutions in SA scenarios. The same principle can be applied to update CSSF tables in other scenarios afterwards.

Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF - Proposal: Rel-19 CSSF optimization can be prioritized for the following cases: Current requirements in TS38.133; Priority. 9.1.5.1.1 EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps; Low. 9.1.5.1.2 SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps; high. 9.1.5.1.3 NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps; Low. 9.1.5.1.4 NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps; Low. 9.1.5.2.1 EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps; Low. 9.1.5.2.2 SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps; high. 9.1.5.2.3 NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps; Low. 9.1.5.2.4 NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps; Low. 9.1.5.2.5 SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps; high. 9.1.5.2.6 NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps; Low. 9.1.5.2.7 NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps; Low. 9.1.5.3 Monitoring of multiple layers within NCSG; Low.

To focus the discussion on the CSSF optimization solutions in SA scenarios. The same principle can be applied to update CSSF tables in other scenarios afterwards.

Searcher assumption

In the WID, it indicates the baseline assumption on number of searchers is 2 which is aligned with current assumption for deriving CSSF values. We understood the searcher assumption is up to UE capability, but the CSSF optimization solution we are defining shall be applicable at least for 2 searchers UEs.

The CSSF optimization solution we are defining shall be applicable at least for 2 searchers UEs.

In last meeting, it was further agreed to discuss the potential solutions based on 3 searchers. As this solution considers using one more i.e. the 3rd searcher to reduce the measurement delay, some new UE implementation may be needed to support this feature. Hence we suggest defining a new UE capability to indicate the support for 3 searchers.

Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF Agreement in online session: On top of the previous agreement on the baseline assumption of 2 searchers, for 3 searchers: - Discuss the applicable scenario(s) and/or solution(s) based on 3 searchers. Decide whether or not to define the requirement later. Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC - Option 3: consider to use 3 searchers for CSSF reduction. - Option 3a(CATT): Three searchers assumption for UE supporting per-FR gap. For option 3, define the applicable scenarios (e.g., both FR1 and FR2 measurement objects are configured). - Option 3b(HW): - RAN4 RRM can define a new UE capability to indicate whether the UE supports up to three searchers. - The 3-searcher solution can be utilised under the following conditions - When all MOs need to be measured outside the gap. - Other conditions can be FFS. - Option 3c(QC): RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS).

To define a new UE capability to indicate the support for 3 searchers.

Regarding to the 3 searchers, some companies explained this is not newly introduced for CSSF optimization in R19. Instead, it is already implemented as implied from the measurement behavior of per-FR gap capable UE hence is only applicable to UEs supporting per-FR gaps. In our view, if the UE indicates the capability of 3 searchers, it shall apply the measurement requirement defined assuming 3 searchers. Even if the UE does not support per-FR gap, the UE may also implement 3 searchers and be able to perform faster measurement fulfilling a measurement delay requirement based on 3 searchers. To simplify the discussion, the 3 searcher solution does not have to be dependent on per-FR gap capability i.e. it is independent from per-FR gap or per-UE gap capability.

For UE indicating the new UE capability, it shall apply the measurement requirement defined based on 3 searchers.

The 3 searcher solution is independent from per-FR gap or per-UE gap capability.

If the UE indicates the capability supporting 3 searchers, we can consider using one more searcher for the measurement outside gaps where 2 searchers have been assumed. In any case, the solution based on 3 searchers shall target reducing the measurement delay for the measurement outside gaps and should not relax the measurement delay requirements assuming 2 searchers.

The solution based on 3 searchers should not relax the measurement requirements assuming 2 searchers.

In any case, the discussion should focus on how the 3 searchers are assumed or used for the SSB-based measurement outside gaps. In case of 2 searchers, we have been assuming one searcher for PCC/PSCC and the other searcher for SCCs. When 3 searchers are available, we need align the assumption for the 3 searchers before discussing the CSSF values.

RAN4 to focus the discussion on how the 3 searchers are assumed for the SSB-based measurement outside gaps.

Solutions for CSSF optimization

- Measuring single CC per band

Some candidate solutions have been proposed for CSSF optimization. The most popular one is to have UE measure only one serving CC per band if there are multiple serving CCs in the same band i.e. Option 1 as listed below. Two candidate options are now concluded based on UE determination i.e. Option 1A or network indication i.e. Option 1B.

Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC Candidate options for further discussion: - Option 1: UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CCs to be measurement, further discuss option 1A and option 1B - Option 1A: Reuse the legacy principle for neighboring cell measurement. - Option 1B: Based on network indication - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band. - Option 2: UE can change the searcher occupancy ratio of measurement among the measured CCs - Option 2A: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions For 3 searchers: In addition to option 1 and 2 - Option 3: consider to use 3 searchers for CSSF reduction. - Depending on issue 2-4.

According to the requirements for FR2 measurement in TS 38.133 clause 9.2.3.2, the UE is required to perform a full measurement on a PCC/PSCC or one of the SCCs. While on the other intra-frequency layers in the same band, a relaxed measurement is allowed as the UE is required to measure 2 SSBs on serving cell i.e. the measurement on neighbor cells on the SCC may be skipped.

TS38.133:

#### 9.2.3.2	Requirements for FR2

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band; or

-	PSCC when UE is configured with EN-DC with PSCC in the band; or

-	PSCC when UE is configured with NR-DC with PSCC in the band; or

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

In existing specification, UE is allowed to skip the neighbour cell measurements on some SCCs in the same band, but still required to measure all the SCCs configured with serving cell measurements.

In Option 1, it intends to apply the same measurement relaxation i.e. to allow the UE skip some of the serving cell measurements in order to reduce the measurement delay. We believe there is potential to minimize the measurements on multiple CCs in some scenarios e.g. FR2 intra-band, but the UE measurement behavior needs to be under network control as the measurement results are used for network decision on mobility etc. Network needs to know exactly which CCs the UE has measured and which CCs the UE does not measure.

The measurement behavior for CSSF optimization needs to be based on network indication i.e. Option 1B is preferred.

As for now, the measurements on serving cells are up to network configuration. According to TS 38.331 clause 5.5, the measurements on serving cells are configured by the network via measConfig and servingCellMO wherein the configuration of servingCellMO is mandatory for PCell/PSCell but optional for SCells. In particular, if a UE is configured with servingCellMO on a serving cell, it is required to measure the associated measObjectID as indicated in measConfig. Otherwise, the UE is not required to measure on the cell. Therefore, the candidate solution of Option 1 can be implemented via network configuration e.g. to configure the UE with servingCellMO only on one CC per band and no MOs are configured on the other CCs, if there are multiple CCs on the same band.

TS38.331 clause 5.5:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

The candidate solution of Option 1 can be implemented via network configuration e.g. to configure the UE with servingCellMO only on one CC per band and no MOs are configured on the other CCs, if there are multiple CCs on the same band.

While existing measurement framework can already be used to minimize the UE measurements on multiple CCs, there are some drawbacks with such network configuration. If the UE is configured with single MO per band, the UE is not expected to measure the other CCs nor sending any measurement report associated with these CCs. In current network deployment, network cannot configure any measurement events on these CCs due to lack of MOs, hence not possible to command handover to any of these cells including serving and neighbor cells.

Since any network action is likely based on the assistance measurement reporting from the UE including the potential target cell, and the UE requirements are conditioned on UE having sent a measurement report for the target cell, there is benefit in having UE being triggering the measurement reporting for the carriers not (being) measured or not configured with MOs. In order to ensure the mobility performance and minimize the impact to network behaviour, RAN4 shall discuss how to handle the measurement reporting if the UE is not configured with measurements on the CCs.

If the UE is configured with single MO per band as in legacy, the UE is not able to trigger a measurement report for a CC which the UE is not measuring, and network cannot command handover to any of the cells on the CC.

To minimize the impact to UE and network behavior, it is simpler to allow the network to configure multiple MOs, but to reduce the measurement efforts and delay, network can further indicate on which of the MOs the measurement is to be performed. Based on the indication, the UE will measure the corresponding CCs indicated and network can trigger measurement reporting on any of the MOs as in legacy.

Network indicates which of the MOs to be measured. The UE is required to measure the indicated CCs i.e. MO and network can trigger measurement reporting on any of the MOs as in legacy

- Searcher occupancy ratio

This has been raised to allow the UE to adjust the searcher occupancy ratio of PCC/PSCC measurement. In our understanding, currently one searcher is dedicatedly used for PCC/PSCC measurement, and the other searcher is for the measurement on all SCCs. If the searcher occupancy ratio of PCC/PSCC measurement is changed, this would lead to extended measurement delay on PCC/PSCC which is not acceptable. While we can further discuss the details to enable the change of search occupancy ratio, but the solution shall not relax the measurement requirement on PCC/PSCC measurement.

The solution of changing searcher occupancy ratio shall not relax the measurement requirement on PCC/PSCC measurement.

- Other candidate solutions

For the measurements on SCCs, UE has been assumed to share one searcher and perform the measurements over multiple SCCs in alternating manner. This leads an increased measurement delay being scaled by the number of CCs to be measured. However, the UE is able to receive simultaneously from the serving CCs e.g. in CA operation, hence it is able to receive SSBs from multiple configured CCs at the same time e.g. within one SMTC window if it is configured with same SMTC on the multiple CCs. This will allow the UE to get the SSB samplings from the multiple CCs in one shot. Afterwards, the UE may process the samples in sequential way offline in order to derive the measurement results for the multiple CCs respectively.

With such measurement behaviour, some additional processing time may be needed but the measurement delay is not scaled by the number of configured CCs hence can be significantly decreased. We would like to define the optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

UE is able to receive SSBs (i.e. get SSB samplings) from multiple configured CCs at the same time e.g. within one SMTC window.

RAN4 to define optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

In addition, the CSSF values are determined considering not only SSB-based measurement but also the CSI-RS based L3 measurements. Although the WID targets SSB based L3 measurement delay reduction, the CSI-RS based L3 measurements contribute to the CSSF values via 2xNSCC_CSIRS i.e. twice of the number of SCells configured with CSI-RS based measurement. This is based on the R16 assumption that one CSI-RS based measurement requires an additional measurement on its associated SSB.

The CSI-RS based L3 measurements contribute to the CSSF values via 2xNSCC_CSIRS, i.e. twice of the number of SCells configured with CSI-RS based L3 measurement.

However, the CSI-RS resources to be measured are up to network configuration which may not be overlapping with the associated SSB. In some cases, the CSI-RS based measurements may not impact the SSB-based measurements e.g. if they are not overlapping in time, hence the CSSF values can be further optimized. We may study the CSSF optimization by minimizing the impact from CSI-RS based measurements on SSB-based measurements.

To consider the CSSF optimization by minimizing the impact from CSI-RS based measurements on SSB-based measurements.

Capability issues

In current specification, CSSF values have been derived to scale the measurement delay requirements when UE is configured to monitor multiple measurement objects. When CSSF optimization is considered in R19, a reduced CSSF implies some new UE measurement behaviour pending on the CSSF optimization solution e.g. measuring single CC per band. We believe this shall be applied to the UE starting from release 19.

In R19, a reduced CSSF implies some new UE measurement behaviour pending on the CSSF optimization solution.

Meanwhile, network may also use the CSSF values to estimate the UE measurement behaviour and make network decision. When reduced CSSF is applied, the network behaviour may need to be adapted accordingly. It is not possible to apply new network behaviour without knowing about the UE’s support of the reduced CSSF. The UE capability is needed to inform network the support of reduced CSSF.

When reduced CSSF is applied, network behaviour may need to be adapted based on the UE indication of its support of the reduced CSSF.

A UE capability is needed to inform network the support of CSSF optimization.

Conclusion

In this paper the following observations and proposals were made:

- Network does not know if the UE is operating multi-Rx based L3 measurement in its implementation, hence network is not able to determine if the optimized CSSF would apply.

- A UE is assumed not operating with multi-Rx based L3 measurement unless it is known by network.

A UE may have to adjust the measurement behavior due to applicability of CSSF optimization based on if the multi-Rx L3 measurement is applied or not.

If a UE is configured with CSSF optimization (CA or DC), the UE is not required to operate with the multi-Rx based L3 measurement.

The independency between CSSF optimization and multi-Rx L3 measurement shall be clear. This would allow the discussion on the CSSF optimization solutions to proceed without impact from the multi-Rx L3 measurement discussion.

To start from SSB-based intra-frequency and inter-frequency measurements without MG to simplify the CSSF optimization discussion.

To focus the discussion on the CSSF optimization solutions in SA scenarios. The same principle can be applied to update CSSF tables in other scenarios afterwards.

The CSSF optimization solution we are defining shall be applicable at least for 2 searchers UEs.

To define a new UE capability to indicate the support for 3 searchers.

For UE indicating the new UE capability, it shall apply the measurement requirement defined based on 3 searchers.

The 3 searcher solution is independent from per-FR gap or per-UE gap capability.

The solution based on 3 searchers should not relax the measurement requirements assuming 2 searchers.

RAN4 to focus the discussion on how the 3 searchers are assumed for the SSB-based measurement outside gaps.

In existing specification, UE is allowed to skip the neighbour cell measurements on some SCCs in the same band, but still required to measure all the SCCs configured with serving cell measurements.

The measurement behavior for CSSF optimization needs to be based on network indication i.e. Option 1B is preferred.

The candidate solution of Option 1 can be implemented via network configuration e.g. to configure the UE with servingCellMO only on one CC per band and no MOs are configured on the other CCs, if there are multiple CCs on the same band.

If the UE is configured with single MO per band as in legacy, the UE is not able to trigger a measurement report for a CC which the UE is not measuring, and network cannot command handover to any of the cells on the CC.

Network indicates which of the MOs to be measured. The UE is required to measure the indicated CCs i.e. MO and network can trigger measurement reporting on any of the MOs as in legacy

The solution of changing searcher occupancy ratio shall not relax the measurement requirement on PCC/PSCC measurement.

- UE is able to receive SSBs (i.e. get SSB samplings) from multiple configured CCs at the same time e.g. within one SMTC window.

- RAN4 to define optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

The CSI-RS based L3 measurements contribute to the CSSF values via 2xNSCC_CSIRS, i.e. twice of the number of SCells configured with CSI-RS based L3 measurement.

To consider the CSSF optimization by minimizing the impact from CSI-RS based measurements on SSB-based measurements.

In R19, a reduced CSSF implies some new UE measurement behaviour pending on the CSSF optimization solution.

When reduced CSSF is applied, network behaviour may need to be adapted based on the UE indication of its support of the reduced CSSF.

A UE capability is needed to inform network the support of CSSF optimization.

- References

- R4-2414026, Way Forward for NR_RRM_Ph5_Part1, Apple

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5