3GPP TSG-RAN4 Meeting #113	R4-2418076

# Orlando, US, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	ATG UE RF requirements for UL-MIMO

Agenda item:	7.10.2.3

Document for:	Discussion

Introduction

In this paper, we provide our views on ATG UE RF requirements for UL-MIMO based on the approved WF[1].

Discussion

Based on the approved WF[1, 2], we would like to discuss ULFPTx mode and single antenna port related requirement for ATG UL-MIMO.

[1] Issue 3-1-1: ULFPTx mode for UL-MIMO WF: - Consider whether and how to introduce this requirement. - Option 1: ULFPTx mode 1/2/Full in the following table can be supported depending on UE PA implementation Table 2: Example of ATG ULFPTx (R-MOP = x dBm) ATG ULFPTx mode; PA1(dBm); PA2(dBm). Mode2; x; (x-3)~(x-1). Mode-Full; x; x. Mode1; (x-3)~(x-1); (x-3)~(x-1). - Option 2: Current ULFPTx mode requirements for legacy UE in TS38.101-1 can be reused for ATG UE, and there is no other specification affect. - Option 3: No need to extend ULFPTx mode to ATG UE. Issue 3-1-2: whether preclude the single antenna port related requirement WF: - Option 1: Yes. - Option 2: No. Issue 3-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18 Agreement in RAN4#102 meeting: - If the legacy requirement for UL MIMO is defined at each antenna connector, - For ATG UE with antenna array, the requirement should be updated [at each TAB connector]. - If the legacy requirement for UL MIMO is defined per layer, FFS - Option 1: for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer. - Option 2: for ATG UE with capability antennaArrayType-r18, the requirement should be updated per layer without explicitly emphasize the TAB connector.. Agreement: - If the legacy requirement for UL MIMO is defined at each antenna connector, - For ATG UE with antenna array, the requirement should be updated at each TAB connector - If the legacy requirement for UL MIMO is defined per layer - for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer Issue 3-2-2: whether to apply coherent UL MIMO requirement for ATG UE Agreement: - Since coherent UL MIMO is the optional requirement, it could be supported without specification impact. Issue 3-3-1: How to test Rx requirements Agreement: - No need to test the Rx requirements under the UL MIMO scenario for ATG UE, since general Rx requirements have been tested. Issue 3-3-2: How to test Tx requirements Agreement: - Further discuss the method of definition for general ATG Tx requirements referring to agreed CR R4-2414321 can be used. Issue 3-3-3: Transmit modulation quality for ATG Agreement: 6.4J.2 Transmit modulation quality for ATG The requirements for transmit modulation quality defined in clause 6.4.2 shall apply for ATG UE except for the phase continuity requirements for DMRS bundling in 6.4.2.5. And the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM. For ATG UE, the requirements for Error Vector Magnitude, Carrier leakage and EVM equalizer spectrum flatness are defined per layer. The requirements for In-band emission are defined at each transmit antenna connector or each TAB connector.. [2] 4-1-1: ULFPTx mode for UL-MIMO - FFS on whether to apply ULFPTx mode for ATG UE 4-1-2: whether preclude the single antenna port related requirement - FFS on whether to preclude the single antenna port related requirement. 4-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18 - If the legacy requirement for UL MIMO is defined at each antenna connector, - For ATG UE with antenna array, the requirement should be updated [at each TAB connector]. - If the legacy requirement for UL MIMO is defined per layer, FFS - Option 1: for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer. - Option 2: for ATG UE with capability antennaArrayType-r18, the requirement should be updated per layer without explicitly emphasize the TAB connector. 4-2-2: whether to apply coherent UL MIMO requirement for ATG UE - FFS on whether to apply coherent UL MIMO requirement for ATG UE 4-3: receiver requirements - Reuse ATG single carrier requirements for following Rx requirements (ZTE, Ericsson) - REFSENSE - Max input level - ACS - Blocking - Spurious emission - Intermodulation - FFS whether to test the Rx requirements under the UL MIMO scenario since Rx requirements is the same for R18 ATG UE and R19 ATG UE supporting UL-MIMO.

ULFPTx mode for UL-MIMO

In NR, ULFPTx mode is specified for PC2 and PC1.5 as follows.

- Single layer for 2 Tx port & 4 Tx port

  - PC2/PC1.5

- 2 layers for 4 Tx port

  - PC1.5

For ATG single layer UL MIMO operation and two antenna-ports, in case that R-MOP of 26dBm is declared for UL-MIMO, ULFPTx mode 1/2/Full can be supported as seen in Table 2.1.

.

Table 2.1: Example of ATG ULFPTx (R-MOP = 26dBm)

| ATG ULFPTx mode | PA1(dBm) | PA2(dBm) |
| --- | --- | --- |
| Mode2 | 26 | 23~25 |
| Mode-Full | 26 | 26 |
| Mode1 | 23~25 | 23~25 |

In WF[1], there is no big difference between Option 1 and Option 2. The existing ULFPTx mode requirements can be reused for ATG UE.

Proposal 1: Consider ULFPTx mode and reuse the existing ULFPTx mode requirements for ATG UL-MIMO.

Whether to preclude the single antenna port related requirement

The existing NR UL-MIMO requirements which are highlighted with yellow colour need to be considered for single layer.

For single layer UL MIMO operation and two antenna-ports,

- the rated maximum output power shall be met to a UE that supports ULFPTx feature and is configured in its declared full power mode.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity,

- the requirements in clause 6.2J apply for at least one antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.2J apply for at least one TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- the rated maximum output power applies for sum of two antenna connectors for the ATG UE not indicating the capability antennaArrayType-r18.

- the rated maximum output power applies for sum of two TAB connectors for the ATG UE indicating the capability antennaArrayType-r18.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity and the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16,

- the requirements in clause 6.2J apply for at least one antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.2J apply for at least one TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

Proposal 2: Consider the single antenna port requirement for single layer.

Conclusion

In this contribution, we provided our view on ULFPTx mode and single antenna port related requirement for ATG UL-MIMO.

Proposal 1: Consider ULFPTx mode and reuse the existing ULFPTx mode requirements for ATG UL-MIMO.

Proposal 2: Consider the single antenna port requirement for single layer.

- References

[1] R4-2417121, “WF on UE RF for ATG enhancement”, CMCC

[2] R4-2414322, “WF on Rel-19 ATG UE RF requirements”, CMCC

[3] RP-240839, “New WID on Enhancements for Air-to-ground network for NR”, Moderator(Apple)