3GPP TSG-RAN4 Meeting #113	R4-2418089

# Orlando, US, 18th – 22nd November, 2024

Title:	NC DL CA vs single CC ACS

Source:	Qualcomm Incorporated

Agenda item:	7.6.3

Release:	Rel-19

SI: 	FS_NR_FR1_DL_Frag_Carrier

Lead WG: 	RAN4

Document for:	Discussion

# 1.	Introduction

WF was approved in RAN4#112bis [1] on fragment DL carriers study. In this paper we discuss what is the a good reference requirement for the this study for ACS.

# 2. 	Discussion

WF [1] defines the scope of the study to be when span of CCs, outer edge to edge, to be 100 MHz. WF also encourages discuss PSD difference as a PSD difference between wanted and aggressor and encourages to start the evaluation as existing ACS as baseline.

- The [PSD] difference means the difference between unwanted signal and the wanted carrier with the lower PSD

- Start evaluation with wanted signal level currently existing ACS requirements as baseline and to see the performance difference

In the following, we discuss what is the current specifications for ACS for the NC DL CA

## 2.1	Current ACS specification for NC DL CA

The current specification defines UE ACS for contiguous DL CA for key parts as follows:

And for the case 1 parameters:

The table indicates that the aggressor power is dependent on the Aggregated power and wanted signal power is dependent on the REFSENS.

The non-contiguous DL CA clause 7.5.2  then specifies:

“For the lower range of test parameters (Case 1), the interferer power Pinterferer shall be set to the maximum of the levels given by the carriers of the respective sub-blocks as specified in Table 7.5-3 and Table 7.5A.1-2a for one component carrier and two component carriers per sub-block, respectively. The wanted signal power levels for the carriers of each sub-block shall then be adjusted relative to Pinterferer in accordance with the ACS requirement for each sub-block (Table 7.5-1 and Table 7.5A.1-1a).”

The highlighted is mildly confusing since it refers to single carrier ACS table 7.5-3 and the above presented contiguous CA ACS table 7.5A.1-2a but does not specify is the intention to take more stringent of each or more relaxed of each or maybe have two requirements. It has been there since 16.0.0 version of the specification. A correction or clarification could be worthy.

Nevertheless, RAN5 has taken the single carrier ACS as the only source for test specification:

Ran4 could use this as baseline ACS for NC DL CA.

As WF indicates that we can use the configurations in Table 7.3A.2.2-1 as reference. If the SCC has ΔRIBNC then the REFSENS is different and if the BWs of both wanted signals is same, then the PSD of the wanted signals is different.

Analysing the PSD difference for this case we come up with the Table 1 for n245(2A) for ΔRIB,NC of 24.6.

Table 1: Aggressor and wanted signal levels and PSDs for n25(2A) and n66(2A) for ACS

|  | REFSENS | Refsens NC CA With ΔRIB,NC | power mW | RBs | PSD mW | PSD dBm (per RB) | NC CA test PSD (REFSENS+14) | Aggressor PSD (REFSENS+39.5) | Aggressor power dBm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 5MHz SCC | -93.3 | -68.7 | 1.35E-07 | 25 | 5.39585E-09 | -82.68 | -68.68 | -23.17940009 | -9.2 |
| n25 40 MHz PCC | -79.5 | -79.5 | 1.12E-08 | 216 | 5.19E-11 | -102.845 | -88.84 |  |  |
| n66 20 MHz SCC | -93.3 | -93.3 | 4.68E-10 | 106 | 4.41E-12 | -113.6 | -99.55 | -60.05 | -46.0 |
| n66 20 MHz PCC | -93.3 | -93.3 | 4.68E-10 | 106 | 4.41E-12 | -113.6 | -99.55 |  |  |

-9.2 dBm aggressor level would be above maximum input level thus not applicable. It also is higher than ACS test case 2 aggressor level of -25 dBm. It is also counterintuitive that while ΔRIB,NC  was meant to be a relaxation, it actually seeming makes ACS more stringent.

For n66 the aggressor power makes sense.

We have not been able to verify this but it is our understanding that RAN5 adjusts the PCC level to the “more comfortable” level since intention is not to test PCC here. Unless RAN4 can find out what test parameters actually are, then maybe an clarificative LS to RAN5 is appropriate.

Observation 1: NC DL CA ACS test parameters are not clear from reading RAN4 and RAN5 specs

## 2.2	Discussion on requirements for single receiver chain

Embedding here with non-zero ΔRIB,NC and unequal channel BW the new discussion with limiting PSD difference maybe difficult. For example, if PSD of the aggressor would be set to no larger than the lower PSD of the wanted signals, it would be lower than the wanted signal PSD under test. This is hardly an appropriate test.

If we can assume that the receiver is capable for the full 100 MHz bandwidth and is therefore the same receiver what is used for 100 MHz channel BW in other bands, we can use 100 MHz requirements as starting point. In this case the Table 7.5-3 is same as the Ran5 shown above. For a 100 MHz wide wanted signal with power level REFENS+14 dB, the aggressor is 5 MHz with power of 39.5 dB above the 100 MHz REFSENS which results in to 25.5 dB power difference. For PSD difference this yields delta of 39.45 dB.

Observation 2: Any UE that is compliant with 100 MHz channel BW should be able to handle 39.5 dB PSD difference between wanted signal and aggressor.

Discussion could be had what is the 100 MHz channel filter role in ACS.

# Conclusion

We discussed reference requirements for NC DL CA fragment carriers study and made two observation.

Observation 1: NC DL CA ACS test parameters are not clear from reading RAN4 and RAN5 specs

Observation 2: Any UE that is compliant with 100 MHz channel BW should be able to handle 39.5 dB PSD difference between wanted signal and aggressor.

# Reference

[1]	R4-2417201, “WF on NR FR1 DL fragmented carriers study”,  Mediatek Inc., 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, Anhui, China, 14th – 18th October, 2024