3GPP TSG-RAN WG4 Meeting #113	R4-2418091

Orlando, USA, 18th -22nd November, 2024

Source:	Qualcomm Incorporated

Title:	Power boosting and MPR reduction

Agenda item:	7.1.1.2.1

Document for:	Approval

# 1.	Introduction

In RAN4#112-bis there was a new WF [1] to further study power domain enhancements for NR single carrier. In this paper we present additional views on this topic.

# 2. 	Discussion

In RAN4#112-bis there was a WF created [1] to further study the open items on power domain enhancements for NR single carrier. Some of the unresolved issues are noted below:

#### Issue 1-1-1: Approaches for scenarios 1-1 and 1-2 (see Reference for description of scenarios)

- WF

  - Strive for a unified solution which can accommodate both scenario 1 and scenario 2 under the framework of converting outer RB allocation to inner RB allocation.

    - It does not preclude separate solutions for each applicable scenario

    - No relaxation of ACLR/SEM/SE values

Issue 1-2-1: Clarification of multi-CC scenario

- WF

  - The approach of converting outer RB allocation to inner RB allocation for MPR reduction should consider multi-carrier spectrum scenario from NW perspective while the UE is configured for single CC operation.

Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation

- WF

  - The approach should consider the following aspects

    - unified solution could be applicable for both scenarios 1 and scenario 2

    - unified solution considering the spectrum allocation status of operators, e.g multi-CC case from NW perspective, to better leverage the feature of MPR reduction

    - Solution could benefit RedCap and non-RedCap with UE CBW less or even identical to BS CBW

    - Solution could include both symmetric and asymmetric CBW extension

    - Whether the extended UE CBW could exceed the BS CBW for different scenarios

    - Signalling overhead is considered for the proposed approach(s)

In the above mentioned WF it was proposed that companies attempt to find a unified solution which can accommodate both scenario1 and scenario2. In RAN4#112-bis when discussing scenario 1 companies agreed not to relax ACLR and SEM specifications. We suggest, as with scenario 2 where the UE BW is smaller than the BS BW that bandwidth expansion be used for scenario 1 as well.

Proposal 1: Use bandwidth expansion for both scenario 1 and scenario 2.

The UE bandwidth expansion should not exceed the BS bandwidth as any region outside the BS BW may not belong to the same operator and expansion of BW into those regions may not be allowed.

Proposal 2: The UE bandwidth expansion should not exceed the BS CBW.

Also, the UE bandwidth expansion should consider the existence of other users in the BS BW. In these multi-user scenarios where a BS is servicing multiple UEs the UE BW expansion should be restricted to prevent the UE bandwidth being extended into another UEs BW.

Proposal 3: UE bandwidth expansion should consider the existence of other users in the BS CBW and in these cases the BS should restrict the UE BW expansion as so to prevent interference with other users serviced by the same BS.

The following was written in the WF[1] and companies were asked to provide further inputs.

#### Issue 1-2-3: Where to use IBE in the larger BS CBW

- WF

  - IBE should be used between edges of UE CBW and extended UE CBW

    - FFS whether IBE could be used in the guard band of UE CBW

    - FFS the full RB allocation scenario

    - FFS the impact to testability

#### Issue 1-2-4: Boundary to apply ACLR and SEM

- Proposals

  - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

  - Proposal 2: ACLR, SEM and spurious emissions would be defined based on BS channel bandwidth.

- WF

  - FFS in next meeting

    - ITU regulation of 250% necessary bandwidth should be considered in further analysis

#### Issue 1-2-5: Boundary to apply SE

- Proposals

  - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW.

  - Proposal 2: SE is applied at BS channel bandwidth.

- WF

  - FFS in next meeting

    - ITU regulation of 250% necessary bandwidth should be considered in further analysis

#### Issue 1-2-6: Which CBW is utilized as the basis for the OOBE requirement and the applicable boundary

- Proposals

  - Proposal 1: The OOBE requirement and applicable boundary should be based on UE CBW.

  - Proposal 2: The OOBE requirement and applicable boundary is based on BS CBW.

- WF

  - FFS in next meeting

The UE bandwidth expansion should be based on BWs currently supported in the standard.

Proposal 4: The expanded UE BW should be a value that is currently supported in the standard.

In our opinion it makes sense to use IBE between the edges of the UE BW to the edges of the extended UE BW.

Proposal 5: IBE should be used between the edges of the UE BW to the edges of the extended UE BW

The original UE BW consists of guard bands on both sides. The RBs consisting of the guard bands are accessible to the BS for allocation to other UEs. When the UE bandwidth is extended these guard band regions will be replaced with RBs allocated to the UE. There is no harm in removing these guard band regions as these RBs are merely being reassigned from BS usage to UE usage. Guard bands will be present at the edges of the extended UE BW. The IBE between the edge of the UE BW and the edge of the extended UE BW should be an extension of the IBE for the UE signal as indicated in figure 1.

Observation 1: When UE bandwidth is extended the guard band in that region will be replaced with RBs allocated to the UE.

Observation 2: There is no harm in removing these guard band regions as these RBs are being reassigned from the BS usage to UE usage.

Figure 1: Extension of IBE mask

Proposal 6: The IBE between the edge of the UE BW and the edge of the extended UE BW should be an extension of the IBE for the signal within the UE BW.

The ACLR, SEM, SE and OOBE should be evaluated from the edge of the extended UE BW. The 250% of the original UE BW stated for the ITU specification which is calculated from the middle of original UE BW should apply from a frequency offset which is larger than the frequency offset specified for the 3GPP SE specifications which apply from a frequency offset of UE BW +5MHz as indicated in the figure below:

Figure 2: Spurious emissions limits

The size of UE BW extension, X on either side of the UE BW should be configured such that the frequency offset from where the ITU emissions start should be greater than the frequency offset from where 3GPP SE limits apply. This means that (2 UE BW) ≥ (X+UE BW + 5MHz). Therefore, the UE BW extension should be limited to X ≤ (UE BW - 5MHz). This equation reveals that UE BW extension is only possible for UE BW≥ 10MHz. Analysis shows that for UE BWs of 3MHz and 5MHz bandwidth extension is not possible if we want to the 3GPP SE frequency limits to also cover the ITU specifications. For UE BW<10MHz any frequency extension leads to the 3GPP SE frequency limit being greater than the ITU emissions limit which would mean that the 3GPP SE specifications do not cover the entire frequency range of the ITU specifications. Also, so as not to overly complicate this feature we think that the UE BW extension should be at least UE BW/2  to ensure all outer allocations become inner allocations on the side where the UE BW extension is placed [2].  The UE BW extension as agreed earlier can be placed on one or both sides of the UE BW [3].

Proposal 7: ACLR, SEM, SE and OOBE should be evaluated from the edge of the extended UE BW and the applicable frequency ranges for each specification should be calculated using the original UE BW.

Observation 3: Exceeding the UE BW extension beyond (UE BW – 5MHz) would cause the ITU frequency emissions limits to be lower than the 3GPP SE frequency limits.

Observation 4: To prevent the ITU frequency emissions limits being lower than the 3GPP SE frequency limits BW extension is not possible for 3MHz and 5MHz UE BWs.

Proposal 8:  To keep the UE BW extension scheme simple the UE BW extension should at least be:   UE BW extension ≥ UE BW/2

Proposal 9: The UE BW extension should be bounded as follows: UE BW/2 ≤ UE BW extension ≤ (UE BW – 5MHz)

# Conclusion

In this paper we further discuss our views on the study of power domain enhancement and make the following proposals:

Proposal 1: Use bandwidth expansion for both scenario 1 and scenario 2.

Proposal 2: The UE bandwidth expansion should not exceed the BS CBW.

Proposal 3: UE bandwidth expansion should consider the existence of other users in the BS CBW and in these cases the BS should restrict the UE BW expansion as so to prevent interference with other users serviced by the same BS.

Proposal 4: The expanded UE BW should be a value that is currently supported in the standard.

Proposal 5: IBE should be used between the edges of the UE BW to the edges of the extended UE BW

Observation 1: When UE bandwidth is extended the guard band in that region will be replaced with RBs allocated to the UE.

Observation 2: There is no harm in removing these guard band regions as these RBs are being reassigned from the BS usage to UE usage.

Proposal 6: The IBE between the edge of the UE BW and the edge of the extended UE BW should be an extension of the IBE for the signal within the UE BW.

Proposal 7: ACLR, SEM, SE and OOBE should be evaluated from the edge of the extended UE BW and the applicable frequency ranges for each specification should be calculated using the original UE BW.

Observation 3: Exceeding the UE BW extension beyond (UE BW – 5MHz) would cause the ITU frequency emissions limits to be lower than the 3GPP SE frequency limits.

Observation 4: To prevent the ITU frequency emissions limits being lower than the 3GPP SE frequency limits BW extension is not possible for 3MHz and 5MHz UE BWs.

Proposal 8:  To keep the UE BW extension scheme simple the UE BW extension should at least be:   UE BW extension ≥ UE BW/2

Proposal 9: The UE BW extension should be bounded as follows: UE BW/2 ≤ UE BW extension ≤ (UE BW – 5MHz)

# References

[1] R4-2417204, WF on power domain enhancements, RAN4#112bis, October 14th – October 18th, 2024

[2] R4-2411631, Power boosting and MPR reduction, RAN4#112, August 19th – August 23rd, 2024

[3] R4-2414441, WF on power domain enhancements, RAN4#112, August 19th – August 23rd, 2024