3GPP TSG-RAN WG4 Meeting #113                                                              R4-2418094

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:	Further discussion on testability and interoperability issues for beam management

Agenda item:			7.17.4.2

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two confirmed use cases (i.e., beam management and positioning accuracy enhancements) [2].

Particularly for the use case of beam management, it is required to provide specification support for DL TX beam prediction for both UE-sided and NW-sided model, and correspondingly necessary RAN4 core requirements for this use case and LCM procedure including performance monitoring [2]. Since AI-BM involves more complicated test procedure compared with Rel-15 FR2 beam management related test, the testability issue are highlighted in previous RAN4 discussion.

In RAN4#111, RAN4#112 and RAN4#112bis, for reported measurements and ground truth, test setup (including channel model) and datasets for training/testing, the following agreements are achieved:

Agreement from RAN4#111 Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded Issue 2-5: Test setup needs Agreement: - maximum number of set B Tx beams that test system should be able to emulate: [8-16] - maximum number of set A Tx beams that test system should be able to emulate: [64-128] - FFS on AoAs - UE rotation during the test: FFS - UE rotation/repositioning between different tests: Yes Issue 2-6: Datasets for training/testing Agreement: - Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training - FFS on proposals to augment training data to avoid overfitting of UE models to the test environment - RAN4 to strive to make the test conditions similar to field deployment conditions Agreement from RAN4#112 Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy. Issue 2-7: UE reporting for network side models Agreement: postpone discussion until other WGs define any reporting mechanisms Agreement from RAN4#112bis Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point.

Based on the outcome of the study objectives captured in TR 38.843 [1] and other RAN1 progress, we would like to discuss the remaining testability and interoperability issues for beam management in this contribution.

# 2. Discussion

## 2.1 UE RX beam knowledge

For AI-BM model inference, no matter using the information of Tx beam ID and/or Rx beam ID explicitly or implicitly, there are two kinds of beam prediction assumed in RAN1, i.e., DL Tx beam prediction with the measurements from (1) the best Rx beam or (2) Tx-Rx beam pair. Although from RAN1 perspective, both methods can be viewed as different AI/ML model implementation, it may have impact on testability discussion especially for determining the applicable channel model. Particularly, for the model inference, the following figure is provided [1] to illustrate the inference procedure:

Observation 1: For UE-sided model, DL Tx beam prediction, testability discussion could be different for AI/ML model input by using the measurements from (1) the best Rx beam or (2) Tx-Rx beam pair.

As analyzed in our accompanying paper for AI-BM core requirement [R4-2418094], RAN4 core requirement shall be general enough to cover both alternatives of UE-side model inputs:

- Alt-1: L1-measurements from the best RX beam taken as model input

- Alt-2: L1-measurements from Tx-Rx beam pair taken as model input

If the core requirement shall cover both Alt-1 and Alt-2, from AI-BM testing perspective, we shall consider the minimum requirement can be tested by considering L1-measurements from the best RX beam taken as model input.

Proposal 1: For UE-sided model inference testing, RAN4 shall study the testability by considering L1-measurements from the best RX beam taken as model input.

## 2.2 Test procedure for AI-BM (UE-side model)

Among two AI-BM use case (Case1 & Case2), we would like to take Case 1 as example to elaborate the test procedure for AI-BM conformance testing.

Proposal 2: The conformance testing procedure for AI-BM UE-side model (using AI-BM Case1 as a example), is provided as:

- Preparation Phase:

- Step-1: TE configures resource sets for both Set A and Set B beams in CSI-ReportConfig, and associated ID as side condition

- Model Inference Phase:

- Step-2: TE configures the test environment to transmit Set B beams

- Step-3: DUT (a.k.a, UE) predicts Set A beams based on the measurement from Set B beams, and reports the prediction results back to TE

- Ideal Measurement Phase:

- Step-4: TE adjusts the test environment to transmit Set A beams, ensuring a certain level of SNR for the ideal measurement of RSRP

- Step-5: DUT measures the Set A beams, and report measurement results to TE

- TE Evaluation Phase:

- Step-6: TE evaluates the success of the prediction by comparing the predicted results from Step 3 with the actual measurement results from Step 5.

## 2.3 Beam prediction KPI

In previous RAN4 discussion, For the metrics/KPI discussion, the following options are given in the Rel-18 study item and captured in TR [1]:

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

Here, RAN4 requirement shall be established by considering the expected behavior and performance from the typical and exemplary UE implementation. When it comes to beam prediction accuracy, it is clear that the network will primarily rely on the UE's top prediction by TCI switching to the Top-1 predicted beam by UE, rendering the second and subsequent strongest beams less relevant. To harness the strengths of both Option 2 and Option 3, we suggest a balanced approach that integrates aspects of Option 2 (Top-1/K) and Option 3:

Proposal 3: For metrics for UE-sided AI-BM requirements/tests, RAN4 adopt the combined Option 2 and Option 3 for compromise:

- Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB”

  - K = 2 and x = 3 as baseline

## 2.4 Test setup for UE-sided AI-BM testing

As discussed over previous RAN 4 meetings, the group agrees to further discuss and down-select the following options considering TE feasibility:

- Option 2: TDL

- Option 5: simplified CDL

  - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

Based on RAN4#112bis, the following agreement is provided for test setup requirements:

Agreement from RAN4#112bis - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A

Here we would like to firstly identify the key difference by using TDL and simplified CDL models from the perspective of testing target (based on the analysis in section 2.1):

- DUT RX beam performance and relevant procedure can only be verified with CDL models, in which the spatial differentiated AoAs can be emulated:

  - For AI/ML model with TX-RX beam pair measurement as model input, CDL model with the spatial differentiated AoAs is required.

  - For AI/ML models by TX beam measurement from the best RX beam as model input, if an ideal spherical coverage (uniform in all directions) is assumed, to test this AI/ML models, the spatial AoA characteristics could be not that important.

Figure 1: AI/ML models by using the measurements from the best Rx beam (left) and Tx-Rx beam pair (right)

From technical aspects, for DFF chamber, RAN4 has studied the method to use one probe to simulate multi-path fading condition, as provided in Section 8.2 in TR 38.810, in which two options are given. However, it should be noted that in the corresponding single probe channel modelling, the UE RX beamforming can be included in the channel model, while since there is only one AoA, it is impossible to test actual RX beam management procedure.

#### 8.2.1.1 Channel model Option 1 Channel model option 1 is based on the TDL methodology in the TR 38.901 [10]: - The multi-path propagation conditions model consists of several parts: - A power delay profile in the form of a "tapped delay-line" (TDL), characterized by a number of taps with certain power at fixed positions on a sampling grid. - The channel model parameters include the Delay spread scaling factor and the maximum Doppler frequency. The test system shall allow flexible control of the respective parameters. - Each tap is modeled based on the Jakes fading model. - Generation of TDL channel models and power delay profiles from CDL channel models by including spatial filters to capture Tx and Rx antenna patterns is not precluded and based on the procedure described in the TR 38.901 [10].

Observation 2: The necessity of introducing spatial differentiated AoAs in AI-BM testing could depends on different AI/ML model inputs:

- For AI/ML model with TX-RX beam pair measurement as model input, CDL model with the spatial differentiated AoAs is required.

- For AI/ML models by TX beam measurement from the best RX beam as model input, to test this AI/ML models, the spatial AoA characteristics could be not that important.

Another aspect is how many TRPs to be simulated for AI-BM. Based on our understanding, the relevant AI-BM prediction are primarily used for single TRP case. At least for Rel-19 AI-BM, RAN4 discussion shall focus only on single TRP case.

Proposal 4: At least for Rel-19 AI-BM, RAN4 discussion shall focus on single TRP case, which is considered as baseline assumption for testability study.

Based on the above test procedure, it has been recognized that relying solely on Set B of beams for the test setup might not suffice. Based on previous RAN4 agreement, the ground truth shall be “the ideal measurement of RSRP on the predicted Tx beam”, which in the conformance test is “the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam”. Our interpretation is that the ideal RSRP measurement on the predicted transmission beam should be executed by producing Set A of beams.

Agreement from RAN4#111 Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded Agreement from RAN4#112bis Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

Proposal 5: AI-BM OTA test setup shall generate the not only Set B of beams, but also Set A of beams for the ideal measurement of RSRP on the predicted Tx beam.

The above proposal could be essential in the testability discussion: In the vendor analysis outlined in document [R4-2411254], it's intriguing to note that only the four most powerful TX beams are emulated using an 8-probe configuration in an enhanced MPAC OTA chamber. However, this approach might fall short if the emulation of Set A beams (with the number of Set A beams much larger than 4) is also necessary to achieve the optimal measurement of RSRP on the predicted transmission beam.

For the selection of OTA chamber, we see the difference for the required probes for MPAC OTA chamber. For instance, the analysis provided in [R4-2411254] requires 8 probes (2 additional one compared with existing 3D MPAC chamber) to simulate 4 strongest TX beams, with acceptable PSP (PAS similarity percentage) for InO and UMi scenarios. On the other hand, in [R4-2411587], upscaling the 3D MPAC chamber to emulate CDL-C UMi requires a test system with >100 probes. The difference may come from how many TX beams and how many clusters to be emulated with accepted accuracy.

From our understanding, the feasibility of enhanced 3D MPAC chamber shall consider the following factors:

- FFS how many beams out of Set A (e.g., 128 beams containing Set B beams) are relevant for AI-BM inference testing;

- All Tx beams from a single TRP;

- Acceptable PAS similarity percentage based on a certain configuration of probes

Proposal 6: The feasibility of enhanced 3D MPAC chamber shall consider the following factors:

- FFS how many beams out of Set A (e.g., 128 beams containing Set B beams) are relevant for AI-BM inference testing;

- All Tx beams from a single TRP;

- Acceptable PAS similarity percentage based on a certain configuration of probes

# 3. Conclusion

In this contribution, we provided our viewpoints on testability and interoperability issues for beam management in this contribution, accordingly the following observations and proposals are obtained:

Observation 1: For UE-sided model, DL Tx beam prediction, testability discussion could be different for AI/ML model input by using the measurements from (1) the best Rx beam or (2) Tx-Rx beam pair.

Proposal 1: For UE-sided model inference testing, RAN4 shall study the testability by considering L1-measurements from the best RX beam taken as model input.

Proposal 2: The conformance testing procedure for AI-BM UE-side model (using AI-BM Case1 as a example), is provided as:

- Preparation Phase:

- Step-1: TE configures resource sets for both Set A and Set B beams in CSI-ReportConfig, and associated ID as side condition

- Model Inference Phase:

- Step-2: TE configures the test environment to transmit Set B beams

- Step-3: DUT (a.k.a, UE) predicts Set A beams based on the measurement from Set B beams, and reports the prediction results back to TE

- Ideal Measurement Phase:

- Step-4: TE adjusts the test environment to transmit Set A beams, ensuring a certain level of SNR for the ideal measurement of RSRP

- Step-5: DUT measures the Set A beams, and report measurement results to TE

- TE Evaluation Phase:

- Step-6: TE evaluates the success of the prediction by comparing the predicted results from Step 3 with the actual measurement results from Step 5.

Proposal 3: For metrics for UE-sided AI-BM requirements/tests, RAN4 adopt the combined Option 2 and Option 3 for compromise:

- Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB”

  - K = 2 and x = 3 as baseline

Observation 2: The necessity of introducing spatial differentiated AoAs in AI-BM testing could depends on different AI/ML model inputs:

- For AI/ML model with TX-RX beam pair measurement as model input, CDL model with the spatial differentiated AoAs is required.

- For AI/ML models by TX beam measurement from the best RX beam as model input, to test this AI/ML models, the spatial AoA characteristics could be not that important.

Proposal 4: At least for Rel-19 AI-BM, RAN4 discussion shall focus on single TRP case, which is considered as baseline assumption for testability study.

Proposal 5: AI-BM OTA test setup shall generate the not only Set B of beams, but also Set A of beams for the ideal measurement of RSRP on the predicted Tx beam.

Proposal 6: The feasibility of enhanced 3D MPAC chamber shall consider the following factors:

- FFS how many beams out of Set A (e.g., 128 beams containing Set B beams) are relevant for AI-BM inference testing;

- All Tx beams from a single TRP;

- Acceptable PAS similarity percentage based on a certain configuration of probes

# 4. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402389, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] 3GPP TR 38.810, v16.7.0, “NR; Study on test methods”.

[6] 3GPP TR 38.827, v16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[7] R4-2405652, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110bis, April 2024.

[8] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[9] R4-2409761, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#111, May 2024.

[10] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[11] R4-2411635, “Further discussion on testability and interoperability issues for beam management”, Samsung, RAN#112, Aug. 2024.

[12] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.

[13] R4-2416493, “Further discussion on testability and interoperability issues for beam management”, RAN#112bis, Oct. 2024.

[14] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.