3GPP TSG-RAN4 Meeting #113	R4-2418102

# Orlando, US, 18th – 22nd November, 2024

Title:	Preliminary A-MPR values for NS_36 and NS_26

Source:	Qualcomm Incorporated

Agenda item:	6.16.1

Document for:	Information

# 1.	Introduction

This contribution provides preliminary NS_26 and NS_36 AMPR specification for the n68 band based on simulations and lab measurements. The suggested A-MPR specifications are presented as information to RAN4. The simulations and measurements assume the same filter assumptions as previously utilized in the Rel-17 band 68 A-MPR specifications [1].

# 2. 	Discussion

## 2.1	Simulation and measurement filter assumptions

In our simulations and measurements, we utilize the filter assumption stated in [1]. The filter assumptions are based on 15MHz passband filter under extreme conditions, as highlighted in yellow in Table 1 below.

Table 1: Filter characteristics

| Frequency (MHz) | Vendor B SAW 15MHz passband |  |
| --- | --- | --- |
|  | Relative rejection (dB) |  |
| MHz | Extreme | Normal |
| 693.5 | 0.0 | 10.0 |
| 692.5 | 4.0 | 16.0 |
| 691.5 | 10.0 | 20.0 |
| 690.5 | 16.0 | 26.0 |
| 689.5 | 20.0 | 30.0 |
| 688.5 | 26.0 | 30.0 |
| 687.5 | 30.0 | 30.0 |
| 686.5 | 30.0 | 30.0 |

## 2.2 A-MPR results and proposed A-MPR

Following, we analyze A-MPR for 5, 10 and 15 MHz channel bandwidths located at the lowest frequencies of n68 band to fulfil -25dBm/8MHz (NS_26) and -42dBm/8MHz (NS_36) below 694MHz, considering extreme filter assumptions given in section 2.1. We also use the following impairment levels in our simulations and measurements:

- Carrier feedthrough = -28dBc

- Image = -28dBc

- CIM3 = -60dBc

### 2.2.1 Proposed regions and corresponding A-MPR specification for NS_26

In Figure 1, we show the proposed A-MPR regions for 5, 10 and 15 MHz channel bandwidths for NS_26 requirement. The regions were calculated based on our simulations and lab measurements.

Figure 1. NS_26 A-MPR regions for 5, 10, and 15 MHz channel bandwidths

Table 2 shows the corresponding NS_26 A-MPR proposal table which is derived based on the regions in Figure 1.

Table 2: Proposed A-MPR for NS_26

| Channel Bandwidth (MHz) | Waveform | Modulation | Region A11,2,3 | Region A24 |
| --- | --- | --- | --- | --- |
| 5 | DFT-s-OFDM | QPSK | 0.0 | 1.5 |
|  |  | 16QAM | 1.0 | 2.0 |
|  |  | 64QAM | 4.0 | 4.0 |
|  |  | 256QAM | 4.5 | 4.5 |
|  | CP-OFDM | QPSK | 2.0 | 3.5 |
|  |  | 16QAM | 2.0 | 3.5 |
|  |  | 64QAM | 3.5 | 3.5 |
|  |  | 256QAM | 6.5 | 6.5 |
| 10 | DFT-s-OFDM | QPSK | 0.0 | 2.5 |
|  |  | 16QAM | 1.5 | 3.0 |
|  |  | 64QAM | 2.5 | 3.5 |
|  |  | 256QAM | 4.5 | 4.5 |
|  | CP-OFDM | QPSK | 2.0 | 4.5 |
|  |  | 16QAM | 3.0 | 4.5 |
|  |  | 64QAM | 4.0 | 5.0 |
|  |  | 256QAM | 6.5 | 6.5 |
| 15 | DFT-s-OFDM | QPSK | 0.0 | 2.5 |
|  |  | 16QAM | 1.5 | 3.0 |
|  |  | 64QAM | 2.5 | 3.5 |
|  |  | 256QAM | 4.5 | 4.5 |
|  | CP-OFDM | QPSK | 2.0 | 4.5 |
|  |  | 16QAM | 3.5 | 4.5 |
|  |  | 64QAM | 4.0 | 5.0 |
|  |  | 256QAM | 6.5 | 6.5 |
| NOTE 1: Region A1 definition for channel bandwidth 5MHz: RBstart_low < RBstart < RBstart_high and LCRB < (ceil(NRB/2)) where RBstart_low = max(1,floor(LCRB/2)) and RBstart_high = NRB - RBstart_low - LCRB NOTE 2: Region A1 definition for channel bandwidth 10MHz: 1.8e6/12/SCS ≤ RBstart ≤ 8.82e6/12/SCS and LCRB ≤ -(0.5128)*RBstart + (4.7031e6/12/SCS) NOTE 3: Region A1 definition for channel bandwidth 15MHz: 3.6e6/12/SCS ≤ RBstart ≤ 13.68e6/12/SCS and LCRB ≤ -(0.6071)*RBstart + (8.4857e6/12/SCS) NOTE 4: Region A2 covers RB allocations not included in Region A1. |  |  |  |  |

Observation 1: NS_26 A-MPR values are captured in Table 2 for various modulations and waveform types.

### 2.2.2 Proposed regions and corresponding A-MPR specification for NS_36

In Figure 2, we show the proposed A-MPR regions for 5, 10 and 15 MHz channel bandwidths for NS_36 requirement for various modulations and waveform types. The regions were calculated based on our simulations and lab measurements, and their corresponding required A-MPR is specified in the legend for ease of reference.

Figure 2. NS_36 A-MPR regions and corresponding values for 5, 10, and 15 MHz channel bandwidths

Observation 2: NS_36 A-MPR values are captured in Figure 2 for various modulations and waveform types.

# Conclusion

A-MPR specification proposals for n68 band NS_26 and NS_36 were presented as information to RAN4, based on lab measurements and simulations. The following observations were made:

Observation 1: NS_26 A-MPR values are captured in Table 2 for various modulations and waveform types.

Observation 2: NS_36 A-MPR values are captured in Figure 2 for various modulations and waveform types.

# Reference

[1]	R4-2417080, “WF on A-MPR for NR band n68”, Ericsson, RAN4#112bis