3GPP TSG-RAN WG4 Meeting # 113	R4-2418103

# Orlando, USA, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for Fast SCell activation for UE supporting R18 EMR

Agenda item:	7.15.3

Document for:	Discussion

Introduction

In this document, we show our views on the remain issues on RRM requirements for fast SCell activation for UE supporting R18 EMR.

Discussion

  - Sub-topic 1-1 General

- Issue 1-1-1: Applicability of fast SCell activation delay requirements Proposals: - Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK) - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured. - Option 2: (OPPO, Nokia, Samsung, Ericsson) - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR. - Option 2a: (OPPO) - Consider the following two approaches to support the case: - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration. - Option 2b: (Samsung) - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured

During the last RAN4#112bis meeting, RAN4 agreed the known condition as below:

Agreement: - RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception. - For FR1, - [Y] equals to [5s] for direct SCell activation - FFS: Y for other SCell activation case. - For FR2, - Reuse the existing value of Y in direct SCell activation - FFS: Y for other SCell activation case.

During the discussion of last RAN4#112bis meeting, the original wording updated from “valid eEMR reporting” to “Rel18 EMR” and “valid measurement report” to “Rel18 EMR”. From both of them the “valid” term was removed. So, we think that measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is not configured case also can be supported.

Proposal 1: RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR.

Sub-topic 1-2 SCell activation delay requirements

- Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting For continued discussion in next meeting: - For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured: - For FR1: - use existing SSB based known SCell activation delay requirement with [2] SSB samples. - For FR2: - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X] - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW) - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion. - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1 - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured) - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT) - K = 0 or 1 if when the “validityduration” is smaller than X - K = 1 or 2 otherwise - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung) - K = 0 or 1 if when the “validityduration” is applied with smaller than X - K = 1 or 2 otherwise - FFS how to test

From issue 1-1-1, if we support the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured, the R18 EMR supporting UE can report R18 EMR which measured at long time ago. In this case the time/frequency synchronization and AGC operation can be necessary. And even when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are configured, according to the validity duration the measurement time can be long time ago. So, we support option 3 or option 4.

Proposal 2: We support option 3 and 4. If when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are configured, according to the validity duration from 0 to 2 SSB samples can be necessary. And if RAN4 support measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured, additional 2 more SSB sample to the SCell activation delay can be considered.

Conclusion

In this contribution, we provide our views on fast SCell activation for UE supporting R18 EMR item.

Proposal 1: RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR.

Proposal 2: We support option 3 and 4. If when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are configured, according to the validity duration from 0 to 2 SSB samples can be necessary. And if RAN4 support measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured, additional 2 more SSB sample to the SCell activation delay can be considered.

Reference

[1] R4-2416854, “WF on RRM requirements for NR_RRM_Ph5_Part2”, CATT.