3GPP TSG-RAN WG4 Meeting # 113	R4-2418105

# Orlando, USA, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Discussion on RRM impacts of UE-to-UE CLI handling

Agenda item:	7.20.3.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on the remain open issues on RRM impacts of UE-to-UE CLI handle described in WF [1].

Discussion

Issue 1-1-2: Baseline for defining requirements - Proposal 1 (LGE, CTC, MTK): - Both R16 CLI measurement requirements and L1-RSRP measurement requirements can be used as baseline to define requirements for L1 based UE-to-UE CLI measurement. - Proposal 2 (Ericsson) - For L1-SRS-RSRP: measurement requirements shall base on simulation results - For L1-CLI-RSSI: Rel-16 CLI-RSSI requirement framework can be used as a baseline. Further updates can be made when RAN1 finalized the measurement RS configuration and method. Recommended WF - Close the issue, and RAN4 to focus on the exact requirement development rather than the baseline.

For L1 based UE-to-UE CLI measurement, both R16 CLI measurement requirements and L1-RSRP measurement requirements can be baseline. Because there is no big difference between them except the frequency domain range. The L1 based UE-to-UE CLI measurement for SBFD should consider only DL sub-band for L1-CLI-RSSI and UL sub-band for L1-SRS-RSRP respectively rather than DL active BWP which can depend on RAN1’s agreement.

Proposal 1: For L1 based UE-to-UE CLI measurement, both R16 CLI measurement requirements and L1-RSRP measurement requirements can be baseline.

Issue 1-1-3: Measurement methods - Proposal 1 (vivo, ZTE, HW, MTK): - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions. - Proposal 2 (LGE): - The UE to support L1 based UE-to-UE CLI measurement has to capable to measure the CLI measurement with sub-band unit rather than DL-active BWP unit. - Proposal 3 (Nokia): - RAN4 to define the CLI measurement requirements in SBFD as the following: - SRS-RSRP measurement requirements for Method#2. - CLI-RSSI measurement requirements for Method#1 and Method#3. - Proposal 4 (Samsung, CTC, QC): - RAN4 shall specify L1 CLI measurement requirements at least for below methods - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. Recommended WF - RAN4 to wait for RAN1 conclusion on the supported methods of L1 based UE-to-UE CLI measurement. - Meanwhile, RAN4 to discuss the impacts of different measurement methods on the requirements.

RAN1 agreed as below:

Agreement (from RAN1#118) For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband

We think RAN4 need to specify L1 CLI measurement requirements for the agreed methods first. And if additional methods are agreed from RAN1, RAN4 can consider the additional methods.

Proposal 2: RAN4 shall specify L1 CLI measurement requirements at least for below methods (Support P4)

  - Method#1: UE measures RSSI within DL subband

  - Method#2: UE measures RSRP of aggressor UE within UL subband

- Additional methods can be considered based on further RAN1 agreements.

Issue 1-1-4: Rx beam - Proposal 1 (CATT, LGE, E///): - The Rx beam assumption for R16 CLI measurement in FR2 can be reused, and it can be revisited if RAN1 has a new assumption. - Proposal 2 (Nokia, MTK, QC): - RAN4 to discuss the CLI measurement requirement considering the potential different Rx beams based on RAN1 conclusion. - Proposal 3 (Samsung): - For FR2 L1 CLI measurement, UE shall follow the configured ‘typeD’ QCI assumption specified for the CLI measurement. - Proposal 4 (HW): - RAN4 to confirm that no Rx beam sweeping is assumed in measurement period requirements with Rx beam configuration/determination for L1 CLI measurement. Recommended WF - RAN4 to wait for RAN1 conclusion on the Rx beam for L1 based UE-to-UE CLI measurement. - Meanwhile, RAN4 to discuss the impacts of Rx beam configuration/determination on the requirements.

Regarding the Rx beam configuration/determination for L1 based UE-to-UE CLI measurement, RAN1 agreed as below:

Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList: - In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic - If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource: - If unifiedTCI-StateType is not configured - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET” - If unifiedTCI-StateType is configured - “Indicated” DL only/joint TCI state as specified in TS 38.214 - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources

So, RAN4 should capture the RAN1 agreements for Rx beam requirements. And based on the further discussion of RAN1, RAN4 can revisit this issue.

Proposal 3: RAN4 should capture the RAN1 agreements for Rx beam requirements. And based on the further discussion of RAN1, RAN4 can revisit this issue.

Issue 1-1-5: Measurement resources - Proposal 1 (CATT): - Configuration with two discontinuous DL subbands should be taken into account when simulating and defining measurement accuracy requirements for CLI RSSI measurements - Proposal 2 (LGE, HW, MTK, Ericsson): - RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements. - Proposal 3 (Nokia): - RAN4 should define new requirements for the sub-band CLI-RSSI reporting if this feature is supported by RAN1. - Proposal 4 (Samsung): - RAN4 focuses on wideband L1-CLI-RSRI and L1-SRS-RSRP for L1 CLI measurement. Recommended WF - RAN4 to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

During the last RAN1#118bis meeting some CLI measurement resource related issues are agreed as below:

Agreement For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter. - FFS: Whether to reuse the legacy SRS resource set - FFS: Whether available slot offset list is also reused For aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter.

According to the agreement, RAN1 is still discussing about measurement resource and has not reached the conclusion yet. So, RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether there is an impact on the requirements.

Proposal 4: RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

Issue 1-1-7: Measurement period Proposals for L1-SRS-RSRP - Proposal 1 (vivo): - Requirements based on 1 sample measurement assumption should be used. - Proposal 1a (QC, Ericson): - RAN4 to define the measurement delay for aperiodic reporting of L1-SRS-RSRP at least based on a single sample. - Proposal 2 (CATT): - Re-use the framework of R16 CLI measurement period except for the sample number. - Proposal 3 (Nokia): - Follow L1-RSRP measurement approach, and both 1 and 3 samples are considered based on network configuration - Proposal 3a (Samsung): - RAN4 can evaluate L1 SRS-RSRP measurement performance considering both single shot and multiple shots measurements if different side condition identified Proposals for L1-CLI-RSSI - Proposal 1 (vivo, Nokia, HW, QC): - Requirements based on 1 sample measurement assumption should be used - Proposal 2 (E///): - Rel-16 CLI-RSSI requirement framework can be used as a baseline. Further updates can be made when RAN1 finalized the measurement RS configuration and method.

For L1-SRS-RSRP and L1-CLI-RSSI, we expect fast measurement. And accuracy perspective, we think that different from R16 CLI measurement, the L1-SRS-RSRP measurement of UL sub-band does not have the DL signal as interference which mean the channel quality is better than R16 CLI measurement case. So, we think the L1-SRS-RSRP requirements can be based on 1 sample measurement assumption. L1-CLI-RSSI also, the measurement period requirements can be defined based on 1 sample. Regarding measurement period, the final decision should be based on simulation results.

Proposal 5: For L1-SRS-RSRP and L1-CLI-RSSI, measurement period requirements are defined based on 1 sample. The final decision should be based on simulation results.

Issue 1-1-8: Measurement reporting - Proposal 1 (vivo): - Define requirements for periodic, semi-persistent reporting. - Proposal 2 (Samsung, CTC, HW, MTK): - Focus on aperiodic CSI reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1 - Proposal 3 (CATT, Nokia, HW, E///): - L1-RSRP measurement reporting requirement for aperiodic reporting can be reused, and wording can be adapted when reporting scheme design is stable. - Proposal 4 (MTK): - FFS whether L1-RSRP measurement reporting requirement can be re-used pending on RAN1 agreement.

RAN1 decided to support aperiodic CSI reporting mode for Rel19 CLI measurement. But regarding other reporting modes, RAN1 doesn’t reach any conclusion yet. So, we would like to focus on aperiodic CSI reporting mode for Rel-19 CLI measurement, and for other reporting modes hold on the discussion pending on further progress in RAN1.

Proposal 6: Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1.

Issue 1-1-9: Measurement accuracy Proposals for L1-SRS-RSRP - Proposal 1 (vivo): - The accuracy requirement in Rel-16 can be used as baseline. - Proposal 2 (ZTE, CTC): - RAN4 not to reuse legacy UE-to-UE CLI accuracy requirements directly. Determining the accuracy requirements based on the evaluation results. - Proposal 3 (HW, MTK): - RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the agreed side condition and measurement period.

For L1-SRS-RSRP, the side condition and measurement sample can be different from Rel16 CLI measurement case. So, RAN4 need to define the measurement accuracy requirements for L1-SRS-RSRP based on the simulation results at agreed side condition and measurement sample.

Proposal 7: RAN4 need to define the measurement accuracy requirements for L1-SRS-RSRP based on the simulation results at agreed side condition and measurement sample.

Issue 1-1-13: Requirement applicability - Proposal 1 (LGE): - L1 based UE-to-UE CLI measurement for SBFD are only applicable for RRC_CONNECTED intra-frequency: - SRS-RSRP measurement resource is fully confined within BW of UL sub-band - CLI-RSSI measurement resource is configured within DL sub-band - Proposal 2 (QC): - The L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

According to the agreed measurement methods from RAN1, the additional applicability for L1 based UE-to-UE CLI measurement is necessary.

Proposal 8: The additional applicability for L1 based UE-to-UE CLI measurement need to be considered as follow:

L1 based UE-to-UE CLI measurement for SBFD are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of UL sub-band - when CLI-RSSI measurement resource is configured within DL sub-band

Summary

Proposal 1: For L1 based UE-to-UE CLI measurement, both R16 CLI measurement requirements and L1-RSRP measurement requirements can be baseline.

Proposal 2: RAN4 shall specify L1 CLI measurement requirements at least for below methods (Support P4)

  - Method#1: UE measures RSSI within DL subband

  - Method#2: UE measures RSRP of aggressor UE within UL subband

- Additional methods can be considered based on further RAN1 agreements.

Proposal 3: RAN4 should capture the RAN1 agreements for Rx beam requirements. And based on the further discussion of RAN1, RAN4 can revisit this issue.

Proposal 4: RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

Proposal 5: For L1-SRS-RSRP and L1-CLI-RSSI, measurement period requirements are defined based on 1 sample. The final decision should be based on simulation results.

Proposal 6: Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1.

Proposal 7: RAN4 need to define the measurement accuracy requirements for L1-SRS-RSRP based on the simulation results at agreed side condition and measurement sample.

Proposal 8: The additional applicability for L1 based UE-to-UE CLI measurement need to be considered as follow:

L1 based UE-to-UE CLI measurement for SBFD are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of UL sub-band - when CLI-RSSI measurement resource is configured within DL sub-band

Reference

[1] R4-2416858, “WF on RRM requirements for NR_duplex_evo”, Huawei