3GPP TSG-RAN4 Meeting #113	R4-2418122

# Orlando, US, 18th – 22nd November, 2024

Title:	TP for TR: Testability aspects of A-IoT device

Source:	Qualcomm Incorporated

Agenda item:	7.21.3.2

Release:	Rel-19

SI: 	FS_Ambient_IoT_solutions

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

In RAN4#112bis an agreement for testability aspect was agreed as below. In this paper we provide a TP for testability aspects.

Issue 3-5: testability - Take OTA test as the baseline for at least Devices 1 and 2a - FFS on the side conditions in the OTA test setup including CW signal and RF energy source, etc.

# 2. 	Text Proposal to TR 38.769

<Start of Text Proposal>

#### 6.7.4.4 Testability

##### 6.7.4.4.1 Testability of device

Figure 6.7.4.4.1-1 describes TX modulator, and it gives some indication how backscattering works. Apart from device 2b, which is assumed to have its own reference clock, device 1 and device 2a architecture is such that they modulate the impedance seem by the antenna. The impedance is either matched to antenna or then it is such that makes the antenna reflect the incoming wave back. The pace of the impedance changes is used to create the modulated BL waveform. It can be assumed that the connection point between the impedance modulator and antenna is not 50 ohm reference but can be assume to be implementation specific.

Figure 6.7.4.4.1-1: Example TX architectures for Device 1/2a/2b

With such highly integrated architectures for device 1/2a, conducted antenna connectors are assumed not be available at device 1 and device 2a. Therefore, OTA testing is considered as the baseline testing methodology for device 1 and device 2a UE RF verification. Far-filed measurement system in an anechoic chamber could be considered as the baseline setup.

##### 6.7.4.4.2 Testability of device with Energy Harvesting procedure

Device goes from OFF state to ON state when it detects energy and has been able to accumulate sufficient amount of energy for the parts to function. This Energy Harvesting Sensitivity (EHS) is the value when device starts to harvest energy. Device behaviour principle with non-constant energy availability is shown in Figure 1.

Figure 6.7.4.4.2-1. Energy harvesting principle

Energizing signal is a trigger for device to turn on. This is specifically for device type 1 and 2a. After device turns on, it needs to monitor the WUS and then after successfully receiving signal to transmit, it needs to transmit back what was requested. Device may also decide to sleep or continue monitoring.

Figure 6.7.4.4.2-2. Energy harvesting frame structure

In simple form, this translates to some ON-OFF pattern of the device. Testing of RF requirements should be done when device is known to be turned on, in phase X, Y and Z in Figure 2.

Energy will be captured at first hand by the device antenna with the antenna in the device and for study purposes, RAN4 assumes 0 dBi antenna.

When incoming RF signal level exceeds EHS, device starts harvesting energy. Energy harvesting in the device is not perfect but happens with some efficiency < 100% and the efficiency is proportional to the incoming signal level. The accumulated energy can be calculated as a function of incoming signal strength, time and efficiency.

Figure 6.7.4.1.2-3. Accumulated energy

Simplifying the frame in Figure 6.7.4.4.2-2 so that the ON time is 2 msec and considering that device power consumption is always 1uW when ON the energy consumed is 5 nWs. Time to harvest 5 nWs is 21 msec with -29 dBm received signal strength and 18% harvester efficiency.

In testing phase, tester must provide sufficient RF waveform electrical field density to the device to ensure it is turned on and time gate the RF testing to the ON periods of the A-IoT device. RAN4 needs to define a list of testable requirements in the WI phase and also the time domain characteristics for the tests. For example, from analysis above, device can be assumed to turn ON after 30 msec starting from the beginning of availability of a -30 dBm energizing signal and then device can be assumed to be on for some period of time.

<End of Text Proposal>