3GPP TSG-RAN4 Meeting #113	R4-2418161

# Orlando, US, 18th – 22nd November, 2024

Title:	Aligning notes with RAN1 language and improving some parts

Source:	Qualcomm Incorporated

Agenda item:	6.10.2

Release:	Rel-19

WI: 	DL_intrpt_combos_TxSW_R19-Core

Lead WG: 	RAN4

Document for:	Approval

# 1.	Introduction

WF was approved in RAN4#112bis [1] on notes clarification for the Dl interruption. In this paper we discuss notes and their language in more detail and propose changes for better readability and alignment with language used in other WGs.

# 2. 	Discussion

WF [1] has two tables, one that amends a note in three band table and one that adds it to four bands table. Below is the agreement for four band which is same as the note in three band tables with the exception of adding the 4.

There are three aspects to consider in the note that may benefit from changing.

First one is the use of phrase “TX switching is scheduled”. The behaviour is such that the UE receives UL grants and when these grants indicate that UE is scheduled to transmit on two different bands at two different times, UE then has to switch the baseband TX chain to be connected to the RF chain for the band with the latter transmission. Therefore, UE has to determine the switching based on scheduled UL grants but switching itself it not scheduled. For this purpose RAN1 uses “triggering” as shown in the snippet from 38.214:

It would be preferrable to align the use of languages between specifications and use also “trigger” in RAN4.

Proposal 1: Align Ran4 specification language with RAN1 and use “switching is triggered” instead of “switching is scheduled”.

Second issue is the reference of “switching among 2, 3 or 4 bands. This leaves it a bit ambiguous what are those 2,3 and 4 bands. We would prefer to detail that there are bands that are configured for UL in this case, since this is how feature works: UE is configured for 2, 3 or 4 bands and then switching is triggered between two bands.

Proposal 2: Add wording to say that the 2, 3 or 4 bands are configured for UL.

With these proposals the note would look like this:

NOTE 2:	Applicable when dynamic Tx switching is triggered among the configured 2, 3 or 4 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. The CA_nX-nY/nX-nY in the requirement of “No for CA_nX-nY/No for nX-nY” refers to the two UL Bands nX and nY that are involved in dynamic Tx switching.

The third issue is the use of the word “applicable” which is also used in earlier releases. The adjective “applicable” means by oxford dictionary as “relevant or appropriate.” And according to Merriam-Webster: “capable of or suitable for being applied” . It is pour understanding that the requirement is that for the band pairs that marked as “No” UE is not allowed to indicate that it needs DL interruption. If the RAN4 specification no says that “No” is suitable for being applied, it implies it is a merely a suggestion. And it makes it even more confusing that the note 2 is not attached to the “No” but to the table column heading stating ‘DL Interruption allowed”. This way Ran4 specification written out says : “DL Interruption is suitable for being applied” and in some cases there is empty cell and in some cases it writes “No”. Much easier would be to say what is meant, that “For band pairs(/configurations) marked as “No” UE is not allowed to indicate DL interruption”

Proposal 3: Do not use word “Applicable” in the note but state it clearly what is the requirement by saying “No” means UE is not allowed to indicate DL interruption.

With this change, the note would look like this:

NOTE 2:	When “No” is indicated for a band pair, UE is not allowed to indicate DL interuptions. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. The CA_nX-nY/nX-nY in the requirement of “No for CA_nX-nY/No for nX-nY” refers to the two UL Bands nX and nY that are involved in dynamic Tx switching.

# Conclusion

We discussed the note language and made three proposals:

Proposal 1: Align Ran4 specification language with RAN1 and use “switching is triggered” instead of “switching is scheduled”.

Proposal 2: Add wording to say that the 2, 3 or 4 bands are configured for UL.

Proposal 3: Do not use word “Applicable” in the note but state it clearly what is the requirement by saying “No” means UE is not allowed to indicate DL interruption.

Depending on which proposals are agreeable, the note languages can be either:

NOTE 2:	Applicable when dynamic Tx switching is triggered among the configured 2, 3 or 4 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. The CA_nX-nY/nX-nY in the requirement of “No for CA_nX-nY/No for nX-nY” refers to the two UL Bands nX and nY that are involved in dynamic Tx switching.

Or

NOTE 2:	When “No” is indicated for a band pair, UE is not allowed to indicate DL interuptions. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. The CA_nX-nY/nX-nY in the requirement of “No for CA_nX-nY/No for nX-nY” refers to the two UL Bands nX and nY that are involved in dynamic Tx switching.

# Reference

[1]	R4-2417057, “WF on notes for downlink interruption clarification”, China Telecom, 3GPP TSG-RAN4 Meeting #112bis, Hefei, Anhui, China, 14th – 18th October, 2024