3GPP TSG-RAN4 Meeting #113	R4-2418168

Orlando, US, Nov 18 – 22, 2024

Agenda item:			7.6.2

Source:	vivo

Title:	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

- Introduction

In RAN4#112bis meeting, the WF[1] has been agreed as follows, and this contribution will discuss about the remaining open issues.

# Main session online agreements #### Sub-topic 1-2: Study scope clarification Agreement: - Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz as illustrated below: - Any update of scope needs be decided by RAN #### Issue 3-1: Discussion on assumptions on test configurations for evaluation Agreement: (a) RAN4 agree on the general assumptions that are same for legacy NR RF performance evaluation: o UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB o Reuse the Rx chain image rejection, which is 25dB, as the starting point. Agreement: - The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC, REFSENS, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos Agreement: - Priority to one uplink carrier Agreement: - Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc - The example bands should be in the scope of existing intra-band non-contiguous CA operating bands - No relaxation on any Tx RF requirements should be assumed in this study item - Both PC3 and PC2 should be considered - Prioritize PC3 - The minimum Wgap for ACS, narrow band blocking and in-band blocking verification should meet: - Wgap ≥ 2∙¦FInterferer (offset),j¦ – BWChannel(j) - It is the condition to apply the requirements - Both the symmetric CC scenario and the asymmetric CC scenario should be considered in the FC SI, and companies are encouraged, but not limited, to study the CC scenarios shared by the operators - [Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements] - RAN4 shall not re-discuss the power imbalance assumption of the wanted carriers further in relation to the SI on fragmented carriers #### Issue 3-2: Example band consideration for evaluation Agreement: - RAN4 doesn’t consider 3MHz channel bandwidth for fragmented carriers - The example band(s) should already have NC DLCA specified - Prioritize the frequency bands of ≤2700MHz - Remove n39 and include n26 in the example bands: Consider n2/n25, n3, n7, n66, n41, n39, n26 as the example band # Ad hoc meeting agreements #### Merge issue 2-0/2-1 discussion: Agreement: The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823) #### Issue 2-2: Study on power spectral density difference between carriers of co-located adjacent channel operators Agreement: To evaluate acceptable [PSD] difference from UE side between wanted signal and un-wanted signal based on the deployment where BSs from different operators are collocated but their radio units (RUs) are not shared. - The [PSD] difference means the difference between unwanted signal and the wanted carrier with the lower PSD - Start evaluation with wanted signal level currently existing ACS requirements as baseline and to see the performance difference

- Discussion

OPEN ISSUES #### Issue 2-3: Signaling related #### Issue 2-4: Discussion on semi-statically switch hardware resources #### Issue 3-3: Observations of impacts on DL Rx requirements RAN4 discuss if following general aspects can be agreeable: - The largest interferer power in-gap shouldn’t larger than -25dBm for fragmented carriers using a single Rx RF chain - It is proposed to consider only same SCS for both CCs to ease receiver design for the common RF chain - ΔRIBNC for SCC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination. Furthermore, ΔRIBNC needs to apply to PCC too, although the value of ΔRIBNC can be different for PCC and SCC - RAN4 shall not consider requirements for fragmented carriers which will result in performance degradation as compared to legacy NC IB CA or – only in case of a strong in-gap interferer – single CC operation

## Impact of asymmetrical target CCs

As has been discussed in the papers [2][3] in the last two meetings, when the two target CCs are asymmetrical allocated with different bandwidth, the image of the in-gap blocker may fall into the frequency range of the target CC. Moreover, in [4] the scenario is distinguished into two more detailed cases: The image may be introduced by the interferer falling in the ACS/IBB1/IBB2 range of the victim CC or another non-victim CC.

As the analysis above, the study on the upper limit of the in-gap blocker’s absolute power and the relative power of the in-gap blocker comparing to the target CC (further, the PSD difference) should be divided into two aspects based on whether the interference image falling into the target CCs. When blocker’s image overlapping with the target CC, then the power limitation of the blocker should be tighter, the degree of ACS\IBB\NBB relaxation should also be different with the non-overlapping cases.

Observation 1: Based on whether the interference image falls into the target CC, the evaluation of PSD difference and RF requirements should be divided.

Proposal 1: Divide section 6 and 7 of the TR into two parts: common cases, and special cases that the interference image overlaps with the target CC.

## Limitation of in-gap blocker

In the current spec, the absolute limitation of the interferer for ACS and IBB testing is as follows: For intra-band C CA the adjacent channel interferer on either side of the aggregated downlink signal should be limited up to -25dBm. For intra-band NC CA the adjacent channel interferer in-gap and out-of-gap should also be limited up to -25dBm. However, the two cases are based on the interferer is out of the passband of the filter and the attenuation performance could be guaranteed to some extent. But for the fragmented carrier case with fully shared RX chain, the in-gap interferer would fall into the filter’s passband and the filter could provide very limited suppression. Moreover, in the fully shared RX chain the interferer allocates between the two target CCs so that the analog filter's suppression of the interferer will not be greater than that of the target CC. In hence the limitation of the interferer’s absolute power should be stricter, at least lower than -25dBm.

Observation 2: The current power limitation of the interferer, -25dBm, is based on the premise that the interferer falls outside the passband of the RF filter.

Proposal 2: For fully shared RX chain the maximum acceptable absolute power of in-gap interferer should be less than -25dBm, specific value is FFS.

As has been specified in the spec that for fully separate and partially shared architecture, the absolute power level of the in-gap interferer and the relative power imbalance between it and the wanted signal is the test parameter and condition of ACS and IBB applying. With the corresponding test parameters, when the throughput is ≥ 95 % of the maximum throughput of the reference measurement channels then the ACS/IBB requirements are considered to fulfill. With RX sharing these requirements should be relaxed to different levels according to the frequency range and bandwidth of the in-gap interferer.

However, there should be a trigger condition on whether RX sharing is allowed. When the condition is met then RX sharing could be executed and then the receive performance and demodulation effect could be verified further. If there is no trigger condition the UE would not know when to assess the receiving performance based on the relaxed set of requirements. RX chain sharing should be an optional capability and not mandatory, nor is it the default state for the UE. Therefore, it is illogical for the UE to first rely on the fully shared architecture for signal reception, then assess whether the demodulation performance meets the relaxed requirements. Instead, the UE should first determine some basic information, such as when the interference signal’s absolute power and relative power density difference satisfy the corresponding requirement then the RX chain could switch to shared state and the received signal could be evaluated by a different set of requirements.

Observation 3: RX chain sharing should be an optional capability and not mandatory, nor is it the default state for the UE. Therefore, there should be a trigger condition for allowing RX sharing.

Proposal 3: The absolute power level of in-gap interferer and the PSD difference between in-gap interferer and target CC should be used as one of the judging conditions for the UE to determine whether RX chain sharing is allowed.

Based on the discussion and WF in the last meeting, the currently existing ACS requirements are considered as a starting point. Therefore, the interference falling into ACS range could be studied firstly and the ACS requirements are assumed to be unchanged. At the same time, the changes to ΔRIBNC caused by Rx sharing do not affect test parameters as both the Power_interf and Power_targetCC will vary with changes in REFSENS, so the power difference between the Power_interf and Power_targetCC keeps the same with the current spec.

On the other hand, currently the SI focuses on the RX chain sharing of two NC CCs, the frequency range below 2700 MHz is prioritized and only CC bandwidths greater than 3 MHz are considered. Therefore, the interferer bandwidth for ACS/IBB should be always set to 5 MHz. Based on the assumption above, the PSD difference where interference falls into ACS range could be evaluated.

Table 7.5-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |
| Pinterferer4 | dBm |  | REFSENS + 45.5 dB | REFSENS + 42.5 dB | REFSENS + 39.5 – 10log10(BWChannel /20) |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

For the target CC and interferer, the relationship between Power difference and PSD difference is as follows:

Power difference = Pinterfer - Pwanted = PSD difference + 10log(BWinterferer/BWwanted)

So according to ACS case 1 in table 7.5-3, when the bandwidth of the target CC belongs to the second column 5/10MHz,

Pinterfer - Pwanted = (REFSENS + 45.5) – (REFSENS + 14) = 31.5

→ PSD difference = 31.5 - 10log(BWinterferer/BWwanted) = 31.5-10lg5+10lg(5 or 10)

= 31.5(BWwanted=5), or 34.5(BWwanted=10)

Similarly, when the bandwidth of the target CC belongs to the third column 15MHz,

Pinterfer - Pwanted = (REFSENS + 42.5) – (REFSENS + 14) = 28.5

→ PSD difference = 28.5 - 10log(BWinterferer/BWwanted)

= 28.5-10lg5+10lg15 = 33.3

When the bandwidth of the target CC belongs to the fourth column 20~100MHz,

Pinterfer - Pwanted = (REFSENS + 39.5 - 10lg(BWwanted /20)) – (REFSENS + 14) = 25.5 - 10lg(BWwanted /20)

→ PSD difference = 25.5 - 10lg(BWwanted /20) - 10log(BWinterferer/BWwanted)

= 25.5 +10lg20-10lg5 = 31.5

Table 7.5-4: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration4 | dBm | -56.5 |  | -53.5 | -50.5 + 10log10(BWChannel /20) |
| Pinterferer | dBm | -25 |  |  |  |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

According to ACS case 2 in table 7.5-4, the calculation procure is similar, and the results are the same:

When BWwanted=5M, PSD difference = 31.5.

When BWwanted=10M, PSD difference = 34.5.

When BWwanted=15M, PSD difference = 33.3.

When BWwanted=20~100M, PSD difference = 31.5.

Observation 4: According to the current test parameters of ACS, the PSD difference (between interferer and target CC) between case 1 and 2 with the same BWwanted is the same.

Observation 5: According to the current test parameters of ACS, the lowest PSD difference is 31.5 (BWwanted= 5M/20~100M), and the highest PSD difference is 34.5 (BWwanted = 15M).

Proposal 4: When the ACS remains unchanged, for blockers falling within the ACS range at frequencies lower than 2700 MHz, the maximum tolerable PSD difference should not be higher 31.5.

## Signaling aspects & CA configuration indication scheme

With the RX chains switching to shared architecture, the RF requirements should be adjusted and some RF chains could be released, potentially allowing for the aggregation of more CCs or MIMO layers. This would result in a wider variety of CA configurations supported by the UE. There are two kinds of methods for a UE to inform the network of new CA configurations it can support with adjusted RF requirements:

- The new CA configuration could be reported to the NW together with other default supported CA configuration, as they are related to the stable hardware capabilities. And it could be reported through a relative static mechanism, such as RRC signaling, as the current technology does. The RX chain state and CA configuration indication could be dynamically switched according to the environment.

- The new CA configuration could be separately reported to the NW when or after the RX chains switching to be shared, which is more dynamic but complex. When the RX chains could be switched to or after switching to the shared architecture, the UE reports to the NW that the new CA configuration it supports with the sharing architecture, and then the NW could send a new CA configuration to indicate to the UE that there are more CCs to be aggregated.

Proposal 5: The new CA configuration supported only by fully shared RX chains could be reported to the NW together with other default supported CA configuration.

## Fall back issue

When the prerequisite of sharing is not met, the UE should fall back to partially-shared or fully-separated architecture, and maybe non-CA state from [5]. The trigger factor for falling back may be that the output power (or PSD) of the in-gap interferer is too high to maintain sharing RX chain with good receiving performance, or the frequency separation between the two target CCs are too wide to be covered in one RX RF chain.

With the RX chains switching to being shared, the RF requirements should be adjusted and some RF chains could be released, potentially allowing for the aggregation of more CCs. This would result in a wider variety of CA configurations supported by the UE. However, these new CA configurations could only be indicated with shared RX chains. Also, when the RX chains fall back to separate architecture, the CA configuration should also switch to default group with less CCs combined. The abandoned CC could be one of the non-contiguous intra-band CCs or the other inter-band CC. For example, the CA config in original separate architecture is CA_nX(2A), after RX chain sharing it may change to CA_nX(2A)-nY as the two CCs in band nX could be received by only one RX RF chain. However, when the fall back mechanism is triggered then the shared architecture should switch to separate state and then the CA config would also fall back to CA_nX(2A) or CA_nX-nY, the decision on which CC to disconnect is up to the network.

Observation 6: The CA configurations supported by shared and separate RX architecture are not always the same, some CA configurations are supported only in the shared state.

Proposal 6: If the separate architecture does not support the CA configuration in shared state, then the fallback of the Rx chain will also require corresponding changes to the CA configuration, which CC should be abandoned is up to the NW.

- Conclusion

Observation 1: Based on whether the interference image falls into the target CC, the evaluation of PSD difference and RF requirements should be divided.

Proposal 1: Divide section 6 and 7 of the TR into two parts: common cases, and special cases that the interference image overlaps with the target CC.

Observation 2: The current power limitation of the interferer, -25dBm, is based on the premise that the interferer falls outside the passband of the RF filter.

Proposal 2: For fully shared RX chain the maximum acceptable absolute power of in-gap interferer should be less than -25dBm, specific value is FFS.

Observation 3: RX chain sharing should be an optional capability and not mandatory, nor is it the default state for the UE. Therefore, there should be a trigger condition for allowing RX sharing.

Proposal 3: The absolute power level of in-gap interferer and the PSD difference between in-gap interferer and target CC should be used as one of the judging conditions for the UE to determine whether RX chain sharing is allowed.

Observation 4: According to the current test parameters of ACS, the PSD difference (between interferer and target CC) between case 1 and 2 with the same BWwanted is the same.

Observation 5: According to the current test parameters of ACS, the lowest PSD difference is 31.5 (BWwanted= 5M/20~100M), and the highest PSD difference is 34.5 (BWwanted = 15M).

Proposal 4: When the ACS remains unchanged, for blockers falling within the ACS range at frequencies lower than 2700 MHz, the maximum tolerable PSD difference should not be higher 31.5.

Proposal 5: The new CA configuration supported only by fully shared RX chains could be reported to the NW together with other default supported CA configuration.

Observation 6: The CA configurations supported by shared and separate RX architecture are not always the same, some CA configurations are supported only in the shared state.

Proposal 6: If the separate architecture does not support the CA configuration in shared state, then the fallback of the Rx chain will also require corresponding changes to the CA configuration, which CC should be abandoned is up to the NW.

- References

- R4-2417201, WF on NR FR1 DL fragmented carriers study, MediaTek Inc. RAN4#112bis.

- R4-2416040, Discussion on the scenarios and the general aspects on the FR1 fragmented carriers study, CHTTL, RAN4#112bis.

- R4-2415758, Discussion on methods for reducing the number of UE RX chains, vivo, RAN4#112bis.

- R4- 2416138, Discussion on UE Rx chain architecture and network assumption, MediaTek Inc., RAN4#112bis.

- R4-2415804, On methods for reducing the number of UE Rx chain, Huawei, HiSilicon, RAN4#112bis.