3GPP TSG-RAN4 Meeting #113	R4-2418170

# Orlando, US, Nov 18 – 22, 2024

Agenda item:			7.1.1.2.1

Source:	vivo

Title:	Discussion on UE RF power enhancement for NR single carrier

Document for:	Discussion

- Introduction

In RAN#112bis meeting, the WF[1] has been agreed, and the remaining issue is as follows:

### Sub-topic 1-1: Approaches to enable MPR reduction for scenario 1 #### Issue 1-1-1: Approaches for scenarios 1-1 and 1-2 (see Reference for description of scenarios) - WF - Strive for a unified solution which can accommodate both scenario 1 and scenario 2 under the framework of converting outer RB allocation to inner RB allocation. - It does not preclude separate solutions for each applicable scenario - No relaxation of ACLR/SEM/SE values Issue 1-2-1: Clarification of multi-CC scenario - WF - The approach of converting outer RB allocation to inner RB allocation for MPR reduction should consider multi-carrier spectrum scenario from NW perspective while the UE is configured for single CC operation. Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation - WF - The approach should consider the following aspects - unified solution could be applicable for both scenarios 1 and scenario 2 - unified solution considering the spectrum allocation status of operators, e.g multi-CC case from NW perspective, to better leverage the feature of MPR reduction - Solution could benefit RedCap and non-RedCap with UE CBW less or even identical to BS CBW - Solution could include both symmetric and asymmetric CBW extension - Whether the extended UE CBW could exceed the BS CBW for different scenarios - Signalling overhead is considered for the proposed approach(s) #### Issue 1-2-3: Where to use IBE in the larger BS CBW - WF - IBE should be used between edges of UE CBW and extended UE CBW - FFS whether IBE could be used in the guard band of UE CBW - FFS the full RB allocation scenario - FFS the impact to testability #### Issue 1-2-4: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. - Proposal 2: ACLR, SEM and spurious emissions would be defined based on BS channel bandwidth. - WF - FFS in next meeting - ITU regulation of 250% necessary bandwidth should be considered in further analysis #### Issue 1-2-5: Boundary to apply SE - Proposals - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW. - Proposal 2: SE is applied at BS channel bandwidth. - WF - FFS in next meeting - ITU regulation of 250% necessary bandwidth should be considered in further analysis #### Issue 1-2-6: Which CBW is utilized as the basis for the OOBE requirement and the applicable boundary - Proposals - Proposal 1: The OOBE requirement and applicable boundary should be based on UE CBW. - Proposal 2: The OOBE requirement and applicable boundary is based on BS CBW. - WF - FFS in next meeting #### Issue 1-2-7: Ratio of extended CBW to UE CBW and to larger BS channel BW - WF - To discuss the following aspects for the extended UE CBW: - Whether the extended CBW with new boundary of FOOB should comply with the recommendation by ITU-R on necessary bandwidth in terms of SE for the original UE BW - Whether default or fixed extension, e.g. 1/2 UE CBW could be stipulated in the spec or the extension ratio could be a UE capability to fulfil the conversion of outer to inner - Whether the extended UE CBW could belong to the regular defined channel bandwidth - Whether some premise should be established for the extension size, e.g. - minimum extended CBW compared to original UE CBW - minimum BS CBW compared to UE CBW - frequency separation of the RB allocation to the BS edge - Other identified issues are not precluded #### Issue 1-2-8: Asymmetrical extended CBW approach - WF - Asymmetrical extension is considered, FFS the solution. #### Issue 1-2-10: Signaling aspects - WF - To further discuss the signalling aspects in conjunction with the solution to be adopted for MPR reduction. - Companies are encouraged to provide thinking on signalling impact together with the proposed solution

- Discussion

## 2.1 Emission requirements

### Applying range of IBE

After extending the UE CBW, the applied evaluation requirement between the edges of UE CBW and extended UE CBW should be discussed. Although the RBs in this “new guardband” could not be utilized in the actual UE, the frequency range still belongs to “in-band” concept as it is a kind of virtual bandwidth set for the UE, so IBE should be applied. Further, the general part of IBE consists of four equations and the max of them is filtered out as the final requirement:

IBE_general=max(PRB - 30 dB, )

Therefore, there are two elements to consider: NRB and ΔRB. They are both related to UE CBW and corresponding RB allocations: NRB represents the maximum transmission bandwidth configuration for each UE channel bandwidth, ΔRB is the starting frequency offset between the allocated RB and the measured non-allocated RB.

According to [2], the NRB of the IBE requirement refers to the total number of RB in the uplink, which should be based on the actual UE bandwidth indicated by the NW, and should not change with the virtual extension of UE CBW edge. Similarly, the calculation of ΔRB should also be limited within the actual UE CBW. The IBE requirement at the edge of actual UE CBW could be used as a reference to evaluate the radiation power level of the entire extended portion, and the guardband should be excluded from the evaluation as usual. The illustration is as follows:

Fig 1. IBE applying range after bandwidth expansion

If calculating IBE based on the entire extended UE CBW, then it would limit the extended bandwidth to the maximum transmission bandwidth range in the spec, i.e., after extension the whole UE CBW should not be wider than 100M or else the mapping NRB could not be found and IBE could not be calculated and evaluated anymore.

Observation 1: Calculating IBE based on the extended UE CBW would limit the extended UE CBW inside the maximum transmission bandwidth range of the spec.

On the other hand, since IBE is tested for each RB separately and the test results are also compared at the RB granularity, using the IBE requirement of the actual UE edge as a reference value for the entire extended portion will not only does not increase the testing burden but actually reduces it, as there is no need to calculate IBE for each individual RB in the extended portion anymore.

Observation 2: Using IBE of the actual UE CBW edge as a reference value to evaluate the emission level of the entire extended portion will reduce the testing burden.

Proposal 1: For IBE of both original CBW and extended CBW, NRB should be based on the original UE CBW, ΔRB should also be limited within the original UE CBW.

Proposal 2: The IBE requirement of the extension portion should be equal to that of the last non-allocated RB in the original UE CBW.

Based on the assumption above, there is another situation that needs to be discussed: Full RB allocation cases. Since there are no remaining unallocated RBs, there is no IBE requirement in the full-RB allocation of the original UE CBW. However, for the extended part, ΔRB can be set to 0 to indicate that the frequency difference between the allocated RB and the edge of the UE is 0, excluding the range occupied by the guard band. At the same time, NRB is equal to LCRB in full RB allocation. Therefore, the IBE requirement could be simplified to the following formula and it is higher than the IBE of the first non-allocated RB of a partial RB allocation. And the illustration is as follows.

IBE_general = max⁡(PRB - 30 dB,

max⁡{-25, 20*log10EVM -3 +5LCRB,

-57dBm + 10log10(SCS/15kHz)- PRB})

Fig 2. IBE applying range after bandwidth extension (full RB allocation for the original UE CBW)

Proposal 3: For UE CBW with full RB allocation, the IBE of the extended part could be simplified to:

IBE_general = max⁡(PRB - 30 dB,

max⁡{-25,  20*log10EVM -3 +5LCRB,

-57dBm + 10log10(SCS/15kHz)- PRB})

### Change of the spurious emission boundary

According to the WF of meeting #111, not relaxing the level of spurious emission requirements has become a consensus. However, the application range of SE still should be altered with the edge of the UE CBW shifting and the change of ACLR and SEM application range. Otherwise, SE would become the limiting factor for MPR reduction, and the application range will partially overlap with SEM, and it would not adhere to the definition and motivation of SE evaluation in 3GPP: Spurious emissions are emissions which are caused by unwanted transmitter effects but exclude out of band emissions unless otherwise stated. In ITU regulation, the definition of SE is similar: Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products but exclude out-of-band emissions.

Proposal 4: The application range of SE should be altered with the shifting of the edge of the UE CBW and the change in the application range of ACLR and SEM.

### The start point of ΔfOOB & integral region of OOBE

ACLR and SEM would be applicable from the edge of extended CBW, and the integral region and the boundary of OOBE should be based on UE CBW. From one side, SE need to be evaluated from the edge of SEM and according to ITU-R regulations, the SE should apply from 250% of the centre frequency of the “necessary bandwidth”. In TR 38.803, it points out for the wider carriers and higher bands the “250% rule” could be replaced by the separation between 150% and 250% of the necessary bandwidth. In total, the acceptable separation ratio should not exceed 250%. If the integral region is based on extended UE CBW, the boundary of OOBE would be extended to not only farther than current 3GPP requirements but also disobey the ITU requirement.

For example, in Figure 1(a), the integral CBW of OOBE is equal to the UE CBW. The SE applies from the edge of SEM and the frequency spacing with centre of UE CBW is 200% * UE CBW+5M, which meets with the requirement of ITU. However, in Figure 1(b), the integral CBW of OOBE is equal to the extended UE CBW. And the SE applies from 300% * UE CBW+5M, which is too much far away from the UE centre especially for the wider UEs and the protection effect of UE co-existence would be weaken a lot.

Proposal 5: For OOBE, the integral region should be based on UE CBW, and the starting point should be applied from edge of the extended UE CBW.

(a)

(b)

Fig 3. Comparison of two integral bandwidths

## 2.2 MPR requirements & Signaling mechanism

### BW basis of MPR evaluation

In the last meeting, it has been agreed that scenario1 and 2 could be accommodated by a unified solution, i.e., extending the UE CBW to switch the outer allocation to inner. At the same time, multi-carrier spectrum scenario from NW perspective should also be considered, which releases the frequency range beyond the BS CBW for extension. However at least for scenario2, BS RF CBW is still the extension boundary. From one side, considering multi-carrier scenario, MPR could not be evaluated based on BS CBW. Because when the UE is as wide as BS, then the extension will have to occupy the range outside of the BS CBW. From the other side, as the analysis above, the applying starting point of SE should consider ITU regulation which should not exceed 250% of the centre frequency of the “necessary bandwidth” no matter how wide the UE CBW is. However, when extending a narrow UE CBW to a wide BS CBW and apply SE from such a far starting point will disobey the regulation and lead to the degradation of the radiation suppression ability of the requirement.

Observation 3: Considering the multi-carrier scenario and the ITU regulation for SE, MPR should not always be evaluated based on the BS CBW.

As for the advertised BS transmission BW concept from several companies, there are also some cons:

From the analysis of [3] it can be seen that the advertised BS CBW may be not totally aligned with the current spec requirements. Therefore, for some extension CBW the corresponding extended RB number may not be able to get at all, also to NRB. And the extension CBW should be limited as well. If considering the asymmetric extension then the RB0shift will be more difficult to calculate and indicate, which is troublesome for the NW. At the same time, if the advertised BS CBW is too wide and even wider than 2times of the UE CBW, then the relaxation would be too much and no more RB allocation could benefit from it.

Observation 4: Considering the extended RB number and the value of NRB, MPR should not be evaluated based on the uncertain BS transmission BW.

In short, simply extending by 2*the original UE CBW (i.e., extend at 0.5 times the UE CBW each side)is sufficient to convert all outer allocations to inner without wasting more extended bandwidth. Therefore, the evaluation of MPR and OOBE requirements should be based on the extended UE CBW. And the extension could be asymmetrical , the extension condition of any side is the gap between the original UE CBW and the RX RF CBW is at least 2 times the UE CBW, so the symmetrical extension should be based on the premise that the gap is wide enough on both sides.

At the same time, the extended UE CBW could exceed the BS CBW in the cases that twice of the original UE CBW is wider than the BS CBW. And there is no need to restrict that the extended UE CBW must belong to the channel bandwidth defined by the rule, since the sum of the extended bandwidths on both sides is equal to the UE CBW, and the extended RBs on both sides can share this extended bandwidth equally. Therefore, the extended RBs on each side can be equal to (NRB/2), where NRB is the maximum transmission bandwidth configuration of the original UE CBW, see Table 5.3.2-1.

Proposal 6: For the extension ratio on one side, only 0.5* UE CBW is considered.

- Symmetrical extension is 0.5 * UE CBW on each side.

- asymmetric extension is 0.5 * UE CBW on either side.

Proposal 7: The extended RBs on each side should be equal to NRB/2, where NRB is the maximum transmission bandwidth configuration of the original UE CBW, see Table 5.3.2-1.

### New inner region

When both sides are extended then all the outer allocations including full RB allocation could convert to inner and the MPR reduction could take effect no matter what the specific RB allocation is. However, for the asymmetric extension the RB region definition of the original UE CBW should change and the UE should judge its allocation according to the new definition. And based on the fixed extended ratio, the new RB region could be calculated directly but the UE’s RB allocation stays the same, so there is no need for the NW to indicate RB0shift and BS_NRB to the UE anymore.

Observation 5: When both sides are extended by 0.5* UE CBW, all the outer allocations including full RB allocation could convert to inner and the MPR could always refer to the inner value regardless the specific RB allocation.

Observation 6: Based on the fixed extended ratio, the new inner RB region of asymmetric extension could be calculated directly but the UE’s RB allocation stays the same, so there is no need for the NW to indicate RB0shift and BS_NRB to the UE anymore.

The following figure shows the range of RB allocation at maximum LCRB in the inner region. The original bandwidth of the UE is A and at maximum LCRB (0.5*NRB), the RB allocation range in the UE CBW should be 1/4A~3/4A. When the UE is extended by 0.5*UE CBW on the higher side, the bandwidth of the extended UE is 1.5A. At maximum LCRB (0.5*NRB_extended UE), the RB allocation range in the extended UE CBW should be 3/8A~9/8A. However, the RBs outside the original UE should not be scheduled so the furthest allocated RB should not exceed A.

According to this principle, assumes RBstart = X at maximum LCRB = A-X for the original UE with extension. For the lower side the parameter RBstart,Low should still follow the principle ≥ max(1, floor(LCRB /2)), and for RBstart,High there is no restriction other than ≤ NRB - LCRB. So A-X=2*X, deducing X= A/3. Therefore, LCRB for the UE with extension should be less than or equal to 2/3 * NRB.

Fig 4. The RB allocation range at maximum LCRB in the inner region, extension on the higher side

Similarly, when the extension is on the lower side, for the original UE with extension, the allowed RB allocation range in the new inner region is 0.5*A~X, X-0.5*A = 2* (1.5*A-X), deducing X= 7/6 *A. The LCRB for the UE with extension should also be less than or equal to 2/3 NRB, and for the higher side the parameter RBstart,High should still follow the principle ≤ NRB – RBStart,Low – LCRB, and for RBstart,Low there is no restriction.

Observation 7: For UEs that extend 0.5 times the original bandwidth on one side, LCRB should be less than or equal to 2/3 * NRB.

Fig 4. The RB allocation range at maximum LCRB in the inner region, extension on the lower side

As the spec shows, the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

RBStart,Low  = max(1, floor(LCRB /2))

RBStart,High = NRB – RBStart,Low – LCRB

If only the right side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be defined as:

RBStart,Low  ≤  RBStart  ≤  NRB – LCRB, and LCRB  ≤  ceil(2/3 *NRB)

Similarly, if only the left side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be defined as:

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2/3 *NRB)

Proposal 8: If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as:

RBStart_Low  ≤  RBStart  ≤  NRB – LCRB, and LCRB  ≤  ceil(2/3 *NRB)

If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as:

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2/3 *NRB)

### Signaling mechanism

The new mechanism of the UE to reduce MPR should be discussed, and based on the fixed extension ratio, i.e., 0.5 * UE CBW on each side, the following one could be considered.

First of all, it is up to the NW to judge the deployment environment, when the gap on one side of the UE CBW is at least greater than 0.5* UE CBW, the NW should send the signaling (e.g., ExtendLowerSideRel19, or ExtendHigherSideRel19) to the UE to indicate extension on the corresponding side. And then the UE would judge its allocation type based on the new inner region defined in Proposal 8, and the corresponding MPR would be used.

If the gap on both sides of the UE is at least greater than 0.5* UE CBW, the NW should send two signaling (e.g., ExtendLowerSideRel19 + ExtendHigherSideRel19) or one combined signaling (e.g., ExtendBothSidesRel19) to the UE to indicate that the extension is on both sides and the MPR of inner region could be used no matter what the RB allocation type is.

Proposal 9: Based on the fixed extension ratio, i.e., 0.5* UE CBW on each side, the following signaling mechanism could be considered:

- It is up to the NW to judge the gap width on both sides of the UE.

- When the gap on one side is ≥ 0.5* UE CBW, the NW sends the signaling (e.g., ExtendLowerSideRel19, or ExtendHigherSideRel19) to indicate the extension on the corresponding side.

When receiving the above extension signaling, the UE would judge its allocation type based on the new RB region defined in Proposal 8.

- When the gap on both sides is ≥ 0.5* UE CBW, the NW sends two signaling (e.g., ExtendLowerSideRel19 + ExtendHigherSideRel19) or one combined signaling (e.g., ExtendBothSidesRel19) to indicate the extension on both sides.

When receiving the above extension signaling, the MPR of inner region could be used regardless of the RB allocation type.

The above extension signaling is rather stable as it depends on the original UE CBW and the gap width, and will not change with the RB allocation state, so the signaling overhead is very low. At the same time, as the extension ratio is fixed it should be stipulated in the spec, and there should be no other UE capability other than support for the BW extension.

Observation 8: The extension signaling overhead is very low as it only depends on the original UE CBW and the gap width and will not change with the RB allocation state.

Proposal 10: The extension ratio on both sides, 0.5* UE CBW should be stipulated in the spec, and there should be no other UE capability other than support for the BW extension.

- Conclusion

Observation 1: Calculating IBE based on the extended UE CBW would limit the extended UE CBW inside the maximum transmission bandwidth range of the spec.

Observation 2: Using IBE of the actual UE CBW edge as a reference value to evaluate the emission level of the entire extended portion will reduce the testing burden.

Proposal 1: For IBE of both original CBW and extended CBW, NRB should be based on the original UE CBW, ΔRB should also be limited within the original UE CBW.

Proposal 2: The IBE requirement of the extension portion should be equal to that of the last non-allocated RB in the original UE CBW.

Proposal 3: For UE CBW with full RB allocation, the IBE of the extended part could be simplified to:

IBE_general = max⁡(PRB - 30 dB,

max⁡{-25,  20*log10EVM -3 +5LCRB,

-57dBm + 10log10(SCS/15kHz)- PRB})

Proposal 4: The application range of SE should be altered with the shifting of the edge of the UE CBW and the change in the application range of ACLR and SEM.

Proposal 5: For OOBE, the integral region should be based on UE CBW, and the starting point should be applied from edge of the extended UE CBW.

Observation 3: Considering the multi-carrier scenario and the ITU regulation for SE, MPR should not always be evaluated based on the BS CBW.

Observation 4: Considering the extended RB number and the value of NRB, MPR should not be evaluated based on the uncertain BS transmission BW.

Proposal 6: For the extension ratio on one side, only 0.5* UE CBW is considered.

- Symmetrical extension is 0.5 * UE CBW on each side.

- asymmetric extension is 0.5 * UE CBW on either side.

Proposal 7: The extended RBs on each side should be equal to NRB/2, where NRB is the maximum transmission bandwidth configuration of the original UE CBW, see Table 5.3.2-1.

Observation 5: When both sides are extended by 0.5* UE CBW, all the outer allocations including full RB allocation could convert to inner and the MPR could always refer to the inner value regardless the specific RB allocation.

Observation 6: Based on the fixed extended ratio, the new inner RB region of asymmetric extension could be calculated directly but the UE’s RB allocation stays the same, so there is no need for the NW to indicate RB0shift and BS_NRB to the UE anymore.

Observation 7: For UEs that extend 0.5 times the original bandwidth on one side, LCRB should be less than or equal to 2/3 * NRB.

Proposal 8: If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as:

RBStart_Low  ≤  RBStart  ≤  NRB – LCRB, and LCRB  ≤  ceil(2/3 *NRB)

If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as:

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2/3 *NRB)

Proposal 9: Based on the fixed extension ratio, i.e., 0.5* UE CBW on each side, the following signaling mechanism could be considered:

- It is up to the NW to judge the gap width on both sides of the UE.

- When the gap on one side is ≥ 0.5* UE CBW, the NW sends the signaling (e.g., ExtendLowerSideRel19, or ExtendHigherSideRel19) to indicate the extension on the corresponding side.

When receiving the above extension signaling, the UE would judge its allocation type based on the new RB region defined in Proposal 8.

- When the gap on both sides is ≥ 0.5* UE CBW, the NW sends two signaling (e.g., ExtendLowerSideRel19 + ExtendHigherSideRel19) or one combined signaling (e.g., ExtendBothSidesRel19) to indicate the extension on both sides.

When receiving the above extension signaling, the MPR of inner region could be used regardless of the RB allocation type.

Observation 8: The extension signaling overhead is very low as it only depends on the original UE CBW and the gap width and will not change with the RB allocation state.

Proposal 10: The extension ratio on both sides, 0.5* UE CBW should be stipulated in the spec, and there should be no other UE capability other than support for the BW extension.

- References

- R4-2417204, WF on power domain enhancements, Huawei, HiSilicon, RAN4 #112bis.

- R4-080678, UE In-band Emission Requirements, Qualcomm Europe, RAN4#46-bis.

- R4-2415273, Using single CC MPR and ULCA related emissions when only one CC has RBs allocated, Skyworks Solutions, Inc. RAN4 #112bis.