3GPP TSG-RAN WG4 Meeting # 113 	R4-2418175

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Co-existence evaluation for 14800 to 15350 MHz frequency range

Agenda Item:	7.2.4.1

Document for:	Approval

- Introduction

Issue 3-2: Scope of scenarios

Agreement:

- Limit simulations to urban macro

Issue 3-9: ISD for simulation

Agreement:

- 450m

Issue 4-17: BS antenna parameters

For Urban Macro:

Agreement:

- Make assumption that reply can say that array size is [3072 (16*24, SA size 4)], and the TR will document that other arrays are possible.

- Double check the coverage and [3072] before making final decision on reply

  - If coverage issues is found for [3072], minimum array size [4096] to achieve coverage could be discussed

UE-type discussion

- LS

  - Only one UE antenna model to be captured.

  - 1Tx omnidirectional antenna UE, understood as the worst case (highest ACIR values) assumptions for sharing studies and not on deployment, coverage, implementation, or requirement aspects.

In [1][2], the several co-existence assumptions for 14800 to 15350 MHz were agreed:

In this contribution, we provide the simulation results based on the latest assumptions.

- Discussion

The Table I summarize the simulation results based on the agreements in the last meeting, and both co-ordination and non-coordination are provided.

Table I Co-existence results under coordinated and un-coordinated

| Urban macro |  | ACIR (UL average throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| layout | UE antenna | 10 | 15 | 20 | 25 | 30 |
| coordinated | Omnidirectional | 0.80 | 0.38 | 0.18 | 0.08 | 0.04 |
| un-coordinated |  | 0.59 | 0.30 | 0.14 | 0.007 | 0.03 |

|  |  | ACIR (UL 5% cell edge throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| layout | UE antenna | 10 | 15 | 20 | 25 | 30 |
| coordinated | Omnidirectional | 5.77 | 2.82 | 0.93 | 0.14 | 0 |
| un-coordinated |  | 5.17 | 2.26 | 1.32 | 0.76 | 0 |

|  |  | ACIR (DL average throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| layout | UE antenna | 10 | 15 | 20 | 25 | 30 |
| coordinated | Omnidirectional | 3.11 | 1.79 | 0.91 | 0.46 | 0.24 |
| un-coordinated |  | 2.52 | 1.41 | 0.77 | 0.37 | 0.19 |

|  |  | ACIR (DL 5% cell edge throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| layout | UE antenna | 10 | 15 | 20 | 25 | 30 |
| coordinated | Omnidirectional | 17.26 | 10.79 | 5.12 | 2.17 | 1.23 |
| un-coordinated |  | 17.71 | 10.13 | 6.21 | 2.67 | 1.49 |

Based on the results above, for the UL scenario, the required the ACIR is around 12 dB and for the DL scenario, the required ACIR is around 22 dB.

Observation 1: The required the ACIR for UL is 12 dB and 22 dB for DL.

Based on the ACIR, we can further discuss the ACLR/ACS for UE and BS. The following table show the ACLR/ACS for UE and BS based on the ACIR above.

Table II ACS&ACLR for UE and BS

| UL ACIR = 12 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| UE ACLR/dB | 15 | 13 | 12 | 12 | 12 |
| BS ACS/dB | 15 | 20 | 25 | 30 | 35 |
| DL ACIR = 22 dB |  |  |  |  |  |
| UE ACS/dB | 25 | 23 | 23 | 22 | 22 |
| BS ACLR/dB | 25 | 30 | 35 | 40 | 45 |

It unnecessary to force UE and BS behave better if the ACIR is already achieved, for UL, the UE ACLR can be 12 dB and BS ACS is 25 dB; For DL the UE ACS can be 23 dB and the ACLR for BS is 35 dB.

Proposal 1: Take the following value for the ACS/ACLR in the LS reply

- BS ACS = 25 dB and BS ACLR = 35 dB

- UE ACS = 23 dB and UE ACLR = 12 dB

- Conclusion

In this contribution, we provide analysis on the co-existence simulation results and ACLR/ACS requirement.

Observation 1: The required the ACIR for UL is 12 dB and 22 dB for DL.

Proposal 1: Take the following value for the ACS/ACLR in the LS reply

- BS ACS = 25 dB and BS ACLR = 35 dB

- UE ACS = 23 dB and UE ACLR = 12 dB

- References

- R4-2417107	WF on 15GHz BS parameters, ZTE, RAN4#112bis

- R4-2417108	WF on 15GHz UE parameters, Qualcomm, RAN4#112bis