3GPP TSG-RAN WG4 Meeting # 113												R4-2418185

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo, Huawei

Title: 	TP on the intermediate UE Tx RF requirement

Agenda Item:	7.21.3.3

Document for:	Approval

- Introduction

In the previous meeting, the RF requirements for the intermediate UE were discussed, and this contribution summarize the conclusion so far.

- Text proposal

## <<Start of Change>>

6.7.3	Intermediate node (UE)

The intermediate UE, under NW control, can send Ambient IoT data and/or signalling to the device, and receive the backscatter signal from the device. The intermediate UE can also operate as a legacy NR UE. For simplicity, non-concurrent operation of Ambient IoT and NR for intermediate UE is assumed, and focus on the case where the intermediate UE acts as an A-IoT Reader.

Considering the intermediate UE need to support both AIoT and NR operation and the requirements in NR and AIoT may be quite similar, it is possible to verify or define some requirements only on the NR side.

6.7.3.1 TX

- Maximum output power

The goal of the maximum output power is to achieve the DL coverage. To reuse of the legacy NR UE hardware, PC3 NR UE can serve as a baseline for intermediate UE using FDD UL spectrum as R2D spectrum. High power UE is not precluded in the future release for better coverage.

- Output power dynamics

Power control is not designed for A-IoT R2D link, and intermediate UE transmit at maximum output power, making a minimum output power and power control accuracy requirements unnecessary.

Transmit OFF power is defined to ensure the UE will not affect the reception of other UE or BS when the transmitter is inactive. Since the AIoT and NR share same hardware at Tx side is a typical assumption, it is enough to verify this requirement in NR side and not need to be defined for AIoT.

For the transmit ON/OFF time mask, refer to the RFID specification, a settling time requirement is used to assess signal ripple characteristics. It can be determined whether or not this settling time is necessary in WI phase.

- Transmitted signal quality

Frequency error mainly depends on the timing and synchronization performance and PLL performance within transceiver chain. A-IoT intermediate UE is expected to reuse the existing timing and synchronization network directly. Meanwhile, similar PLL performance of A-IoT intermediate UE as NR UE is also expected as they are operating at the same frequency range. With the above considerations, frequency error requirement of NR UE can be considered reusing for A-IoT intermediate UE.

For legacy NR UE，transmit modulation quality includes EVM, carrier leakage and In-band emissions. EVM may not be suitable for R2D OOK waveform, but a requirement to guarantee the modulation quality is still needed.  As a reference, the EPC RFID uses RF envelope to measure the transmitted signal quality.

The carrier leakage requirement intends to reduce the DC offset which will distort the constellation for QAM signal. For OOK signal, the DC offset will increase the amplitude of waveform and the carrier leakage should also be low enough to ensure the received signal still within the dynamic range. Considering OOK/BPSK is less sensitive than the higher order modulation in NR when carrier leakage exists, the NR requirement is enough to limit the impairment and no need to defined same requirement for AIoT.

The in-band emission in NR can limit the interference between UEs, but when intermediate UE perform as AIoT reader, this requirement is unnecessary.

- Output RF spectrum emissions

Output RF spectrum emissions for legacy NR UE includes occupied bandwidth, out of band emission (ACLR), spurious emissions and transmit intermodulation.

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage β/2 (=0.5%) of the total mean transmitted power. Occupied bandwidth: Potentially needed for intermediate UE. The specific value is dependent on the physical layer design of R2D, which has not yet been finalized at this stage.

ACLR can be defined based on the co-existence study.  During SI co-existence evaluation, the IBE of legacy NR is used as emission leakage model, and performance degradation is observed in the results, so the ACLR better than IBE is required, and the details can be determined in WI phase.

Existing spurious emission requirement for legacy NR UE can be applicable, since this is coming from the ITU-R recommendation SM.329.

The transmit intermodulation in NR is the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna. Since the AIoT and NR share same RF hardware in Tx side, the non-linear performance of component can be verified in NR side and it is unnecessary to define it for AIoT intermediate UE.

A summary for TX part requirements is captured in the Table 6.7.3.1-1.

Table 6.7.3.1-1 Summary for TX part requirements for intermediate UE

| RF Requirement for A-IoT intermediate UE- TX part |  |  | Potentially needed or not |
| --- | --- | --- | --- |
| Maximum output power |  |  | YES |
| Output power dynamics | Minimum output power |  | NO |
|  | Transmit OFF power |  | NO |
|  | Transmit ON/OFF time mask |  | TBD in WI phase |
|  | Power control |  | NO |
| Transmitted signal quality | Frequency error |  | YES |
|  | Transmit modulation quality | EVM | YES |
|  |  | Carrier leakage | NO |
|  |  | In-band emissions | NO |
| Output RF spectrum emissions | Occupied bandwidth |  | YES |
|  | ACLR |  | YES |
|  | Spurious emissions |  | YES |
|  | Transmit intermodulation |  | NO |

## <<End of Change>>

- References

[1] R4-2417088, WF on impacts of Ambient IoT on RF requirements, Moderator (Huawei), RAN4#112bis, Oct.,2024