# 3GPP TSG-RAN WG4 Meeting #113	R4-2418191

# Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on UE RF requirements for less than 5MHz in FR1-NTN bands

Agenda Item:	7.8.3.2

Document for:	Approval

Introduction

In the last meeting, we further discussed UE RF requirements for less than 5MHz in FR1-NTN bands. All the open issues for RF requirements are captured in [1]. In this contribution, we will discuss the UE RF requirements for less than 5MHz in FR1-NTN bands.

Discussion

The remaining issues for less than 5MHz for NTN are captured in WF [1].

Tx requirements

For NR-NTN UE, the Tx requirements will be based on PC3 for 3MHz channel bandwidths in FR1-NTN bands.

For MPR requirements, FR1-NTN just refers to the requirements defined in TN. In this case, no extra MPR work needs to be performed for 3MHz since TN already specifies the requirements. However, for A-MPR, extra work needs to be evaluated considering the addition of 3MHz for FR1-NTN bands. We suggest to evaluate A-MPR for 3MHz channel bandwidth for some specific NS values.

For other Tx requirements (Min/OFF power, OBW, SEM, ACLR), it is suggested to extend them to support 3MHz since no big issues were found.

Rx requirements

For NR-NTN UE Rx requirements, one work is to define 3MHz REFSENS requirements for bands n256, n255, n254.

For 3MHz REFSENS, the transmission bandwidth is 15RB with 15kHz if we follow the NRB number from TN. Based on this Rx BW, we can calculate the REFSENS for 3MHz as follows:

| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 | -93.8 |
| n256 | 30 |  |  | -96.6 | -94.6 | -94.0 |
|  | 60 |  |  | -97.0 | -94.9 | -94.2 |
|  | 15 | -102.2 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |
|  | 60 |  |  | -97.0 | -94.9 |  |

For the Uplink configuration for reference sensitivity, we need to add 3MHz as follows:

Table 7.3.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  |  | 24 | 36 | 50 | FDD |
|  | 60 |  |  | 10 | 18 | 24 |  |
|  | 15 | 15 | 25 | 50 | 75 | [75] |  |
| n255 | 30 |  |  | 24 | 36 | [36] | FDD |
|  | 60 |  |  | 10 | 18 | [18] |  |
|  | 15 | 15 | 25 | 50 | 75 |  |  |
| n254 | 30 |  |  | 24 | 36 |  | FDD |
|  | 60 |  |  | 10 | 18 |  |  |

For other Rx requirements (Maximum input level, ACS, blocking…), we suggest to extend them to support 3MHz since there were no big issues.

Conclusion

This contribution discusses NR-NTN UE RF requirements for less than 5MHz. The following proposals are made:

Proposal 1: For UE Rx requirements for 3MHz,

- Define 3MHz REFSENS requirements for bands n256, n255, n254

| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 | -93.8 |
| n256 | 30 |  |  | -96.6 | -94.6 | -94.0 |
|  | 60 |  |  | -97.0 | -94.9 | -94.2 |
|  | 15 | -102.2 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |
|  | 60 |  |  | -97.0 | -94.9 |  |

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  |  | 24 | 36 | 50 | FDD |
|  | 60 |  |  | 10 | 18 | 24 |  |
|  | 15 | 15 | 25 | 50 | 75 | [75] |  |
| n255 | 30 |  |  | 24 | 36 | [36] | FDD |
|  | 60 |  |  | 10 | 18 | [18] |  |
|  | 15 | 15 | 25 | 50 | 75 |  |  |
| n254 | 30 |  |  | 24 | 36 |  | FDD |
|  | 60 |  |  | 10 | 18 |  |  |

- For other Rx requirements (Maximum input level, ACS, blocking…), extend them to support 3MHz

- References

[1] R4-2416587, Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4#112bis.