3GPP TSG-RAN WG4 Meeting # 113	R4-2418200

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	NR NTN HPUE Tx Requirements and feasiblility analysis

Agenda Item:	7.8.2.2.1

Document for:	Approval

Introduction

In RAN4#112, the HPUE for NTN [1] was started. A summary can be referenced in [2] and a WF [3] has been agreed. A paper [4] was submitted including an initial feasibility study and analysis for both NR-based and Iot-based NTN HPUE Tx requirements.

In RAN4#112bis, an comprehensive discussion paper including PA and duplexer analysis is incorporated in [5]. A summary is in [6] and WF in [7]. In addition, the duty cycle related discussion was treated with Iot documents and the WF was documented in [8].

In this paper, further discussion was made for NR-NTN, and some discussion on duty cycle for NR-NTN was provided.

HPUE Impact for Tx Requirements

RF Requirements

Currently the co-existence study is still on-going, and the latest agreements on Tx requirements are as following:

Issue 2-5: ACLR

Agreement:

- Co-existence study results and conclusion is an important factor for deriving out-band emission requirements

  - Check the derived ACLR for A-MPR and additional spurious emission against regulatory information.

Issue 2-8: SEM

Agreement:

- To re-use existing SEM requirements from PC3 to other PCs as starting point.

Issue 2-9: MOP tolerance

Agreement:

- Re-use NR TN TS 38.101-1 nominal MOP and +2/[-3] tolerance for PC2 for NR-NTN Non-Redcap HPUE

Here we further discuss some of the requirements:

ACLR

There is an even earlier agreement as following in [3]:

  - The study of NTN HPUE Tx requirements generally would have to consider co-existence results, UE implementation feasibility, and existing HPUE requirements.

Co-existence results is by far still the most basic reference that should be considered. For NR NTN, re-using the legacy TN components are basic starting point of the implementations, and reusing legacy requirements also have strong technical support. These trade offs can be done after the co-existence study are completed.

Proposal 1: For ACLR, considering trade offs between co-existence requirements and existing TN requirements.

SEM

In last meeting, there are proposals to consider more regulatory requirements dedicated to satellite communication. This is in theory, somewhat reasonable, since the SEM is most originated from regulatory. However, considering this is a rather basic requirement, changing the SEM for NTN may bring considerable difficulty in reusing TN components, since TN components would only consider SEM already defined and stabled for almost a decade.

Observation 1: SEM in TN has been stable for a long time. Changing NR NTN SEM would bring difficulty to reusing the TN components for NR-NTN.

Considering that chance to reusing TN components are important, it is proposed to keep the NR NTN SEM aligned with TN. Furthermore, the PC3 SEM was already reused for other power classes in TN, and it seems no difficulty to do this for NR-NTN.

Proposal 2: Re-use existing SEM requirements from PC3 to other PCs.

MRP & A-MPR

The MPR should be based on ACLR and SEM, and can be discussed after ACLR/SEM are set. There is one interested option, that if TN PC3 ACLR is used for PC2 NR-NTN, would a slightly lower MPR is possible and needed?

For A-MPR, more discussion can be made later.

Proposal 3: Discuss MPR/A-MPR further after SEM/ACLR is set.

Duty cycle Related

In last meeting, there were some preliminary discussion on duty cycle based approach. Generally there are some support of further discussion but concrete agreements were reached.

As pointed out earlier, the current duty cycle method defined in TN actually did not use the wording “duty cycle”, but “the percentage of uplink symbols transmitted in a certain evaluation period” which can be adaptable to both FDD/TDD. The main problematic part is that there are no definition of “a certain evaluation period” and no coordination of evaluation window between network and UE. This makes the scheme quite flexible, but difficult to be implemented. During discussion in FR2, these limitations of duty cycle method are already been known. However, the duty cycle was still also defined for FR2.

In case of NTN, what should be the basic behaviour? Is evaluation period needed to be specified? What if duty cycle are exceeded and what if not exceeded? These should discussed as part of the schemes.

Proposal 4: Discuss more on the duty cycle behaviours and schemes, and using existing TN as starting point.

Feasibility and Requirements applicability

In last meeting, some PA testing and analysis, plus duplexer analysis were made in [5]. For the UE architecture and type considerations, the following agreements have been made:

Issue 2-2: UE architecture/type considerations for NR-NTN PC2 UE

Agreement:

- Follow previous agreement

Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - ...

Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE

Agreement:

- Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions:

- FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified

- FFS whether PC1.5 handheld UE can be specified or not

Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE

Agreement:

- Confirm feasibility of PC1 1Tx for non-handheld UE.

- For PC1, at least 1Tx to be considered for requirements.

- Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE.

- Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE.

Now it is suggested that the discussion can focus on what requirements should be considered and defined, considering that there is already common understanding in what the typical implementation for NR NTN could be, and handheld and non-handheld UE basically share many requirements except MPR.

Based on the previous analysis, we have updated the feasibility summary as following for handheld UE, and it can be seen duplexer poses more restriction compared to PA:

Observation 2:

Table 1. Current restriction summary for handheld UE for NR NTN (With Update)

|  | PA | Duplexer | Conclusion based on PA&Duplexer |
| --- | --- | --- | --- |
| PC2 1Tx, PC1.5 2Tx | Feasible with sufficient margin | (Almost) feasible | [Feasible] |
| PC1.5 1Tx, PC1 2Tx | May feasible but insufficient margin | Not that feasible | [Not feasible] |
| PC1 1Tx | May not feasible | Not feasible | Not feasible. |
| PC1 3Tx (New) | May feasible | May feasible | FFS |

It should be noted that although PC1 3Tx was not considered before, it actually can be implemented by using three separate PC2 RF chain (30.77dBm), this implementation has the potential to using PC2 duplexer, thus may have some advantage in size and power consumption for a PC1 UE.

Based on the feasibility analysis and the already agreed WF and UE architecture, the following requirements applicability status are as following:

Observation 3:

Table 2: Current Status for requirements consideration and feasibility (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx] | Non-handheld |
|  | [3Tx] | [TBD] |
| PC1.5 | [1Tx] | Non-handheld |
|  | [2Tx] | [Both] |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

For us, PC1 2Tx is not that useful, since it can not use PC2 duplexer, and still need multiple large form factor duplexers. This implementation seems do not have enough advantages compared to 1Tx and 3Tx. Considering that there are already many cases, some simplifications are preferred.

For PC1.5, it is interesting to discuss whether 1Tx is needed or not. Since PC2 duplexer cannot be reused for PC1.5 1Tx, the need for this requirement seems would focus on non-handheld. Then, whether PC1 is better choice compared to PC1.5 may be arguable. We do not have strong views on this part. For 2Tx, we tend to define requirements, although there are still discussion on the feasibility. Considering the transmit power would be almost always welcome, and this seems the only option to use current components for TN.

In all, the following proposal is provided for requirements definition

Proposal 5: The following suggestions for requirements definition were made:

For PC1:

- 2Tx requirements need not be defined.

- 3Tx requirements can be further discussed, especially for handheld.

For PC1.5,

- 1Tx requirements can be further discussed for non-handheld;

- 2Tx requirements can be defined.

Table 3: Proposed cases for requirements definition

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx](discuss) | [Non-handheld] |
|  | [3Tx] (discuss) | [TBD] (discuss) |
| PC1.5 | [1Tx] (discuss) | Non-handheld |
|  | [2Tx] (discuss) | [Both] (discuss) |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Conclusion

In this paper, the following observations and proposals are provided:

RF Requirements

Proposal 1: For ACLR, considering trade offs between co-existence requirements and existing TN requirements.

Observation 1: SEM in TN has been stable for a long time. Changing NR NTN SEM would bring difficulty to reusing the TN components for NR-NTN.

Proposal 2: Re-use existing SEM requirements from PC3 to other PCs.

Proposal 3: Discuss MPR/A-MPR further after SEM/ACLR is set.

Duty cycle Related

Proposal 4: Discuss more on the duty cycle behaviours and schemes, and using existing TN as starting point.

Feasibility and Requirements applicability

Observation 2:

Table 1. Current restriction summary for handheld UE for NR NTN (With Update)

|  | PA | Duplexer | Conclusion based on PA&Duplexer |
| --- | --- | --- | --- |
| PC2 1Tx, PC1.5 2Tx | Feasible with sufficient margin | (Almost) feasible | [Feasible] |
| PC1.5 1Tx, PC1 2Tx | May feasible but insufficient margin | Not that feasible | [Not feasible] |
| PC1 1Tx | May not feasible | Not feasible | Not feasible. |
| PC1 3Tx (New) | May feasible | May feasible | FFS |

Observation 3:

Table 2: Current Status for requirements consideration and feasibility (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx] | Non-handheld |
|  | [3Tx] | [TBD] |
| PC1.5 | [1Tx] | Non-handheld |
|  | [2Tx] | [Both] |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Proposal 5: The following suggestions for requirements definition were made:

For PC1:

- 2Tx requirements need not be defined.

- 3Tx requirements can be further discussed, especially for handheld.

For PC1.5,

- 1Tx requirements can be further discussed for non-handheld;

- 2Tx requirements can be defined.

Table 3: Proposed cases for requirements definition

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx](discuss) | [Non-handheld] |
|  | [3Tx] (discuss) | [TBD] (discuss) |
| PC1.5 | [1Tx] (discuss) | Non-handheld |
|  | [2Tx] (discuss) | [Both] (discuss) |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412099, Feasibility study and Initial analysis for NTN HPUE RF Tx Requirements, vivo, RAN4#112

[5] R4-2415788, Feasibility study and analysis for NR NTN HPUE Tx Requirements, vivo, RAN4#112bis

[6] R4-2415228, Topic summary for [112bis][124] NR_IoT_NTN_HPUE_part1, Samsung, RAN4#112bis

[7] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung, RAN4#112bis

[8] R4-2417205, WF on HPUE for IoT NTN, vivo, RAN4#112bis