3GPP TSG-RAN WG4 Meeting # 113	R4-2418205

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	On 4Tx power degradation for SRS antenna switching

Agenda Item:	5.7.1

Document for:	Approval

Introduction

In RAN4#110, an issue has been raised for ΔPPowerClass for 4Tx for SRS antenna switching in [1], for this parameter has not been considered as what has been specified for 2Tx. In RAN4#110bis, a set of tentative solutions were raised in [2] with an accompanying draft CR in [3]. However, that scheme was deemed too complicated and not agreed, while a WF has been agreed in [4]. In RAN4#111, several papers were submitted in [5][6][7][8] on this topic, but still no agreements could be reached.

In RAN4#112, several papers including [9] is provided in which a complete history of this problem is summarized, and a WF [10] is agreed. This is a break through but still not settled.

In this paper, the history of the topic was re-summarized.

Discussion

In [5], it is illustrated that the root course of the current problem is that 4Tx has many variations of architectures, thus can not be bounded by ΔPPowerClass to reflect the restriction of architecture implementation like in 2Tx TxD. Even for 4Tx TxD we do not have an assumption for the architecture, let alone other 4Tx cases.

In [9], a complete history of this problem is summarized. The latest WF in [10] is as follows:

<Way forward/Agreement>: For PC1.5 UE not indicating txDiversity4Tx-r18 capability

-	Option 1: Limit the SRS transmit output power to:

  - 23dBm for t1rx AS capability by applying ΔPPowerClass = 6 dB

  - 26dBm for t2rx AS capability by applying ΔPPowerClass = 3 dB

  - 29dBm for t4rx AS capability by applying ΔPPowerClass = 0 dB

- For some cases the SRS power would be “unnecessarily” limited, e.g., t1rx AS capability, the UE equipped with 4 x 26dBm PAs, but this approach minimizes the uncertainty at the network side about which power was used by the UE.

-	Option 2: Limit the SRS transmit output power to:

  - 26dBm for t1rx AS capability by applying ΔPPowerClass = 3 dB. Allow extra 3dB relaxation by using ΔTRxSRS.

  - 29dBm for t2rx AS capability by applying ΔPPowerClass = 0 dB. Allow extra 3dB relaxation by using ΔTRxSRS.

  - 29dBm for t4rx AS capability by applying ΔPPowerClass = 0 dB.

- For some cases the UE would be allowed by the specification to unnecessarily lower the power by 3dB even if it is not needed, e.g., t1rx AS capability, the UE sounds with the PA of 26dBm, it is allowed to relax it to 23dBm, even if not needed. The second problem is, even if the UE which wants to sound with the PA of 23dBm legally wants to relax the power to 23dBm by applying ΔPPowerClass = 3dB and ΔTRxSRS=3dB, the network cannot know it, but this approach allows the UE to transmit higher power as much as possible if the UE uses the PA of 26 dBm as much as possible.

Note that other options are not precluded

<Way forward/Agreement>: For PC1.5 UE indicating txDiversity4Tx-r18 capability

-	Option 1: Limit the SRS transmit output power to:

  - 23dBm for t1rx AS capability by applying ΔPPowerClass = 6 dB

  - 26dBm for t2rx AS capability by applying ΔPPowerClass = 3 dB

  - 29dBm for t4rx AS capability by applying ΔPPowerClass = 0 dB

- Note that other options are not precluded

Among these options, we have slightly preference to allow extra 3dB relaxation by using ΔTRxSRS, let UE’s capability not be restricted is by our understanding more meaningful. However, it is not quite clear why txD capability would means different options. In addition, considering this issue has already postponed for long enough, compromise is really needed before this issue go to dead end.

Proposal: Slightly prefer to allow using ΔTRxSRS, but are open to all options to close this issue.

Conclusion

In this paper, the latest progress of this topic is raised and a simple proposal is provided.

Proposal: Slightly prefer to allow using ΔTRxSRS, but are open to all options to close this issue.

- References

[1] R4-2400341, Delta PpowerClsss for 4Tx for SRS antenna switching, Nokia, RAN4#110

[2] R4-2404658, 4Tx power degradation for SRS antenna switching, vivo, RAN4#110bis

[3] R4-2404659, Draft CR for 4Tx power degradation for SRS antenna switching, vivo, RAN4#110bis

[4] R4-2406590, WF on 4Tx SRS issues, vivo, RAN4#110bis

[5] R4-2408113, Further discussion on 4Tx power degradation for SRS antenna switching, vivo, RAN4#111

[6] R4-2407583, 4 Tx DeltaP_PowerClass for SRS antenna switching, Qualcomm, RAN4#111

[7] R4-2407682, Delta Ppowerclass and Delta TRxSRS for 4Tx for SRS antenna switching, Huawei, HiSilicon, RAN4#111

[8] R4-2409772, On DeltaP_PowerClass for SRS antenna switching for 4Tx, Ericsson, RAN4#111

[9] R4-2408928, Topic summary for [111][117] FR1_enh2_R18, Moderator (vivo), RAN4#111

[10] R4-2412089, On 4Tx power degradation for SRS antenna switching, vivo, RAN4#112

[11] R4-2414458, WF on Delta PPowerClass and TRxSRS for 4Tx, Huawei, HiSilicon, RAN4#112