3GPP TSG-RAN WG4 Meeting #113		R4-2418210
Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.12.2.2

Source:	vivo

Title:	Discussions on NTN OTA test method

Document for:	Approval

1	Introduction

This contribution provides our further views on NTN OTA test method and performance metric.

2	Discussion

RAN4 made good progress on usage scenarios and performance metric for NR-NTN in last meeting [1]:

Issue 3-1-1: Usage scenarios for NR-NTN mobile handheld UE

Agreements:

  - NTN UEs not supporting voice call or only supporting voice call with loud speaker etc. are only required and tested with hand only mode.

Issue 3-1-2: Check whether the UE orientation is arbitrary or directional during NR-NTN communication

Agreements:

  - For NR-NTN, the target usage of NR-NTN UE is close to normal NR UE.

Issue 3-2-1: Performance metric for NTN device

Agreements:

  - For NR-NTN UEs, take a whole sphere measurement with TRP and TRS as the performance metrics for all usage modes as baseline. Other performance metric can be considered if issue is identified.

  - For IoT-NTN UEs, FFS performance metric.

Issue 3-2-2: Whether performance metric should be different for GEO and NGEO for handheld UEs

Agreements:

  - FFS whether different metrics for GEO and NGEO would be needed.

IoT-NTN the usage scenario and performance metric are not decided, in our understanding, current IoT-NTN is mainly used for message or low data mode, but the Rel-19 NTN HPUE WI is developing PC2/PC1 IoT-NTN handheld device, which is targeting to support high speed data transmission and voice call service.

With the above condition, we believe the usage scenario and performance metric discussion of IoT-NTN should not be limited. More potential usage scenarios like head+hand voice call and hand only browsing mode should also be considered.

Observation 1: IoT-NTN will support high power class and voice call service.

Proposal 1: For IoT-NTN, the usage scenarios should not be limited. At least head+hand voice call and hand only browsing mode should be considered.

In last RAN4 meeting, there are open issues about the detailed test parameters for NTN n254 and IoT-NTN [1].

Issue 3-3-1: detailed test parameters for NR-NTN band n254

Agreements:

  - RAN4 further check whether the test parameters in Table 1 and Table 2 can be used as test parameters for NR-NTN band n254 OTA testing or not.

Issue 3-3-2: Detailed test parameters for IoT-NTN

Agreements:

  - RAN4 further check whether the following parameters can be used for IoT-NTN:

  - For NB IoT-NTN, select QPSK with 1@0 Ntones as TRP-related test parameter, QPSK with 12 Subcarriers as TRS-related test parameters.

  - For Cat M1 NTN, select QPSK with 1RB allocation as TRP-related test parameters, QPSK with 4 RB allocation as TRS-related test parameters.

Given the conformance test spec in RAN5 for NR-NTN band n254 is not finalized yet, we suggest RAN4 tentatively conclude the test parameters in Table 1 and Table 2 in issue 3-3-1 [1] for n254 OTA testing. In case any technical issue is identified, then RAN4 can further revisit these agreements.

In addition, the test parameters for IoT-NTN in issue 3-3-2 can also be confirmed.

Proposal 2: Conclude the test parameters in Issue 3-3-1 in WF [1] for NR-NTN n254 OTA testing.

Proposal 3: Conclude the test parameters in Issue 3-3-2 in WF [1] for IoT-NTN OTA testing.

3 Conclusion

In this paper, we provide our further views on NTN OTA.

Observation 1: IoT-NTN will support high power class and voice call service.

Proposal 1: For IoT-NTN, the usage scenarios should not be limited. At least head+hand voice call and hand only browsing mode should be considered.

Proposal 2: Conclude the test parameters in Issue 3-3-1 in WF [1] for NR-NTN n254 OTA testing.

Proposal 3: Conclude the test parameters in Issue 3-3-2 in WF [1] for IoT-NTN OTA testing.

4	References

- R4-2416590, Way Forward for [112bis][323] TRP_TRS_Ph3, vivo, RAN4#112bis