3GPP TSG-RAN WG4 Meeting #113	R4-2418297

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.22.2.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on OD-SSB requirements

Document for:	Discussion

- Introduction

During last meeting, RAN4 reached some agreements, but there are still many open items and it is summarized in WF [1]. This document focus on the OD-SSB requirements and proposes possible enhancements.

- Discussion

  - OD-SSB transmission timeline

In the last meeting, the time interval between OD-SSB triggering MAC CE and actual OD-SSB transmission. WF is copied below.

Issue 1-1-1: RAN1 LS - Background RAN1 has made the following working assumption that the value of T_min for MAC-CE based OD-SSB indication is the same value as UE’s MAC-CE processing time for SCell activation command. RAN1 would like to ask RAN4 if the working assumption of T_min=. can be confirmed to agreement. RAN1 #118 Agreement (Note: Delete the redundant wording) The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst which is at least T after the where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ m+3Nslotsubframe,μ+1m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.. - Proposal - Option 1: Ericsson, CMCC, CATT, Samsung, ZTE, Huawei, Mediatek, LG, [vivo] - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1. - Option 2: Qualcomm, Apple, [vivo] - Additional processing time (T△) is needed. - Option 2-1: T△ > 1 slot - Option 2-2: 5ms - Option 3: Huawei - RAN4 to further study the following scenarios if agreed by RAN1 - Scenario 2 in issue 1-3-1: when the OD-SSB is very close to the last always-on SSB in time domain - Scenario 3: when the OD-SSB and always-on SSB is in different frequency(frequency location w.r.t active BWP)

The discussion point in previous meeting is whether additional processing time is needed or not. According to RAN1 discussion, the time instance A is the starting point of OD-SSB. After UE receives MAC CE, first UE decode MAC CE and it requires 3ms after UE sends HARQ ACK based on legacy specification. Slot n and m in RAN1 LS indicate MAC CE receiving instance and HARQ ACK transmission instance respectively.

Figure 1: OD-SSB transmission timeline

Based on this timeline, the UE is not required any signal processing other than MAC CE decoding before time instance A. Thus additional time is not needed.

Observation 1: Based on the timeline assumed by RAN1, the UE is not required any signal processing other than MAC CE decoding before time instance A.

Proposal 1: RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1.

  - Known/unknown condition

In previous meeting, it is agreed that known/unknown condition is generally defined and legacy known condition principle can be reused.

Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded.

For case 1, since there are no always-on SSB, known/unknown condition should be defined based on only OD-SSB. For case 2, known/unknown condition should be defined based on both always-on SSB and OD-SSB.

Proposal 2: For case 1, since there are no always-on SSB, known/unknown condition should be defined based on only OD-SSB. For case 2, known/unknown condition should be defined based on both always-on SSB and OD-SSB.

- Conclusion

In this contribution, we propose possible enhancements.

Observation 1: Based on the timeline assumed by RAN1, the UE is not required any signal processing other than MAC CE decoding before time instance A.

Proposal 1: RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1.

Proposal 2: For case 1, since there are no always-on SSB, known/unknown condition should be defined based on only OD-SSB. For case 2, known/unknown condition should be defined based on both always-on SSB and OD-SSB.

References

[1] 3GPP, R4-2416916, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson