3GPP TSG-RAN WG4 Meeting # 113	R4-2418306

# Orlando, USA, 18th - 22th, Nov 2024

Source: 	Samsung

Title: 	Views on MSD for HPUE TN

Agenda Item:	7.1.1.1.1

Document for:	Discussion

- Introduction

The agreements and open issues of last meeting are captured in WF [1]. We present our further consideration in this paper on MSD issues.

- 2. Discussion/Conclusion

4 Options have been formed and discussed during last meeting. Among them, option1 and option3 are similar and the difference is the MSD requirements (by equations/LUT/Simplified guidance) are normative or informative. Though it is mentioned that it is up to RAN5 to decide whether/how to simplify the conformance test, our expectation/guess is that if the MSD requirements (by equations/LUT/Simplified guidance) are normative, high likely RAN5 will still test indicated power class with specific MSD values calculated by themselves, and if informative, RAN5 may would only test the default power class. Note that in RAN5 most requirements are only verified under indicated power class, except MOP and ACLR requirements.

(TS 38.521-1 clause 6.2.1, Test procedure for UE maximum output power)

(TS 38.521-1 clause 6.5.1, Test procedure for occupied bandwidth)

From our observation the group is already sick of calculating MSD, checking MSD and correcting MSD, which is super time consuming but actually the values specified cannot be fully verified by conductive tests given part of MSD values having OTA attribute is the determinants. In addition, defining MSD in spec delay the NW deployment as the “HPUE Note” has to be added after the MSD evaluation according to current procedure. In our view Rel-19 is the appropriate time to reconsider it and make some simplification, from procedure perspective.

Furthermore, as mentioned by companies, this issue has to be discussed thoroughly with all stakeholders, especially operators, and some operators prefer Option1 and some prefer Option2 during last meeting.

Considering all views from vendors and operators and also the initial examples provided by Skyworks and Nokia captured in the annex of WF, we feel Option1 maybe a more desirable way for sake of smooth transition. And we can also accept Option3 as compromise. More specific, for Option1, we support to replace the current MSD tables with equations/LUT/Simplified guidance thus RAN4 does not need to introduce specific MSD values with innumerable TPs anymore, all combos all eligible for higher power class. Note current procedure for PC3 does not change.

Proposal 1: Option1(with minor modification) is our first preference. We can also accept Option 3.

Option 1:

  - RAN4 agrees on the following directions in this meeting and continues on details in subsequent meetings.

  - RAN 4 defines the default power class inter-band CA/DC and intra-band CA/DC MSD requirements, and RAN4 specifies equations/simplified guidelines/lookup tables “LUT” that define the relationship between the default power class MSD and the HPUE MSD requirements to replace the existing HPUE MSD tables:

  - Equations/LUT/Simplified guidelines identifying the HPUE MSD requirements are expected to be captured in RAN4 spec as normative requirements, in Rel-19 if possible, the details are FFS

  - FFS the expected spec changes and procedure changes (i.e., stop providing TP’s for each combination, remove all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19

- Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done

  - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus]

- Agree to capture just single worst case MSD (by Equations/LUT/Simplified guidelines) in TS for HPUE PC which is agnostic to number of TX, for each kind of MSD

NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done.

NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow.

- 3. Reference

[1] R4-2417098, WF on HPUE RF requirements, Samsung, Skyworks, Nokia, RAN4#112bis