3GPP TSG-RAN WG4 Meeting#113		R4-2418320

Orlando, United States, Nov 18th -22nd, 2024

Agenda item:	7.21.3.3

Source: 	Spreadtrum Communications

Title: 	Discussion on RF requirements impact for intermediate node (UE)

Document for:	Discussion

Introduction

In RAN4#112bis meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follow.

Discussion

TX requirements

Agreement: - Whether all the requirements defined need to be tested for intermediate UE can be further discussed in WI phase. - Whether requirements potentially needed or not is summarized in the following table: RF Requirement for A-IoT intermediate UE- TX part; Potentially needed or not. Maximum output power; YES, as RAN4#112 agreed. Output power dynamics; Minimum output power; NO. Transmit OFF power; TBD. Transmit ON/OFF time mask; TBD. Power control; NO. Transmitted signal quality; Frequency error; YES, as RAN4#112 agreed. Transmit modulation quality; EVM; YES. Carrier leakage; TBD. In-band emissions; TBD. Output RF spectrum emissions; Occupied bandwidth; YES, as RAN4#112 agreed. ACLR; Depends on the co-existence, as RAN4#112 agreed. Spurious emissions; YES, as RAN4#112 agreed. Transmit intermodulation; TBD.

Transmit off power

It is observed that excessive transmitter off power may increase the Rise over Thermal (RoT), thus reducing the cell coverage area of other UEs. If the off power of UE is too high, it means that the system floor noise of the whole cell has been raised. In order to meet a certain SNR requirement, the terminal receiving service will have to raise the transmission power, which in turn will cause higher noise for other UEs. As a result, the system capacity of the whole cell decreases. Whatever in factory or indoor, there must be multiple readers (the intermediate node (UE)). In our understanding, transmit off power should be defined to avoid increasing the system floor noise and save power consumption.

Proposal 1: Transmit off power is needed to define for the intermediate node (UE).The power level of transmit off power needs to be further discussed.

Transmit ON/OFF time mask

For the transmit ON/OFF time mask, the output power of the terminal during the rising edge/falling edge period when the transmission is off and on is mainly investigated to ensure the correct power output; Sending the wrong power will interfere with other channels or increase the transmission error of uplink channel. However, power control accuracy may not need to be like that of normal UE. Hence, transmit on/off time mask maybe is not needed for the intermediate node (UE).

Proposal 2: Transmit on/off time mask maybe not be necessary for the intermediate node (UE).

Carrier leakage

Carrier leakage is an additive sinusoid waveform whose frequency is the same as the modulated waveform carrier frequency. The receiver is A-IoT device for intermediate UE. These devices cannot cancel the carrier leakage, hence, it is necessary for intermediate UE to define carrier leakage.

Proposal 3: Carrier leakage is needed to define for the intermediate node (UE).

In-band emissions

According to the definition of in-band emission (IBE), IBE includes three parts: general, IQ image and carrier leakage. In our understanding, IQ image and carrier leakage should exist, but generally maybe rely on RAN1’s design for channel design. But anyway, in-band emission needs to be considered.

Proposal 4: In-band emission needs to be considered.

Transmit intermodulation

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna. Intermediate UE is similar to normal UE. It has non-linear elements (i.e., PA,).Hence, transmit intermodulation is needed to define for the intermediate node (UE).

Proposal 5: Transmit intermodulation is needed to define for the intermediate node (UE).

Conclusions

In this paper, we provide our views on RF requirements impact of intermediate node (UE), we have made the following proposals.

Proposal 1: Transmit off power is needed to define for the intermediate node (UE).The power level of transmit off power needs to be further discussed.

Proposal 2: Transmit on/off time mask maybe not be necessary for the intermediate node (UE).

Proposal 3: Carrier leakage is needed to define for the intermediate node (UE).

Proposal 4: In-band emission needs to be considered.

Proposal 5: Transmit intermodulation is needed to define for the intermediate node (UE).

Reference

- R4-2417088, WF on impacts of A-IoT  on RF requirements, Huawei, TSG RAN WG4 meeting #112bis