3GPP TSG-RAN WG4 Meeting #113	R4-2418337

# Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on MPR reduction for FR2 CA

Source:	Samsung

Agenda item:	7.1.1.2.3

Document for:	Discussion

1.	Introduction

In last RAN4 meeting, the configuration based MPR reduction for FR2 CA was agreed [1] and draft CR was endorsed [2]. Then the remaining job is to further discuss the activation based MPR reduction.

Issue 3-2-2: CC activation-based MPR improvement - WF - Further discuss activated based MPR improvement. - FFS using common UE capability between configuration based and activation based, or another UE capability for activated-based improvements. - FFS MPR can be determined based on activated UL CCs BW instead of Cumulative aggregated channel bandwidth (CABW). (i.e., activation based also applies to UL CA case) - FFS limitation the enhancement to cases where the activated UL CCs form a contiguous block.

In this contribution, we further discuss the activation based MPR reduction for FR2 CA.

2. 	Discussion

2.1 activation based MPR reduction

According to the WID, the Rel-19 MPR enhancement based on activation is in the scope. Moreover, a UE may support further enhancement based on activation than based on configuration, assuming activation-based UE can fast tune the LO depending on activated CCs but configuration-based UE can only tune the LO depending on the configured CCs regardless of how many CCs are activated.

Proposal 1:	RAN4 to specify activation based MPR reduction besides the configuration based MPR reduction

So, there will be two levels of MPR enhancement for FR2 CA, one is configuration based and the other is activation based. It should be both optional feature and not release independent. New UE capability is needed to address the optional FR2 MPR enhancement feature. In our understanding it is not necessary to specify two IEs for configuration based and activation based respectively. One IE for the FR2 MPR enhancement feature is enough, with two different values for the new IE.

Proposal 2:	A common UE capability (i.e., one IE with two enumerated values) for the FR2 MPR enhancement feature is enough

2.2 MPR applicability when bandwidth basis = 400Mhz or 800MHz

The draft CR was endorsed in last meeting by changing the CABW to bandwidth basis [2], e.g., for PC1 and PC3

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-1 Waveform Type; bandwidth basis3. < 400 MHz; ≥ 400 MHz and < 800 MHz; ≥ 800 MHz and ≤ 1400 MHz; > 1400 MHz and ≤ 2400 MHz. DFT-s-OFDM; Pi/2 BPSK; ≤ 5.51; ≤ 7.7; ≤ 8.2; ≤ 8.7. QPSK; ≤ 6.51; ≤ 8.7; ≤ 9.7; ≤ 9.7. 16 QAM; ≤ 6.5; ≤ 8.7; ≤ 9.2; ≤ 9.7. 64 QAM; ≤ 9.0; ≤ 10.7; ≤ 11.2; ≤ 11.7. 256 QAM2; ≤ 12.5; ≤ 14.2; ≤ 14.7; ≤ 15.7. CP-OFDM; QPSK; ≤ 6.5; ≤ 8.7; ≤ 8.7; ≤ 9.7. 16 QAM; ≤ 6.5; ≤ 8.7; ≤ 8.7; ≤ 9.7. 64 QAM; ≤ 9.0; ≤ 10.7; ≤ 11.2; ≤ 11.7. 256 QAM2; ≤ 12.5; ≤ 14.2; ≤ 14.7; ≤ 15.7. NOTE 1: (Void) NOTE 2: Refer to clause 6.1 for 256 QAM applicability. NOTE 3: If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, [configured UL BWchannel_CA] is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis.. Table 6.2A.2.4-1: Maximum power reduction (MPRC_CA) for UE power class 3 in FR2-1 bandwidth basis2. ≤ 400 MHz; > 400 MHz and < 800 MHz; ≥ 800 MHz and ≤ 1400 MHz; > 1400 MHz and ≤ 2400 MHz. DFT-s-OFDM; Pi/2 BPSK; ≤ 5.01; ≤ 7.7; ≤ 8.2; ≤ 8.7. QPSK; ≤ 5.01; ≤ 7.7; ≤ 8.2; ≤ 9.7. 16 QAM; ≤ 6.5; ≤ 8.7; ≤ 9.3; ≤ 9.7. 64 QAM; ≤ 9.0; ≤ 10.7; ≤ 11.2; ≤ 11.7. CP-OFDM; QPSK; ≤ 5.0; ≤ 7.5; ≤ 8.0; ≤ 9.7. 16 QAM; ≤ 6.5; ≤ 8.7; ≤ 9.2; ≤ 9.7. 64 QAM; ≤ 9.0; ≤ 10.7; ≤ 11.2; ≤ 11.7. NOTE 1: (Void). NOTE 2: If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, [configured UL BWchannel_CA] is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used..

CABW is determined by downlink bandwidth, when changing the MPR dependency from downlink to uplink, it is worth to mention that the uplink CA bandwidth can only support up to 800MHz up to now. It would be helpful in the feature CR to highlight 800MHz upper limit when applying the MPR values in the tables.

Proposal 3:	Highlight 800MHz upper limit for the enhanced MPR feature when applying MPR values in the tables.

It is also noticed that the MPR applicability is ambiguous when the bandwidth basis exactly equals to 400MHz or 800MHz. E.g., for PC1, the MPR value of bandwidth basis = 400MHz would be in the 400~800MHz column, but for PC3, the MPR value of bandwidth basis = 400MHz would be in the <=400MHz column. For bandwidth basis = 800MHz case, the MPR value would be in the 800~1400MHz column.

Compared with FR1 MPR applicability, FR1 MPR is aligned with the CA BW class:

Table 6.2A.2.1-1 of TS38.101-1: Contiguous RB allocation for Power Class 3 (FR1)

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

For FR2, 400MHz and 800MHz are always included in its lower range in FR2 CA BW class table.

Table 5.3A.4-1 of TS38.101-2: CA bandwidth classes (FR2)

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4,5 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| V (Note 4) | 1200 MHz < BWChannel_CA ≤ 1600 MHz | 4 |  |
| W (Note 4) | 1600 MHz < BWChannel_CA ≤ 2000 MHz | 5 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| R | 800 MHz < BWChannel_CA ≤ 1000 MHz | 5 |  |
| S | 1000 MHz < BWChannel_CA ≤ 1200 MHz | 6 |  |
| T | 1200 MHz < BWChannel_CA ≤ 1400 MHz | 7 |  |
| U | 1400 MHz < BWChannel_CA ≤ 1600 MHz | 8 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| R2 | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 2 | 5 |
| R3 | 300 MHz ≤ BWChannel_CA ≤ 600 MHz | 3 |  |
| R4 | 400 MHz ≤ BWChannel_CA ≤ 800 MHz | 4 |  |
| R5 | 500 MHz ≤ BWChannel_CA ≤ 1000 MHz | 5 |  |
| R6 | 600 MHz ≤ BWChannel_CA ≤ 1200 MHz | 6 |  |
| R7 | 700 MHz ≤ BWChannel_CA ≤ 1400 MHz | 7 |  |
| R8 | 800 MHz ≤ BWChannel_CA ≤ 1600 MHz | 8 |  |
| R9 | 900 MHz ≤ BWChannel_CA ≤ 1800 MHz | 9 |  |
| R10 | 1000 MHz ≤ BWChannel_CA ≤ 2000 MHz | 10 |  |
| R11 | 1100 MHz ≤ BWChannel_CA ≤ 2200 MHz | 11 |  |
| R12 | 1200 MHz ≤ BWChannel_CA ≤ 2400 MHz | 12 |  |

Observation 1:	FR1 MPR applicability is aligned with CA BW class but existing FR2 MPR table is not exactly aligned with CA BW class.

To avoid potential ambiguity, it is better to further discuss the MPR applicability when bandwidth basis = 400MHz or 800MHz.

Proposal 4:	RAN4 to further discuss the MPR applicability when bandwidth basis = 400MHz or 800MHz.

3. 	Conclusion

Proposal 1:	RAN4 to specify activation based MPR reduction besides the configuration based MPR reduction

Proposal 2:	A common UE capability (i.e., one IE with two enumerated values) for the FR2 MPR enhancement feature is enough

Proposal 3:	Highlight 800MHz upper limit for the enhanced MPR feature when applying MPR values in the tables.

Observation 1:	FR1 MPR applicability is aligned with CA BW class but existing FR2 MPR table is not exactly aligned with CA BW class.

Proposal 4:	RAN4 to further discuss the MPR applicability when bandwidth basis = 400MHz or 800MHz.

4. 	References

- R4-2417204     “WF on power domain enhancements”, Huawei, HiSilicon

- R4-2417110     “draft CR to TS 38.101-2 for FR2 CC configuration based MPR improvement”, NTT DOCOMO, INC., vivo