# 3GPP TSG-RAN WG4 Meeting #113	R4-2418358

# Orlando, US, 18th – 22nd November, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on power domain enhancements for single carrier

# Agenda item:	7.1.1.2.1

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is about power boosting and/or MPR reduction as follows. In this contribution, we want to share some further views on power domain enhancements for single carrier.

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives

# 2 Discussion

  - 2.1 Approach for scenario 1 and scenario 2

- Issue 1-1-1: Approaches for scenarios 1-1 and 1-2 (see Reference for description of scenarios) - WF - Strive for a unified solution which can accommodate both scenario 1 and scenario 2 under the framework of converting outer RB allocation to inner RB allocation. - It does not preclude separate solutions for each applicable scenario - No relaxation of ACLR/SEM/SE values Issue 1-2-1: Clarification of multi-CC scenario - WF - The approach of converting outer RB allocation to inner RB allocation for MPR reduction should consider multi-carrier spectrum scenario from NW perspective while the UE is configured for single CC operation. Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation - WF - The approach should consider the following aspects - unified solution could be applicable for both scenarios 1 and scenario 2 - unified solution considering the spectrum allocation status of operators, e.g multi-CC case from NW perspective, to better leverage the feature of MPR reduction - Solution could benefit RedCap and non-RedCap with UE CBW less or even identical to BS CBW - Solution could include both symmetric and asymmetric CBW extension - Whether the extended UE CBW could exceed the BS CBW for different scenarios - Signalling overhead is considered for the proposed approach(s)

After discussion in last meeting, we tend to seek a unified approach applicable for both scenarios 1 and scenario 2. The scenarios are listed as follows:

Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator)

Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators)

Scenario 2: Narrower UE channel BW within wider BS bandwidth

In last meeting, the criteria to distinguish scenario 1-1 from scenario 1-2 was given in [2], which is to determine whether  there is sufficient gap between operator holding spectrums. However, from our understanding, the criteria to distinguish scenario 1 and scenario 2 is not clear as scenario 1 with no adjacent in-band/out-of-band co-existence issue includes the case that UE channel BW is narrower than BS bandwidth. Hence, we propose that as long as one of the scenarios is met, the unified solution can be used. Furthermore, taking into account all aspects that have been listed under issue 1-2-2 in the WF, it is proposed to consider the extended UE CBW based approach, and the details are as follows.

- For single carrier operation from NW perspective, if the minimum offset between UE CBW and BS CBW is larger than 1/2 UE CBW, the extended UE CBW based approach can be applied. The extension ratio of extended UE CBW to UE CBW is two, which can convert full RB allocation in UE CBW to new inner RB allocation.

- Figure 2.1: the extended UE CBW based approach for single carrier operation

- For multi-carrier case from NW perspective, when there is no adjacent in-band/out-of-band co-existence issue, and when the minimum offset between UE CBW and BS RF bandwidth is greater than 1/2 UE CBW, the extended UE CBW based approach can be applied, in which, the extended UE CBW could exceed the BS CBW. The extension ratio of extended UE CBW to UE CBW is two, which can convert full RB allocation in UE CBW to new inner RB allocation.

- Figure 2.1: the extended UE CBW based approach for multi-carrier case

Proposal 1: As long as one of the scenarios is met, the extended UE CBW based approach can be applied, and the details are as follows.

- For single carrier operation from NW perspective, if the minimum offset between UE CBW and BS CBW is larger than or equal to 1/2 UE CBW, the extended UE CBW based approach can be applied. The extension ratio of extended UE CBW to UE CBW is two, which can convert full RB allocation in UE CBW to new inner RB allocation.

- For multi-carrier case from NW perspective, when there is no adjacent in-band/out-of-band co-existence issue, and when the minimum offset between UE CBW and BS RF bandwidth is no less than 1/2 UE CBW, the extended UE CBW based approach can be applied, in which, the extended UE CBW could exceed the BS CBW. The extension ratio of extended UE CBW to UE CBW is two, which can convert full RB allocation in UE CBW to new inner RB allocation.

  - 2.2 Consideration on requirements

In-band emissions

- Issue 1-2-3: Where to use IBE in the larger BS CBW - WF - IBE should be used between edges of UE CBW and extended UE CBW - FFS whether IBE could be used in the guard band of UE CBW - FFS the full RB allocation scenario - FFS the impact to testability

According to the following equation of general requirement for in-band emissions, the limit is related to the UE channel bandwidth, allocated RB and the starting frequency offset between the allocated RB and the measured non-allocated RB. Based on the reason for limiting in band emission, the interference from UE that is received with high power at the gNB may drastically reduce the SNR for another UE that is received with weaker signal strength, and the situation improves significantly if some sort of transmit power control in the uplink is used. If IBE is used between edges of UE CBW and extended UE CBW, the general limit equation of IBE should be still based on the UE CBW instead of extended UE CBW since we deploy UE based on the real UE CBW rather than extended UE CBW.

Furthermore, the in-band emission is measured as the ratio of the UE output power in a non-allocated RB to the UE output power in an allocated RB. Given that guard band is not multiples of RB size, we think IBE can not be used in the guard band of UE CBW.

Regarding the general limit formula of IBE, when UL allocation reaches the maximum and corresponds to the transmission bandwidth configuration of UE CBW, the first term achieves an upper limit of -25 dB, which represents a relaxation for wide allocations. From this perspective, when IBE is used between edges of UE CBW and extended UE CBW for full RB allocation, NRB and LCRB all corresponds to the transmission bandwidth configuration of UE CBW, the limit of -25 dB should be used as general IBE requirement between edges of UE CBW and extended UE CBW.

Proposal 2: IBE should be used between edges of UE CBW and extended UE CBW

    - The general limit equation of IBE should be still based on the UE CBW instead of extended UE CBW

    - IBE can not be used in the guard band of UE CBW

    - Extend the IBE limit in the outermost RB of UE CBW to the range between edges of UE CBW and extended UE CBW for non-full RB allocation scenario

    - Use the IBE limit of -25 dB between edges of UE CBW and extended UE CBW for full RB allocation scenario

ACLR, SEM, SE

- Issue 1-2-4: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. - Proposal 2: ACLR, SEM and spurious emissions would be defined based on BS channel bandwidth. - WF - FFS in next meeting - ITU regulation of 250% necessary bandwidth should be considered in further analysis - Issue 1-2-5: Boundary to apply SE - Proposals - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW. - Proposal 2: SE is applied at BS channel bandwidth. - WF - FFS in next meeting - ITU regulation of 250% necessary bandwidth should be considered in further analysis - Issue 1-2-6: Which CBW is utilized as the basis for the OOBE requirement and the applicable boundary - Proposals - Proposal 1: The OOBE requirement and applicable boundary should be based on UE CBW. - Proposal 2: The OOBE requirement and applicable boundary is based on BS CBW. - WF - FFS in next meeting

According to the principle stated in ITU-R SM.329, the spurious domain generally consists of frequencies separated from the centre frequency of the emission by 250% of the necessary bandwidth of the emission, If we take 250%  necessary bandwidth into consideration, comparing the difference of OOB boundary between 3GPP and ITU, there are 1/2 UE CBW and even more OOB boundary can be relaxed. Hence, ACLR and SEM can be applicable from the edge of extended UE CBW and the OOBE requirement and applicable boundary should be based on UE CBW, which represents OOB boundary is relaxed by 1/2 UE CBW.

However, if ACLR and SEM are applied from the edge of BS CBW, for RedCap UE operating with 20MHz channel bandwidth located at the center of 100MHz BS channel bandwidth, ACLR and SEM will apply from the edge of 100MHz BS channel bandwidth, it means spurious emission boundary will relax 40MHz, which will be looser than the requirement specified in ITU.

Proposal 3: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

Proposal 4: The application range of SE should be altered with the shifting of the edge of the UE CBW.

Proposal 5: The OOBE requirement and applicable boundary should be based on UE CBW.

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Proposal 1: As long as one of the scenarios is met, the extended UE CBW based approach can be applied, and the details are as follows.

- For single carrier operation from NW perspective, if the minimum offset between UE CBW and BS CBW is larger than or equal to 1/2 UE CBW, the extended UE CBW based approach can be applied. The extension ratio of extended UE CBW to UE CBW is two, which can convert full RB allocation in UE CBW to new inner RB allocation.

- For multi-carrier case from NW perspective, when there is no adjacent in-band/out-of-band co-existence issue, and when the minimum offset between UE CBW and BS RF bandwidth is no less than 1/2 UE CBW, the extended UE CBW based approach can be applied, in which, the extended UE CBW could exceed the BS CBW. The extension ratio of extended UE CBW to UE CBW is two, which can convert full RB allocation in UE CBW to new inner RB allocation.

Proposal 2: IBE should be used between edges of UE CBW and extended UE CBW

    - The general limit equation of IBE should be still based on the UE CBW instead of extended UE CBW

    - IBE can not be used in the guard band of UE CBW

    - Extend the IBE limit in the outermost RB of UE CBW to the range between edges of UE CBW and extended UE CBW for non-full RB allocation scenario

    - Use the IBE limit of -25 dB between edges of UE CBW and extended UE CBW for full RB allocation scenario

Proposal 3: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

Proposal 4: The application range of SE should be altered with the shifting of the edge of the UE CBW.

Proposal 5: The OOBE requirement and applicable boundary should be based on UE CBW.

# 4 Reference

- RP-230828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei

- R4-2416177, On power domain enhancements for single carrier, Huawei, HiSilicon, China Unicom, Orange