# 3GPP TSG-RAN WG4 Meeting #113	R4-2418359

# Orlando, US, 18th – 22nd November, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on MPR applicability for FR1 intra-band UL CA

# Agenda item:	7.1.1.2.2

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is about power boosting and/or MPR reduction as follows. In this contribution, we want to share some further views on MPR applicability for FR1 intra-band UL CA.

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives

# 2 Discussion

  - 2.1 Intra-band contiguous CA

- Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE - Proposals - Proposal 1: activated CC CBW or aggregated CBW adopted for integral region and boundary of spurious emissions/ACLR/SEM depending on dualPA-Architecture IE indication. - Proposal 2: Emission requirements are based on aggregated channel bandwidth also in case that only one CC is activated. - WF - FFS in next meeting - Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell - Proposals - Proposal 1: CA requirements of ACLR/SEM/SE should be applied for intra-band contiguous CA with single activated cell. - Proposal 2: - For UE supporting contiguous ULCA with the dualPA IE: - The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively. - For UE supporting contiguous ULCA without signalling the dualPA IE or signalling PC1.5 or TxD or UL MIMO support: - The applicable SEM, ACLR and spurious emissions are the configured contiguous ULCA - Proposal 1: After the initial configuration of contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the aggregated CA BW SEM as stated in sub-clause 6.5A.2.2.1 in TS38.101-1. Further discuss SEM mask for single CC fall back only if dualPA-architecture is supported - Proposal 3: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. - WF - FFS in next meeting

The key of the above issues is whether to distinguish integral region, applying boundary of ACLR/SEM and applicable SEM/ACLR/SE requirements depending on the indication of dualPA-Architecture. Based on the observation in [2], for a single LO frequency intra-band ULCA implementation when single CC is activated, any RB allocation < 6.48 MHz bandwidth fails the -13dBm/MHz SEM limit when the single carrier SEM requirement is used, but using the configured contiguous ULCA SEM works. For intra-band ULCA implemented with two LO (dualPA-Architecture indication), SEM/ACLR/SE requirements of single carrier can be applied when single carrier MPR is used.

Thus, for intra-band contiguous CA with single activated cell, it’s no controversy to apply SEM/ACLR/SE requirements of configured contiguous ULCA, and integral region, applying boundary are based on aggregated CBW when dualPA-Architecture is indicated. Furthermore, if dualPA-Architecture is not indicated, there is also no need to use SEM/ACLR/SE requirements of single carrier. As stated in [3], for intra-band contiguous CA with single activated cell, IBE requirement defined in Table 6.4A.2.1.2-1 and 6.4A.2.1.2-2 is applied. According to the definition of IBE for intra-band contiguous CA, it is the interference falling into the non-allocated resource blocks for all component carriers. If SEM/ACLR/SE requirements of single carrier are applied, the applicable range will overlap with IBE applying range. For intra-band contiguous CA with single activated cell, if dualPA-Architecture is not indicated, SEM/ACLR/SE requirements of configured contiguous ULCA should be applied, and integral region, applying boundary are based on aggregated CBW.

Proposal 1: For intra-band contiguous CA with single activated CC, no matter whether dualPA-Architecture is indicated, SEM/ACLR/SE requirements of configured contiguous ULCA should be applied. The integral region and applying boundary are based on aggregated CBW.

  - 2.1 Intra-band non-contiguous CA

- Issue 2-2-1: Applicable MPR for FR1 intra-band non-contiguous UL CA - Proposals Option 1: No spec impact or minimum spec impact with some clarification but no changes of requirements - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required. - Proposal 2: adding 1CC activation as a supplementary scenario in the spec but no changes of the requirements - Proposal 3: single CC MPR applied with single CC emission requirements: Option 2: Single CC MPR/SEM applied on indication of dualPA IE - Proposal 4: To apply non-CA (single CC) MPR based on indication of dualPA IE - Proposal 5: After the initial configuration of non-contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the composite SEM of the 2CCs as stated in sub-clause 6.5A.2.2.2 in TS38.101-1. Further discuss to support the single CC SEM only if dualPA-architecture is supported. Option 3: Possible spec impact on LO retuning - Proposal 6: Discuss impact on requirements if LO needs to be re-configured to support single CC MPR. This discussion would be required for intra-band contiguous and non-contiguous carrier aggregation - WF - FFS on whether for PC2 and PC3, no MPR requirements to be changed. - FFS PC1.5 when NC CA requirements are available - FFS clarification for the applicable emission requirements whether they are single CC based or composite based Agreement in main session - Drop the discussion for LO retuning since it is out of WI scope - FFS for FR2

For PC3 and PC2 intra-band non-contiguous CA, the standard has already specified that for OR (LCRB1 = 0, LCRB2 = 0) case, the corresponding single carrier MPR can be applied. From our understanding, OR (LCRB1 = 0, LCRB2 = 0) case represents only one CC is activated for intra-band non-contiguous CA. Hence, no further changes to the standard are required.

Furthermore, for the applicable emission requirements, the current spec has accounted for the case when UE indicates support for dualPA-Architecture IE, in which, composite spectrum emission mask should be applied. Thus, for PC3 and PC2 intra-band non-contiguous CA with single activated CC, applicable emission requirements are composite based.

Proposal 2: For PC2 and PC3 intra-band non-contiguous CA with single activated CC, no MPR requirements to be changed. Applicable emission requirements are composite based.

- 6.5A.2.2.2 Spectrum emission mask for intra-band non-contiguous CA For intra-band non-contiguous carrier aggregation the spectrum emission mask requirement is defined as a composite spectrum emissions mask. Composite spectrum emission mask applies to frequencies up to ΔfOOB starting from the edges of the sub-blocks. Composite spectrum emission mask is defined as follows a) Composite spectrum emission mask is a combination of individual sub-block spectrum emissions masks b) In case the sub-block consist of one component carrier the sub-lock general spectrum emission mask is defined in subclause 6.5.2.1 c) If for some frequency sub-block spectrum emission masks overlap then spectrum emission mask allowing higher power spectral density applies for that frequency d) If for some frequency a sub-block spectrum emission mask overlaps with the sub-block bandwidth of another sub-block, then the emission mask does not apply for that frequency. For intra-band non-contiguous carrier aggregation, the spectrum emission mask is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Proposal 1: For intra-band contiguous CA with single activated CC, no matter whether dualPA-Architecture is indicated, SEM/ACLR/SE requirements of configured contiguous ULCA should be applied. The integral region and applying boundary are based on aggregated CBW.

Proposal 2: For PC2 and PC3 intra-band non-contiguous CA with single activated CC, no MPR requirements to be changed. Applicable emission requirements are composite based.

# 4 Reference

- RP-230828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei

- R4-2415273, Using single CC MPR and ULCA related emissions when only one CC has RBs allocated, Skyworks Solutions, Inc.

- R4-2415527, ULCA Single CC MPR Applicability after SCELL deactivation, Murata Manufacturing Co. Ltd