# 3GPP TSG-RAN WG4 Meeting #113	R4-2418365

# Orlando, US, 18th – 22nd November, 2024

# Source: 	ZTE Corporation, Sanechips

# Document for:	Approval

# Title: 	Discussion on coexistence study based on ITU regulations for NR NTN Ku band

# Agenda item:	7.9.2

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about NR NTN Ku band from coexistence study perspective.

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

# 2 Discussion

- Sub-topic 2-1: SAN equivalent antenna aperture - Use 9.1 m DL, 7.14 UL (GEO ), 0.91 m DL, 0.714 UL (LEO DL 600 & 1200) - Issue 2-8: Coexistence TN channel model For TN channel model, reuse existing version of TR 38.901. - Sub-topic 2-2: SAN antenna pattern Use section 6.4.1 from 38.811 - Sub-topic 2-3: SAN Tx antenna maximum gain Use 58.5 dBi (GEO), 38.5 dBi (LEO 600 & 1200) - Sub-topic 2-4: SAN Rx antenna maximum gain Use 58.5 dB (GEO), 38.5 dB (LEO 600 & 1200) - Sub-topic 2-5: SAN antenna beamwidth Use 0.177 (GEO), 1.768 (LEO 600 & 1200) - Sub-topic 2-7: SAN max EIRP 43 dBw/MHz (GEO), 13 dBw/MHz (LEO 1200), 7 dBW/MHz (LEO 600) using 100 MHz bandwidth - Sub-topic 2-9: VSAT equivalent antenna aperture Use 60 cm - Sub-topic 2-10: VSAT polarization Assume circular first and if necessary derive the linear result - Sub-topic 2-11: VSAT max Tx antenna gain Use 36.6 dBi (option 1 2W), 36.2 dBi (option 2 10W) - Sub-topic 2-12: VSAT max Rx antenna gain 34.3 dBi - Sub-topic 2-15: VSAT noise figure Type 1 2.5 dB for VSAT LEO and GEO Type 2 6 dB for VSAT to LEO Other options are not precluded - Sub-topic 2-16: VSAT max Tx power 40 dBm (10 W) - Sub-topic 2-21: Channel model 38.811 is the channel model for satellite used for Ka. It can be scaled for Ku. For TN use 38.863 and change the frequency to 11 & 14 GHz - Sub-topic 2-22: TN parameters NTN to NTN won’t be considered. Continue with the TN - NTN coexistence study noting that the applicability of the VSAT ACS simulation result may have to be relaxed for the core requirement derivation. Use the IMT parameters from TR38.922 version 0.3.0 for the IMT study on 15 GHz as a starting point.

The above is initial agreements reached in last meeting [2], based on which, we give the following coexistence simulation assumptions.

  - 2.1 System parameters

    - 2.1.1 TN parameters

According to [3], we can use the following TN parameters for 14 GHz carrier frequency.

Table 2.1.1-1: Simulation parameters of TN for 14 GHz carrier frequency

| Parameters | Urban macro |
| --- | --- |
| Channel bandwidth | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | 100 MHz |
| Scheduled channel bandwidth per UE (UL) | 100 MHz |
| The number of active UE (DL) | Same as the number of BS beam |
| The number of active UE (UL) | 1 UE per slot (first priority) 3 UE per slot (second priority) |
| Traffic model | Full buffer |
| DL power control | NO |
| UL power control | YES |
| BS max TX power in dBm (2 polarizations) | 46dBm |
| UE max TX power in dBm | 23dBm |
| UE min TX power in dBm | -40dBm |
| BS Noise figure in dB | 8 |
| UE Noise figure in dB | 11 |
| Handover margin | 3dB |

According to TR 38.921, TN parameters for 11GHz carrier frequency for co-existence study are given in Table 2.1.1-1.

Table 2.1.1-2: Simulation parameters of TN for 11GHz carrier frequency

| Parameters | Urban macro |
| --- | --- |
| Carrier frequency | 11 GHz |
| Channel bandwidth | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | 98.28 MHz |
| Scheduled channel bandwidth per UE (UL) | 98.28 MHz (1 user, Note 4) 32.76 MHz (3 user) |
| The number of active UE (DL) (Note 1) | 1 |
| The number of active UE (UL) (Note 1) | 1 or 3 |
| Traffic model | Full buffer |
| DL power control | NO |
| UL power control | YES |
| BS max TX power in dBm | 43 (Note 3) |
| UE max TX power in dBm | 23 or 20 (Note 2) |
| UE min TX power in dBm | -33 |
| BS Noise figure in dB | 7 (@10 GHz) |
| UE Noise figure in dB | 9 or 13 (Note 5) |
| Handover margin | 3 dB |
| Note 1 Same as the number of BS beam(s); Note 2: 20 dBm as optional case where CLx-ile should be reduced by 3 dB; Note 3: BS max TX power is defined per polarization; Note 4: 1 user scheduling is a baseline assumption for coexistence evaluation; Note 5: NF of 9 dB is a baseline assumption and NF of 13 dB is as optional case. |  |

Proposal 1: It’s proposed to use simulation parameters of TN in Table 2.1.1-1 and Table 2.1.1-2 for 14GHz and 11GHz carrier frequency respectively.

    - 2.1.2 NTN satellite parameters

Based on the input in [4], the following table is proposed for satellite parameters in last meeting. The parameters highlighted in green were agreed in last meeting. The left parameter G/T also can be agreed.

Table 2.1.2-1: NTN satellite parameters

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 9.1 m | 0.91 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.177 deg | 1.768 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 37 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 7.14 m | 0.714 m | 0.714 m |
| G/T |  | 31.4 dB K-1 | 11.4 dB K-1 | 11.4 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Proposal 2: 31.4 dB/K (GEO) and 11.4 dB/K (LEO 600 & 1200) for satellite G/T can be used.

  - 2.2 Antenna and beamforming pattern modelling

    - 2.2.1	NTN satellite and UE antenna and beam forming pattern modelling

According to [5], Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.811.

Satellite antenna pattern

Parabolic antenna pattern for SAN:

Take the normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ−3dB).

| Equation | Half-power beam-width 2θ−3dB Value | GEO constellation type @ 17GHz | LEO constellation type @ 17GHz |
| --- | --- | --- | --- |
| Fθ=2 J1uu | 2θ−3dB=2×Arcsin1.616×λπD | 0.1763 degrees | 1.7337 degrees |

where:

- Jix  is the Bessel function of first type and itℎ order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)D : Antenna diameter

- : Wavelength

Parabolic antenna pattern for NTN UE:

  - Use the following parabolic antenna model for NTN UE

Fθ=2 J1uu

With:

- Jix  is the Bessel function of first type and itℎ order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Satellite and UE beam forming pattern

The following table is agreed for the beam layout definition for a single satellite simulation.

Table 2.2.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |  |  |
| --- | --- | --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |  |  |
| UV plane illustration (extracted from [19]) |  |  |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |  |  |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) with ABS [degree]=180/pi x ABS[rad] and with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern. |  |  |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: TBD° for GEO and LEO |  |  |
|  | Option 1: FRF=1 | Option 2: FRF=3 | Option 3: FRF=2 |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |  |  |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |  |  |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |  |  |
| UE configuration | Fixed and mobile VSAT for GSO Fixed VSAT for NGSO |  |  |
| UE orientation | Ideal Tracking serving beam |  |  |
| UE attachment | RSRP |  |  |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). |  |  |  |

Proposal 3: To consider NTN satellite and UE antenna and beam forming pattern modelling for Ka band as a starting point for Ku band coexistence study.

    - 2.2.2	TN BS and UE antenna and beam forming pattern modelling

    - 2.2.2.1	TN BS and UE antenna and beam forming pattern modelling for 11GHz carrier frequency

According to TR 38.921 clause 4.2.3, in Table 2.3.2-1, the parameters used by the parameterized array antenna model are described.

Table 2.3.2-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 2.3.2-2.

Table 2.3.2-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=−min−−min12φφ3dB2,Am−min12θ−90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm−1dvλcosθ+n−1dℎλsinθsinφ wm,n=1MNexpj2πm−1dvλsinθetilt−n−1dℎλcosθetiltsinφescan | dBi |

In Table 2.3.2-3, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain determined by calculating the directivity from a given geometry including beam widths. The element directivity can be calculated based on the pattern described by Table 2.3.2-3 in dBi as:

GE,max=DE,max−LE

, where the peak directivity DE,max is calculated from given values on 3dB, 3dB, dh and dv as:

DE,max=10log104πAlinθ,φmax−ππ0πAlinθ,φsinθdθdφ

, where Alin(,) is defined in linear scale as:

Alinθ,φ=10Aθ,φ10

Table 2.3.2-3: BS array antenna parameters

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban | Small cell indoor |
| --- | --- | --- | --- | --- |
| Am (dB) | 30 | 30 | 30 | 30 |
| SLAv (dB) | 30 | 30 | 30 | 30 |
| 3dB (deg.) | 90 | 90 | 90 | 90 |
| 3dB (deg.) | 65 | 90 | 90 | 90 |
| GE,max (dBi) | 6.4 | 5.5 | 5.5 | 5.5 |
| LE (dB) | 2.0 | 2.0 | 2.0 | 2.0 |
| (M, N) | (16, 8) | (16, 8) | (8,8) | (4, 4) |
| Number of supported polarizations, P | 2 | 2 | 2 | 2 |
| dh (m) | 0.5 | 0.5 | 0.5 | 0.5 |
| dv (m) | 0.7 | 0.5 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | +/- 60 | +/- 60 | +/- 60 | N/A |
| Vertical coverage range (deg.) | 90 to 100 | 90 to 120 | 90 to 120 | N/A |
| Conducted power (before ohmic loss) per antenna element, Ptx (dBm) | 22 (Note 4) | 22 (Note 4) | 16 (Note 5) | 9 (Note 6) |
| Mechanical downtilt (deg.) | 6 | 10 | N/A | N/A (Note 7) |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element). Note 5: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 6: The conducted power per element assumes 4x4x2 elements (i.e. power per H/V polarized element). Note 7: Boresight direction is perpendicular to the ceiling. |  |  |  |  |

Based on the array parameters sets fundamental characteristics such as peak EIRP can be derived. For radiated power considerations peak Equivalent Isotropic Radiated Power (EIRP) is calculated in logarithmical scale calculated as:

EIRP=Ptx+GE,max+20log10MN+10log10P

, where P, M, N, Ptx, GE,max is given by Table 2.3.2-3.

According to TR 38.921, UE operating in 10 - 10.5 GHz will most likely therefore have a conducted interface with an assumed isotropic radiation pattern antenna and no beamforming.

Proposal 4: To use TN BS and UE antenna and beam forming pattern modelling in TS 38.921 for 11GHz carrier frequency for Ku band coexistence study.

    - 2.2.2.2	TN BS antenna and beam forming pattern modelling for 14GHz carrier frequency

To model an AAS BS equipped with a sub-array antenna geometry an extended antenna model is required. A sub-array antenna geometry is created by combining vertical elements to sub-arrays as indicated in Figure 6.1.2.3.2-1. The antenna model extension was created to model AAS base station operating within the frequency range 14800 - 15350 MHz required for sharing studies in ITU-R.

Figure 6.1.2.3.2-1: Sub-array structure

In Table 6.1.2.3.2-1, the parameters used by the parameterized array antenna model supporting sub-array geometries are described.

Table 6.1.2.3.2-1: Extended parameter definitions

| Level | Parameter | Symbol | Unit |
| --- | --- | --- | --- |
| Element | Front to back ratio | Am | dB |
|  | Side lobe suppression | SLAv | dB |
|  | Horizontal half power beamwidth | 3dB | Degrees |
|  | Vertical half power beamwidth | 3dB | Degrees |
|  | Array element peak gain | GE,max | dBi |
| Sub-array | Number of element rows in sub-array | Msub | Integer |
|  | Vertical element separation | dv,sub | m |
|  | Electrical pre-set sub-array down-tilt angle | subtilt | Degrees |
| Array | Number of elements/sub-array rows | M | Integer |
|  | Number of elements columns | N | Integer |
|  | Horizontal element separation | dh | m |
|  | Vertical element/sub-array separation | dv | m |
|  | Electrical down-tilt angle | etilt | Degrees |
|  | Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 6.1.2.3.2-2.

Table 6.1.2.3.2-2: Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=−min−−min12φφ3dB2,Am−min12θ−90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm−1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm−1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm−1dvλsinθetilt−n−1dℎλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm−1dvλcosθ+n−1dℎλsinθsinφ |

In Table 6.1.2.3.2-3, representable parameter sets relevant for an AAS base station operating within 14800 - 15350 MHz are provided.

Table 6.1.2.3.2-3: Antenna array parameters

| Parameter | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 5 | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 4) | [8 x 8 / 4 x 4] | 16 x 24 / 8 x 32 | 16 x 24 / 8 x 32 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.5 of wavelength for V | 0.5 of wavelength for H, 2.8/5.6 of wavelength for V (Note 5) | 0.5 of wavelength for H, 2.8/5.6 of wavelength for V (Note 5) |
| Number of element rows in sub-array | N/A | 4/8 | 4/8 |
| Vertical element separation in sub-array (dv,sub) | 0.5 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A | 3 | 3 |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | [2/8] | 17.14/18.9 | 7.1/8.9 |
| Base station horizontal coverage range (degrees) | +/-90 | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 0-180 | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 90 | 6 | 6 |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: The conducted power per sub-array assumes 16 × 24 / 8 x 32 sub-arrays and 2 polarizations for urban macro and dense urban cases; the conducted power per element assumes [8 × 8 / 4 x 4] elements and 2 polarizations for indoor case. Note 4: 16 × 24/32 means there are 16 vertical and 24/32 horizontal radiating sub-arrays for urban macro and dense urban cases; [8 × 8 / 4 x 4] means there are [8 / 4] vertical and [8 / 4] horizontal radiating elements for indoor case. Note 5: For the case of 4/8 elements per sub array, dv will be 2.8/5.6 wavelengths. |  |  |  |

Proposal 5: To use TN BS antenna and beam forming pattern modelling in TS 38.922 for 14GHz carrier frequency for Ku band coexistence study.

  - 2.3 ACIR model

For ACIR model, we suggest to use the same model as Ka band coexistence study for Ku band.

- To use clause 5.1.1.4.1 and 5.1.1.4.2 as in 3GPP TR 36.942 as baseline for the ACLR modelling.

- To use 1 UE for both TN and NTN in the DL

- To use 10 UEs for NTN and 1 UE for TN in the UL

- To use flat ACLR modeling for co-existence study.

Proposal 6: It is suggested to use the following ACIR model for Ku band coexistence study.

- To use clause 5.1.1.4.1 and 5.1.1.4.2 as in 3GPP TR 36.942 as baseline for the ACLR modelling.

- To use 1 UE for both TN and NTN in the DL

- To use 10 UEs for NTN and 1 UE for TN in the UL

- To use flat ACLR modeling for co-existence study.

  - 2.4 Propagation model

The following table can be used for propagation model.

Table 2.4-1: Propagation model for Ku band co-existence study

| Link | Propagation model |
| --- | --- |
| TN BS to Fixed VSAT on roof | Free space path loss |
| TN BS to L-ESIM at 1.5 m | UMa as in 3GPP TR 38.803 |
| TN BS to TN UE | UMa as in 3GPP TR 38.803 |
| TN UE to Fixed VSAT on roof | UMa as in 3GPP TR 38.803 (BS is to be replaced with VSAT) |
| TN UE to L-ESIM | Umi |
| Satellite to TN BS/UE | 3GPP TR 38.821 |
| Satellite to VSAT/ESIM | 3GPP TR 38.821 |
| TN BS to Satellite | 3GPP TR 38.821 |
| Note1: For the propagation models which use the 3GPP TR 38.821 [3], to use same assumptions as in [3] and consider the atmospheric losses and the scintillation losses. Note2: TN BS height is 25m |  |

Proposal 7: It is suggested to use the propagation model in Table 2.4-1 for Ku band coexistence study.

  - 2.5 Transmission power control model

    - 2.5.1 TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

    - 2.5.2 NTN UL TPC

For calibration purpose, reuse the same TN TPC for NTN with SNR target 15dB.

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted but needs to revise CLx-ile to align with UE UL power control parameters used in TR38.821. The CLx-ile value should be adapted for rural scenario.

    - 2.5.3 DL TPC

For downlink scenario, no power control scheme is applied.

Proposal 8: It is proposed to reuse transmission power control model as Ka for Ku band coexistence study.

  - 2.6 Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

Proposal 9: It is proposed to reuse received power control model as Ka for Ku band coexistence study.

  - 2.7 Performance metric

For NR,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

For NTN,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

Proposal 10: It is proposed to reuse the above performance metric for Ku band coexistence study.

  - 2.8 Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics

# 3 Conclusion

In this contribution, we shared some views on coexistence study for NTN Ku band and the proposal is made as follows:

Proposal 1: It’s proposed to use simulation parameters of TN in Table 2.1.1-1 and Table 2.1.1-2 for 14GHz and 11GHz carrier frequency respectively.

Proposal 2: 31.4 dB/K (GEO) and 11.4 dB/K (LEO 600 & 1200) for satellite G/T can be used.

Proposal 3: To consider NTN satellite and UE antenna and beam forming pattern modelling for Ka band as a starting point for Ku band coexistence study.

Proposal 4: To use TN BS and UE antenna and beam forming pattern modelling in TS 38.921 for 11GHz carrier frequency for Ku band coexistence study.

Proposal 5: To use TN BS antenna and beam forming pattern modelling in TS 38.922 for 14GHz carrier frequency for Ku band coexistence study.

Proposal 6: It is suggested to use the following ACIR model for Ku band coexistence study.

- To use clause 5.1.1.4.1 and 5.1.1.4.2 as in 3GPP TR 36.942 as baseline for the ACLR modelling.

- To use 1 UE for both TN and NTN in the DL

- To use 10 UEs for NTN and 1 UE for TN in the UL

- To use flat ACLR modeling for co-existence study.

Proposal 7: It is suggested to use the propagation model in Table 2.4-1 for Ku band coexistence study.

Proposal 8: It is proposed to reuse transmission power control model as Ka for Ku band coexistence study.

Proposal 9: It is proposed to reuse received power control model as Ka for Ku band coexistence study.

Proposal 10: It is proposed to reuse the above performance metric for Ku band coexistence study.

# 4 Reference

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat et al.

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

- TR 38.922

- R4-2416503, On the VSAT and Satellite Parameters for Ku-band Coexistence, THALES

- R4-2309771, Simulation assumptions for NTN co-existence study in bands above 10GHz, Samsung