# 3GPP TSG-RAN WG4 Meeting #113	R4-2418367

# Orlando, US, 18th – 22nd November, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NR NTN Ku band

# Agenda item:	7.9.4

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about NR NTN Ku band from UE RF requirements perspective.

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

# 2 Discussion

  - 2.1 Off-axis EIRP emission density limit within the operating band

- Issue 1-1-3: Regulations to be considered - For the NTN Ku band 10.7 – 12.75 GHz (S-to-E) and 14.0 - 14.5 GHz (E-to-S) for region 1 - Consider the requirements based on ETSI Harmonized Standards EN 301 428, EN 302 448, EN 302 977, EN 303 980, EN 303 981, EN 302 340, EN 301 430 at least - The ECC Decision(18)04, ECC Decision(18)05, ECC Decision(19)04, ECC Decision(05)10, ECC Decision(17)04, ECC Decision(03)04, ECC Decision(06)03, ERC Decision(00)08 will also be considered at least - For the NTN Ku band in other regions: - The requirements based on FCC Title 47 Chapter I Subchapter B Part 25 can also be considered. - Further investigate other regulations to be covered and the relation to the band plan. - Issue 1-2-4: Off-axis EIRP density limits - For the UL band 14.0 - 14.5 GHz (E-to-S) for region 1: - The off-axis EIRP requirements standardized by the ETSI should be considered for Ku-Band VSAT UE. - Fixed VSAT based on EN 301 428 - Mobile VSAT based on EN 302 340, EN 302 977

According to way forward in last meeting, consider the requirements based on ETSI Harmonized Standards and ECC Decisions for region 1. In addition, the requirements based on FCC Title 47 Chapter I Subchapter B Part 25 can also be considered.

EN 301 428 (14.0-14.5 GHz)

EN 302 340, EN 302 977 (14.0-14.5 GHz)

FCC 25.218 (14.0-14.5 GHz)

Comparing off-axis EIRP density limits between ECC Decisions and FCC regulation, it can be observed that not only the range of angles is different, but also the limits are not equivalent. Converting the limits to the same measurement bandwidth, FCC requirement is more stricter than ECC in the plane tangent for the first three ranges but more relaxed for the last range. In the plane perpendicular, FCC requirement is more stricter for the first range.

If we define an unified band definition for all regions, to allow for global circulation of terminals, the more stricter limits between two regulations should be selected, which is more complicated. However, if we define two bands based on referred regulations as we proposed in the following table in [] , we can consider off-axis EIRP density limits for band nX based on EN 301 428,EN 302 340 and EN 302 977, and specify off-axis EIRP density limits for band nY based on FCC 25.218.

Table 2.1: Band definitions for Ku band

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| nX(Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| nY(Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decisions and ETSI Harmonized Standards. NOTE 2: This band is applicable in the countries subject to FCC 47 CFR part 25. |  |  |  |

Proposal 1: Consider off-axis EIRP density limits for band nX based on EN 301 428,EN 302 340 and EN 302 977, and specify off-axis EIRP density limits for band nY based on FCC 25.218.

  - 2.2 On-axis spurious requirement and off-axis spurious requirement

- Issue 1-2-5: On-axis spurious and off-axis spurious requirement - For the UL band 14.0 - 14.5 GHz (E-to-S) for region 1: - Specify the same on-axis spurious limits in “Emissions disabled” and “Carrier-off” states requirement as Ka band n512 - FFS on on-axis spurious limits in “Carrier-on” state - FFS on off-axis spurious limits

For on-axis spurious and off-axis spurious requirement, they are only specified in ECC Decisions, while not defined in FCC 47 CFR part 25. Hence, it only needs to define on-axis spurious and off-axis spurious requirement for band nX.

On-axis spurious requirement in “Carrier-on” state

EN 302 977

EN 303 980, EN 303 981

For on-axis spurious limits in “Carrier-on” state, EN 302 977 also give different requirements for outside and inside with dBW in any 100 kHz for Ku band in UL 14.0–14.5 GHz, which can be converted to the same value with dBm in 1 MHz as specfied in 38.101-5 for Ka band n512. However, EN 303 980, EN 303 981 define the same requirement for both outside and inside as 44-K dBm/MHz. To merge these requirements into one, it’s better to select the most stringent. It is proposed to define the same on-axis spurious limits in “Carrier-on” state for both outside and inside as specified in EN 303 980, EN 303 981.

Observation 1: For on-axis spurious limits in “Carrier-on” state, EN 302 977 gives the same requirement for Ku band in UL 14.0–14.5 GHz as specfied in 38.101-5 for Ka band n512. However, EN 303 980, EN 303 981 define the same requirement for both outside and inside as 44-K dBm/MHz.

Proposal 2: It is proposed to specify the same on-axis spurious limits in “Carrier-on” state for both outside and inside as specified in EN 303 980, EN 303 981.

Off-axis spurious requirement

For radiated field strength limits at 10m from the NTN VSAT, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is same as Ka band in TS 38.101-5.

Observation 2: For radiated field strength limits at 10m from the NTN VSAT, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is same as Ka band in TS 38.101-5.

For off-axis spurious limits in “Emissions disabled” state, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is also same as Ka band in TS 38.101-5.

Observation 3: For off-axis spurious limits in “Emissions disabled” state, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is also same as Ka band in TS 38.101-5.

Proposal 3: For general and off-axis spurious limits in “Emissions disabled” state, it is proposed to specify the same requirement for Ku band in UL 14.0–14.5 GHz as Ka band.

For off-axis spurious limits in “Carrier-on” and “Carrier-off” state, EN 303 980, EN 303 981 and EN 302 977 define a little different requirements for Ku band in UL 14.0–14.5 GHz.

EN 303 980, EN 303 981

EN 302 977

Observation 4: For off-axis spurious limits in “Carrier-on” and “Carrier-off” state, EN 303 980, EN 303 981 and EN 302 977 define a little different requirements for Ku band in UL 14.0–14.5 GHz.

Proposal 4: We need to consider how to specify off-axis spurious limits in “Carrier-on” and “Carrier-off” state for Ku band in UL 14.0–14.5 GHz based on EN 303 980, EN 303 981 and EN 302 977.

  - 2.3 Antenna pointing accuracy

In EN 303 980 and EN 303 981, they only describe the purpose of antenna pointing accuracy without any specification as follows.

Observation 5: In EN 303 980 and EN 303 981, they only describe the purpose of antenna pointing accuracy without any specification.

Proposal 5: We need to consider whether to specify antenna pointing accuracy for Ku band in UL 14.0–14.5 GHz.

  - 2.4 Receiver antenna off-axis performance

For receiver antenna off-axis performance, EN 302 340, EN 303 980, EN 303 981 and EN 302 977 specify different requirements for different VSATs. Receiver antenna off-axis performance in EN 303 977 and EN 302 340 is same as the requirement for Ka band in TS 38.101-5.

EN 303 980

EN 303 981

EN 303 977 and EN 302 340

Observation 6: For receiver antenna off-axis performance, EN 302 340, EN 303 980, EN 303 981 and EN 302 977 specify different requirements for different VSATs. Receiver antenna off-axis performance in EN 303 977 and EN 302 340 is same as the requirement for Ka band in TS 38.101-5.

Proposal 6: We need to consider whether to redefine receiver antenna off-axis performance for Ku band in UL 14.0–14.5 GHz.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: Consider off-axis EIRP density limits for band nX based on EN 301 428,EN 302 340 and EN 302 977, and specify off-axis EIRP density limits for band nY based on FCC 25.218.

Observation 1: For on-axis spurious limits in “Carrier-on” state, EN 302 977 gives the same requirement for Ku band in UL 14.0–14.5 GHz as specfied in 38.101-5 for Ka band n512. However, EN 303 980, EN 303 981 define the same requirement for both outside and inside as 44-K dBm/MHz.

Proposal 2: It is proposed to specify the same on-axis spurious limits in “Carrier-on” state for both outside and inside as specified in EN 303 980, EN 303 981.

Observation 2: For radiated field strength limits at 10m from the NTN VSAT, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is same as Ka band in TS 38.101-5.

Observation 3: For off-axis spurious limits in “Emissions disabled” state, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is also same as Ka band in TS 38.101-5.

Proposal 3: For general and off-axis spurious limits in “Emissions disabled” state, it is proposed to specify the same requirement for Ku band in UL 14.0–14.5 GHz as Ka band.

Observation 4: For off-axis spurious limits in “Carrier-on” and “Carrier-off” state, EN 303 980, EN 303 981 and EN 302 977 define a little different requirements for Ku band in UL 14.0–14.5 GHz.

Proposal 4: We need to consider how to specify off-axis spurious limits in “Carrier-on” and “Carrier-off” state for Ku band in UL 14.0–14.5 GHz based on EN 303 980, EN 303 981 and EN 302 977.

Observation 5: In EN 303 980 and EN 303 981, they only describe the purpose of antenna pointing accuracy without any specification.

Proposal 5: We need to consider whether to specify antenna pointing accuracy for Ku band in UL 14.0–14.5 GHz.

Observation 6: For receiver antenna off-axis performance, EN 302 340, EN 303 980, EN 303 981 and EN 302 977 specify different requirements for different VSATs. Receiver antenna off-axis performance in EN 303 977 and EN 302 340 is same as the requirement for Ka band in TS 38.101-5.

Proposal 6: We need to consider whether to redefine receiver antenna off-axis performance for Ku band in UL 14.0–14.5 GHz.

# 4 Reference

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat et al.

- R4-2315766, NTN enhancements – UE RF requirements, Ericsson

- R4-2402331, NTN Enhancement – VSAT spurious requirement, Ericsson