TSG-RAN Working Group 4 (Radio) meeting #113	R4-2418396

Orlando, US, 18th to 22nd November 2024

Source:	Ericsson

Title:	On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1

Agenda item:	7.11.1

Document for:	Discussion

# Introduction

At the last RAN4 meeting (RAN4#112-bis in Hefei) a discussion triggered by a Liaison Statement (LS) from ETSI TC MSG/TFES [1] on the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in the upper region of FR1. In [2] some early observations on ETSI TFES concern related to current requirement levels. The following discussion was captured in a Way Forward (WF) contribution [3] with guidance for coming meetings. The WF included following:

- Identify operating bands and related network-node specifications to be included in requirement re-evaluation.

- Re-evaluate requirement levels if needed.

- Identify affected requirements, besides the requested TX spurious emissions co-existence requirement.

- Inform ETSI TC MSG/TFES, that RAN4 have received the considerations and have started a discussion on how to improve network-node specifications.

As a acknowledge to ETSI TFES, RAN4 prepared a response LS in [4]. In the LS, RAN4 inform ETSI TFES, that the topic of TX spurious emissions for co-existence requirements limits for upper FR1 bands has been discussed and it was identified as one potential area for further improvements of RAN4 network nodes specifications. Furthermore, RAN4 agreed to further investigate whether other (co-location or co-existence) requirements may require adjustments for upper FR1 bands. RAN4 will keep ETSI TFES informed on future developments in the area, which are expected to be initiated November 2024, with conclusions expected 2025.

In this contribution we continue the discussion on improving requirement levels for BS transmitter spurious emission in upper region of FR1.

# Discussion

In ETSI TC MSG/TFES discussion on Rel-17 versions of Harmonized Standards for base station specifications (i.e. E-UTRA BS, MSR BS, AAS BS, NR BS) is ongoing. During the discussion it was brought to attention that coexistence requirement for transmitter spurious emissions, for 3GPP operating bands introduced in Rel-17/18 versions of 3GPP specifications may require some further checking.

  - Operating band and requirement overview

During the discussion in ETSI TC MSG/TFES it was brought to the attention that some requirement limits for the transmitter spurious emission co-existence requirement (e.g. as captured in TS 38.141-1, Table 6.6.5.5.1.3-1 for BS type 1-C, BS type 1-H and TS 38.141-2, Table 6.7.5.4.5-1 for BS type 1-O) may require further analysis and checking. More specifically, RAN4 need to reassess the relevance of re-using transmitter spurious emissions co-existence limits originally established for bands in the lower region of FR1 for bands in the upper region of FR1. A band survey based on current BS specifications for E-UTRA, MSR and AAS and NR. All band defined within FR1 above 5 GHz is listed in Table 2.1-1.

Table 2.1-1: FR1 band above 5 GHz

| Band number | Uplink (UL) operating band FUL,low – FUL,high (MHz) | Downlink (DL) operating band FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- | --- |
| 46, n46 | 5150 to 5925 | 5150 to 5925 | TDD |
| 47 | 5855 to 5925 | 5855 to 5925 | TDD |
| n96 | 5925 to 7125 | 5925 to 7125 | TDD |
| n102 | 5925 to 6425 | 5925 to 6425 | TDD |
| n104 | 6425 to 7125 | 6425 to 7125 | TDD |

It can be noticed that BS RF core specification for E-UTRA (TS 36.104), MSR (TS 37.104 and TS 37.105) and NR (TS 38.104) will be impacted if requirement levels is updated. The BS specifications with bands defined in the upper region of FR1 will be affected if RAN4 decides to update requirement levels. The BS requirement levels for spurious emissions have been used also for other network nodes, such as IAB, NCR and Repeater.

Observation 1: Requirement levels for bands in the upper region of FR1 require re-evaluation. More specifically, requirement levels should be reevaluated for band 46, n46, 47, n96, n102 and n104.

Observation 2: The following specifications would be affected: TS 36.104, TS 37.104, TS 37.105, TS 38.104, TS 36.141, TS 37.141, TS 37.145-1, TS 37.145-2, TS 38.141-1, TS 38.141-2 and corresponding specifications for IAB, NCR and Repeater.

  - Requirement background

In current BS RF specifications additional spurious emissions requirements have been defined to protect other networks operating in other bands in the same geographical area. These requirements may be applied for the protection of system operating in frequency ranges other than the BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the BS, or they may be set by local or regional regulation as a mandatory requirement for an NR operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM, CDMA, UTRA, E-UTRA, NR, etc.). The spurious emission basic limits are provided in in e.g., TS 38.104, Table 6.6.5.2.3-1 for a BS where requirements for co-existence with the system listed in the first column apply.

The different requirement levels can be traced to different protection situations, as visualised in Figure 2.2-1.

# Figure 2.2-1: Interference situations captured by the requirement

For both cases the requirement level is calculated based on a fixed assumption of Minimum Coupling Loss (MCL) for the following scenarios:

- BS-to-UE: Technical background information is traced back to a RAN4 contribution [5].

- BS-to-BS: Technical background information is traced back to a RAN4 contribution [6].

The MCL is the minimum coupling loss expected for a given scenario. In reality, for a given scenario the coupling loss will statistically have a wide distribution. An alternative approach would be to adopt typical coupling instead of MCL.

It can be noticed that different requirement levels apply to FDD and TDD based on different technical background. For bands in the upper region of FR1, the requirement allocation for FDD and TDD is listed in Table 2.2-1.

Table 2.2-1: Requirement allocation

| Duplex method | BS-to-BS (dBm/MHz) | BS-to-UE (dBm/MHz) |
| --- | --- | --- |
| FDD | -49 | -52 |
| TDD | Not defined | -52 |

The background for the requirement derivation is captured in section 2.2.1 for BS-to-UE and section 2.2.2 for BS-to-BS.

From the BS RF core specifications, the requirement levels for defined bands within FR1 above 5 GHz is listed in Table 2.2-2.

Table 2.2-2: Transmitter co-existence spurious emissions requirement levels applicable for bands above 5 GHz

| Band | Frequency range (MHz) | Requirement level (dBm/MHz) |
| --- | --- | --- |
| 46, n46 | 5150 to 5925 | -52 |
| 47 | 5855 to 5925 | -52 |
| n96 | 5925 to 7125 | -52 |
| n102 | 5925 to 6425 | -52 |
| n104 | 6425 to 7125 | -52 |

From the specifications it can be noticed that requirement levels have been re-used within the whole frequency range FR1. Depending on duplex method (FDD or TDD) and protected system, the requirement levels may differ, as indicated in Table 2.2-1.

In current specifications the tables used for transmitter coexistence spurious emission are very large. An alternative approach on how to write the specification without duplicating the requirement level for each band would be to have common level where applicable. For some bands, different requirement levels may be needed. Those special cases could be handled in a table. Improvement of the specification structure have been suggested part of BS RF evolution WI in Rel-19.

    - Protection of UE receiver (BS-to-UE)

For this case the transmitter co-existence spurious emission requirement for BS is defined to protect the downlink of another network in the same geographical area. According to TR 25.942, subclause 5.1.4.1, the coupling loss between BS and UE was set to:

- MCL equal to 70 dB for the Macro-cellular environment,

- MCL equal to 53 dB for the Microcell environment.

For the requirement background, the MCL for Microcell environment was adopted for requirement level derivation. The Microcell MCL was selected since it was seen as the worst-case MCL [5].

At the time the concept of BS classes was not established. Now when the specification has different requirements established for Wide Area BS intended for Macro environments and Medium Range BS intended for Micro cell environments it would be obvious to differentiate the requirement levels per BS class.

The assumed values are listed in Table 2.2.1-1.

Table 2.2.1-1: Original assumptions

| Parameter | Variable | Value |
| --- | --- | --- |
| UE receiver noise figure | F | 9 dB |
| BS-to-UE coupling loss | L | 53 dB |

The current requirement level is derived based on a link budget calculation in logarithmical scale:

Pem=kT+B+F+L

, where kT is equal to -174 dBm/Hz and B is equal to 1 MHz. Assuming following values for the UE noise figure, F equal to 9 dB and L equal to 53 dB, the maximum allowed spurious emission level, Pem at the BS transmitter can be calculated to -52 dBm/MHz.

It can be noticed that the UE noise figure assumed for 2 GHz is not relevant for frequencies in the upper region of FR1.

Observation 3: It can be observed that MCL equal to 53 dB relevant for Microcell environment is used for requirement derivation of requirements for Wide Area BS.

    - Protection of BS receiver (BS-to-BS)

The reason for the transmitter co-existence spurious emission requirement for BS is to protect the uplink BS receives in another network in the same geographical area.

Traditionally the requirement level was determined by a static link budget calculation based on assumptions of BS noise figure and BS-to-BS coupling loss. For the BS-to-BS case the requirement levels were derived based on similar background as for co-location requirement level [6]. The assumed values are listed in Table 2.2.2-1.

Table 2.2.2-1: Original assumptions

| Parameter | Variable | Value | Note |
| --- | --- | --- | --- |
| BS receiver noise figure | F | 7 dB |  |
| BS-to-BS coupling loss | L | 67 dB | The current requirement level is derived based on an assumption of MCL between two base stations operating in different networks in the same geographical area of 67 dB (see TR 25.942, subclause 10.2.1). |
| Protection criteria | M | 9 dB | Assumed value corresponds to 0.5 dB degradation of victim receiver sensitivity. |

The current requirement level is derived based in logarithmical scale as:

Pem=kT+B+F+L-M

, where kT is equal to -174 dBm/Hz, B is equal to 1 MHz and parameter values in Table 2.2.2-1. With the assumption the maximum allowed spurious emission level in another band is calculated to -49 dBm/MHz.

It can be observed that the requirement level for BS-to-BS co-existence spurious emission is based on the same link budget as co-location spurious emission but with a different assumption on coupling.

    - Approaches for re-evaluation of coupling loss

In TR 25.942, subclause 4.1.1.2 detailed assumptions related to the derivation of BS-to-UE MCL and BS-to-BS MCL used for current requirement levels can be found. These assumptions are based on BS with passive antennas operating around 2 GHz. When MCL values for requirement derivation was derived free-space path loss was assumed.

To re-evaluate co-existence requirement levels with the intention to find relevant requirement levels a study will be required. For a successful co-existence evaluation in RAN4 relevant parameter assumption for the BS antenna and the UE antenna, Propagation model assumption (including BS-to-UE propagation, BS-to-BS propagation) and deployment scenario should be established.

For band n104 in the upper region of FR1, AAS BS will be very common for Macro deployment scenarios. With the AAS BS, the BS antenna radiation pattern will adopt dynamically to achieve beamforming towards scheduled UEs. This means that the BS-to-UE coupling, and BS-to-BS coupling will have different statistical characterises compared to the case where passive antennas is used. Also, at higher frequencies the trend is that the AAS BS antenna gain is higher while the propagation condition is different compared to what originally was assumed at 2 GHz. Based on an analysis of the coupling characteristics, the coupling suitable for requirement derivation can be established.

When the coupling for BS-to-UE was established, the worst case was based on the Micro network deployment scenario. Now with BS classes introduced in specifications it is not obvious that Micro network MCL is a relevant assumption for Wide Area BS intended for Macro deployment.

Observation 4: Fundamental aspects related to the technical background for coexistence spurious emission requirement derivation could possibly need a re-visit for bands in the upper region of FR1.

When requirements for adjacent channel coexistence (ACLR and ACS) is defined for BS and UE, RAN4 have adopted a methodology based on statistics and an acceptable level of degradation of the victim system. A similar approach could also be used for derivation of coexistence spurious emission requirement level, as an alternative approach to be adopted for bands in the upper region of FR1.

When the coupling loss was originally evaluated for BS-to-BS and BS-to-UE passive antennas operating at 2 GHz with 17 dBi antenna gain was considered. With the introduction of AAS BS in frequencies up to 7125 MHz the impact of beams pointing towards the UE and considerably larger antenna gain, sub-array antenna structures in addition to propagation conditions should possibly be considered in the coupling loss evaluation.

Due to extent of such work, the work should take place over a number of meetings within the context of a RAN4 project unless other simpler methods are presented and agreed. Thus, a way forward would be to include such re-evaluation in the ongoing Rel-19 BS RF evolution (NR_BS_RF_req_evo-Core) Work Item (WI) rather than being handled as regular maintenance or a separate new SI/WI.

To include co-existence into the scope of the BS RF evolution WI, a revision of the Work Item Description (WID) at the RAN plenary meeting in December (RAN#107 in Madrid) would be required.

# Conclusion

In this contribution we have identified bands in the upper region of FR1 to further include in a RAN4 requirement re-evaluation work package. The current requirement is used for E-UTRA, MSR, AAS and NR and corresponding specification for other network nodes (IAB, NCR and Repeater).

Analysing the technical background for the current requirement levels, it is clear that the requirement level need to be re-evaluated for relevant assumptions for deployment scenario, BS type, operating frequency, etc.

In addition, we also made following observations:

Observation 1: Requirement levels for bands in the upper region of FR1 require re-evaluation. More specifically, requirement levels should be reevaluated for band 46, n46, 47, n96, n102 and n104.

Observation 2: The following specifications would be affected: TS 36.104, TS 37.104, TS 37.105, TS 38.104, TS 36.141, TS 37.141, TS 37.145-1, TS 37.145-2, TS 38.141-1, TS 38.141-2 and corresponding specifications for IAB, NCR and Repeater.

Observation 3: It can be observed that MCL equal to 53 dB relevant for Microcell environment is used for requirement derivation of requirements for Wide Area BS.

Observation 4: Fundamental aspects related to the technical background for coexistence spurious emission requirement derivation could possibly need a re-visit for bands in the upper region of FR1.

# References

[1]	R4-2414917, “Transmitter spurious emissions: co-existence requirements limits for high frequency bands”, ETSI TFES

[2] 	R4-2415973, “On the topic BS transmitter co-existence spurious emissions requirement limits for bands in upper part of FR1”, Ericsson

[3]	R4-2416582, “Way Forward on coexistence levels for bands in upper region of FR1”, Ericsson

[4]	R4-2416588, “LS to ETSI MSG TFES on Tx spurious emissions: co-existence requirements limits for high frequency bands”, Huawei

[5]	R4-99a01, “BS TDD Spurious Emission Requirements for Co-Existence UTRA-FDD/ UTRA-TDD”, SIEMENS

[6]	R4-030641, “General corrections on co-existence and co-location requirements for UTRA-FDD BS”, Nortel Networks, Ericsson, Siemens, Motorola

[7]	TR 25.942, “Radio Frequency (RF) system scenarios”, v18.0.0