TSG-RAN Working Group 4 (Radio) meeting #113	R4-2418397

Orlando, US, 18th to 22nd November 2024

Source:	Ericsson

Title:	TP to TR 38.922: Addition of MIMO modelling simulation results in subclause 7.3

Agenda item:	7.2.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#112-bis in Hefei) technical information relevant for future IMT systems was extensively discussed. At this stage in the study information have been provided to ITU-R WP 5D for the frequency range 4400 to 4800 MHz in [1] and the frequency range 7125 to 8400 MHz in [2]. A third LS to ITU-R WP 5D with information relevant for 14800 to 15350 MHz and additional information is planned to be finalized at this meeting.

At previous meeting simulations results was presented in [4]. Simulation assumption and simulation results have been captured in a text proposal to TR 38.922 is attached at the end of this contribution.

# Discussion

In the LS received by RAN4, ITU-R WP 5D [3] is asking for additional information on specific topics:

- The particular type of AAS implementation and associated model for IMT base stations, i.e., AAS composed of an array-of-sub-arrays or AAS composed of single-element arrays with its technical justification.

- If providing specific technical AAS implementation parameters for the array-of-sub-arrays case, WP 5D would welcome:

- The pre-set sub array tilt is understood as a parameter (a pre-tilt within the elements combined to form a sub array) that is used to focus the main beam below the boresight where a base station is intended to serve its UEs, similarly to the mechanical down-tilt. Noticing that the mechanical down-tilt of the BS antenna is environment (urban/suburban/rural) dependent, WP 5D welcomes further confirmation whether the preset sub-array down-tilt varies with the environment or not, what would be the range of values for this parameter and the associated environments and if multiple set of configurable preset sub-array down-tilt per BS is foreseen in the future.

- Information of advancements in MIMO architectures that could be applicable for future systems.

- Whether AAS at IMT user equipment is likely or not. If AAS at IMT user equipment is considered as a possibility WP 5D welcomes information relating to the structure of the AAS at the IMT user equipment, associated frequency bands, and the associated parameters of the AAS.

- IMT base station (inclusive of its AAS-related parameters) and IMT user equipment technical parameters for analysing adjacent band compatibility of terrestrial IMT systems with other co-primary services in the ITU Radio Regulations.

WP 5D also seeks information about how the AAS performs in adjacent bands i.e., does its performance drops down to that of a single element or still some array features remain. If the latter is true how far in frequency should one assume the continuity of array features even with some degradation?

- Considering the system-level simulation work typically conducted in WP 5D for sharing and compatibility studies using 3D modelling of antenna pattern, WP 5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern is welcomed.

Answers to questions (a), (b) and (c) will be provided per frequency range. In this contribution we will provide technical input relevant for developing answers to question (e).

In addition to parameter assumptions ITU-R WP 5D have also requested information related to some additional topics. In this contribution we will provide information related to modelling of AAS BS beamforming including simulation results comparing different beamforming approaches.

This contribution is based on revised information presented in an earlier contribution [4].

Technical background information relevant for MIMO beamforming modelling is captured as a text proposal to TR 38.922, subclause 7.3 [5] at the end of this contribution.

# Conclusion

Based on our updated simulation results presented in a previous contribution [4], we have corroborated that there is no need to change the AAS BS beamforming assumptions in 3GPP RAN4 and ITU-R WP 5D. The differences introduced by ZF beamforming technique, compared to the traditional array antenna model, are minor. The simulation results show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire angular sphere around the antenna. Therefore, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model.

Based on the simulation results we observe following:

Observation 1: Based on our simulation results, there is no need to change the beamforming model assumptions in RAN4 TRs and Recommendation ITU-R M.2101. The differences introduced by ZF beamforming technique, compared to the model currently used, are minor. The simulation results show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire sphere around the antenna.

To progress the work, propose following:

Proposal 1: Based on outcome of beamforming modelling evaluation, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model.

Proposal 2: Capture simulation assumption, simulation results and corresponding conclusion in TR 38.922.

Proposal 3: In the LS response to ITU-R WP 5D answer that the current array antenna model approach remains effective for its intended purpose and include a reference to TR 38.922 for proper reference to technical background information.

A text proposal for TR 38.922 with technical background information is attached at the end on this contribution and it is presented for approval.

# References

[1]	R4-2410576, “LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, RAN4

[2]	R4-2414449, “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, RAN4

[3]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, ITU-R WP 5D

[4] 	R4-2415970, “TP to TR 38.922: Addition of new subclause for MIMO modelling aspects in subclause 7.3”, Ericsson

[5]	R4-2417200, “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz;”, V0.3.0, Ericsson

TEXT PROPOSAL:

## 7.3	MIMO modelling

### 7.3.1	Simulation methodologies

##### 7.3.1.1	Methodology 1

The following simulation methodology was adopted to produce simulation results:

- For the k-th UE, generate array coefficients, Ukn,m, to reflect the UE angles from the BS antenna array in both azimuth and elevation (NOTE 1): 

Ukn,m=expj2πn-1dvλcosθk+m-1dhλsinθksinφk

where, for the k-th UE, θk and φk are the angles of elevation and azimuth, respectively. The ranges of θk and φk are 0,π and [-π,π] radians. The size of the matrix Ukn,m is N×M, where N is the number of rows and M is the number of columns of the antenna array.

- Assess the angular separation between the K UEs in the cell (NOTE 2).

- For each UE (NOTE 3), if there is at least another UE which with the minimum angular separation requirement is fulfilled, create a concatenated K×MN channel matrix, Hk (NOTE 4):

Hk=hkT…hpTT

where T is the transpose operator and hk corresponds to the vectorization of Uk resulting in a vector of size MN×1 (NOTE 5).

- For each UE, the zero-forcing beamforming (precoding) matrix Wk (MNxK) is:

Wk=HkHHkHkH-1

where Hk represents the channel matrix with its number of columns corresponding to the number of RX antennas (i.e., UEs) and its number of rows corresponding to the number of TX antennas (BS antenna ports), and H is the conjugate transpose operator.

- The entire Wk matrix can be normalized by diving it by its Frobenius norm, or, doing a column wise normalization by diving each column with its norm (NOTE 6).

- The actual precoding weights for each UE (wk) are obtained by selecting the first column of Wk. The size of wk is MN×1.

- Obtain the individual Zero Forcing beam pattern: The precoding weights for each beam wk of size MNx1 may be used to obtain the k-th zero forcing beam pattern by reordering the column vector wk of size MNx1 into a matrix Wk of size NxM which can be used as element weight in:

Akθ,φ=AEθ,φ+10log10m=1Mn=1Nwkn,m∙vkn,m2

where wk refers to the entry in the n-th row and m-th column of weighting matrix Wk and νn,m is the superposition vector. Note that for AAS with sub-arrays, AE(θ,ϕ) is updated by Asub(θ,ϕ):

Akθ,φ=Asubθ,φ+10log10m=1Mn=1Nwkn,m∙vkn,m2

where Asub(θ,ϕ) calculation follows the method defined in the extended AAS modelling supporting vertical sub-array configurations.

- The resulting composite ZF beam pattern may be obtained by summing up the individual Zero Forcing beam patterns in linear scale (and transforming them back to dB):

AZFθ,φ=10log10k=1K10Akθ,φ10

where AZF(θ,ϕ) refers to the composite ZF beam pattern and Ak(θ,ϕ) refers to the k-th individual zero forcing beam pattern.

NOTE 1: For the computation of Uk, θk and φk are in Local Coordinate System (LCS) which accounts for the mechanical down-tilt.

NOTE 2: The total 3D angular separation should be considered to determine the fulfillment of the minimum angular separation requirement. Additionally, only the azimuth angular separation may be considered in cases where the elevation angle does not significantly contribute to the total 3D angular separation.

NOTE 3: Hk must be constructed for each UE separately, since in some instances, one UE can be paired with all other UEs while another UE could be paired with only a single other UE. The next figure depicts this situation: UE1 can be paired with UE2 and UE3 since the min. angular separation is fulfilled (as shown in (a)), UE2 can only be paired with UE1 (as shown in (b)), and UE3 can only be paired with UE1 (as shown in (c)).

NOTE 4: Hk only includes the UEs that fulfill the minimum angular separation requirement depending on the size of the BS antenna, so Hk may have two or three columns (K=2 or 3) in each Monte Carlo snapshot depending on the geometry of the BS and the UEs. For the UEs that cannot be paired with any other UEs (K=1), Hk is not constructed, and the current beamforming model is used.

NOTE 5: The vectorization must ensure that the vector of size 1×MN correspond to the concatenation of the rows of Uk:

u11⋯u1m⋮⋱⋮un1⋯unm→concatenation→u11…u1mu21…u2m…un1…unm

NOTE 6: The total power constraint simplifies the design problem and leads to simple and efficient precoders. However, the magnitude of some elements of wk may be larger than 1/(MN), which can introduce per-antenna port transmit power larger than 1/(MN). In practice, many systems are subject to individual per-antenna port power constraints.

The parameters values listed in Table 7.3.1.1-1 and 7.3.1.1-2 is used for the simulation.

Table 7.3.1.1-1: IMT deployment-related parameters

| Parameter | Macro Urban |
| --- | --- |
| Base station |  |
| Carrier frequency | 6 GHz |
| Channel bandwidth | 100 MHz |
| BS antenna height | 18 m |
| Cell size | 300 m |
| Sectorization | 1 sector 1 |
| Frequency reuse | 1 |
| User Equipment |  |
| UE height | 1.5 m |
| UE density for terminal that are transmitting simultaneously | 3 UEs per sector |
| NOTE 1: This study considers only a single-entry scenario. |  |

Table 7.3.1.1-2: IMT base station beamforming antenna characteristics for IMT

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to Recommendation ITU-R M.2101 Annex 1 |
| Element gain (incl. Ohmic loss) (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-120 1 |
| Mechanical down-tilt (degrees) | 10 |
| NOTE 1: The vertical coverage range includes the mechanical down-tilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

For the sake of completeness, additional results are provided with an AAS with a subarray configuration. The parameters for this AAS are taken from the WRC-23 studies 5D/1235 and 5D/1461 and are presented in the Table 7.3.1.1-3.

Table 7.3.1.1-3: IMT base station beamforming antenna characteristics for IMT with subarray configuration

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to the extended AAS model in 5D/716 (Annex 4.4 - WRC23) |
| Element gain (incl. Ohmic loss) (dBi) | 6.4 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 16 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 1.4 of wavelength for V |
| Number of element rows in sub-array | 2 |
| Vertical radiating element spacing in sub-array | 0.7 of wavelength |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-100 1 |
| Mechanical down-tilt (degrees) | 6 |
| NOTE 1: The vertical coverage range includes the mechanical down-tilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

##### 7.3.1.2	Methodology 2

{Editor’s note: Input from other company}

### 7.3.2	Simulation results

##### 7.3.2.1	Methodology 1

The IMT network consists of a single site with a single sector. In each snapshot of the Monte Carlo simulation, 3 UEs are uniformly distributed within the sector, and the BS serves each UE with a dedicated beam. These beams can be generated by both the traditional antenna model and as described in subclause 7.3.1.1, assuming a UE minimum angular separation of 10 degrees.

To assess the interference to possible interfered-with receivers, two scenarios are considered, where these receivers are represented by sample points:

- Equidistant sample points towards the horizon around the IMT BS (Figure 7.3.2.1-1a)

- Equidistant sample points on a hemisphere above the IMT BS excluding the sample points towards the horizon (Figure 7.3.2.1-1b)

It is noted that the angular separation between sample points is approximately 15.5 degrees, with scenario 1 having 23 sample points and scenario 2 having 62 sample points.

# Figure 7.3.2.1-1: Equidistant sample points (a) towards the horizon around the BS, and (b) on a hemisphere above the BS

In this section the Monte Carlo simulations results are presented. For each sample point in the considered scenarios, 10.000 snapshots are simulated. The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the sample points of the different scenarios are shown in Figures 7.3.2.1-2 to 7.3.2.1-5. The CDF results for scenario 1 using a BS without and with subarrays are shown in Figures 7.3.2.1-2 and 7.3.2.1-3 respectively. The CDF results for scenario 2 using a BS without and with subarrays are shown in Figures 7.3.2.1-4 and 7.3.2.1-5 respectively. For instance, since scenario 1 has 23 sample points, Figures 7.3.2.1-2a and 7.3.2.1-3a show 23 pairs of CDFs.

# Figure 7.3.2.1-2: Scenario 1 (without subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 7.3.2.1-3: Scenario 1 (with subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 7.3.2.1-4: Scenario 2 (without subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 7.3.2.1-5: Scenario 2 (with subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

To assess the impact on long- and short-term statistics, the 80th percentile and 99.9th percentile are considered respectively. For Figures 7.3.2.1-2a, 7.3.2.1-3a, 7.3.2.1-4a, and 7.3.2.1-5a, Table 7.3.2.1-1 contains the summary of the average BS antenna gain difference between traditional beamforming model and ZF beamforming model.

Table 7.3.2.1-1: Average BS antenna gain difference between traditional beamforming model and ZF beamforming

| Scenario | Long-term (80th percentile) | Short-term (99.9th percentile) |
| --- | --- | --- |
| Scenario 1 (without subarrays) | 0.39 dB | 0.14 dB |
| Scenario 1 (with subarrays) | 0.05 dB | ~0 dB |
| Scenario 2 (without subarrays) | 0.48 dB | 0.06 dB |
| Scenario 2 (with subarrays) | 0.08 dB | ~0 dB |

Furthermore, the BS antenna gain integrated over parts of or the entire angular sphere around the antenna, i.e., the Total Integrated Gain (TIG), is also evaluated to provide a set of more consistent results by considering all angular directions. The TIG is calculated as follows:

TIG≈π2NθNφφ=0Nφ-1θ=0Nθ-1Gθ,φsinθ 		(Eq.7.3.2.1-1)

where θ and φ represent the elevation and azimuth angles, respectively, Nθ and Nφ are the number of samples on elevation and azimuth, respectively, and G is the antenna gain in linear units at the spherical coordinates (θ, φ).

For this assessment, the BS antenna gain is integrated over the whole sphere and over only the upper hemisphere by limiting the range of θ. The integrated gain is calculated for each individual beam for 10000 snapshots, resulting in three values per snapshot. Note that the integrated gain of the upper hemisphere of the AAS, where θ = 90 degrees is the AAS boresight direction including down-tilt, is not exactly the integrated above the horizon but includes most of it. This is depicted in Figure 7.3.2.1-6.

# Figure 7.3.2.1-6: Upper hemisphere: (a) upper hemisphere above horizon, and (b) upper hemisphere considered in this study

Figures 7.3.2.1-7and Figure 7.3.2.1-8 show the BS antenna gain integrated over the whole sphere and over only the upper hemisphere (Figure 7.3.2.1-6b), including the horizon, for the BS without and with subarrays, respectively. For simplicity, it is noted that for all the presented results, the integrated gain is always normalized by π2NθNφ, as indicated in Eq.7.3.2.1-1, regardless of the number of samples considered.

# Figure 7.3.2.1-7: CDFs of the BS antenna gain without subarrays, integrated over (a) the upper hemisphere, including the horizon, and (b) the entire angular sphere around the antenna

# Figure 7.3.2.1-8: CDFs of the BS antenna gain with subarrays, integrated over (a) the upper hemisphere, including the horizon, and (b) the entire angular sphere around the antenna

As can be seen, in all cases considered, the TIG difference between the beamforming schemes is negligible.

##### 7.3.1.2	Methodology 2

{Editor’s note: Input from other company}

### 7.3.3	Summary and conclusion

Based the collection of simulation results, RAN4 have corroborated that there is no need to change the beamforming assumptions for the antenna model used in RAN4 and ITU-R WP 5D. The differences introduced by ZF beamforming technique, compared to the traditional array antenna model, are minor. The simulation results show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire angular sphere around the antenna. Therefore, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model.