3GPP TSG-RAN WG4 Meeting#113		R4-2418403

Orlando, United States, Nov 18th -22nd, 2024

Agenda item:	7.21.2.2

Source: 	Spreadtrum Communications

Title: 	Discussion on co-existence evaluation results for Ambient IoT and NR

Document for:	Discussion

Introduction

In RAN4#112bis meeting, co-existence formal simulation assumptions and results were reached and a WF was approved [1]. In this paper, we present updated simulation results and provide some views about the co-existence aspects.

Discussion

Deployment scenario and simulation cases

We have simulated below cases as follow.

| Deployment scenario No. (Case No.) | Topology | spectrum |
| --- | --- | --- |
| 1-1(a/b/c/d) | D1T1-A2- NR UE only outdoor | R2D: DL |
|  |  | CW2D and D2R: DL |
| 1-2(a/b/c/d) | D1T1-A2- NR UE indoor | R2D: DL |
|  |  | CW2D and D2R: DL |
| 2-1(e/f/c/d) | D1T1-B- NR UE only outdoor | R2D: DL |
|  |  | CW2D and D2R: UL |
| 2-2(e/f/c/d) | D1T1-B- NR UE indoor | R2D: DL |
|  |  | CW2D and D2R: UL |

Simulation results

Co-existence evaluation results

Agreement: - Consider 3 SINR values for D2R and R2D respectively and derive the outage percentage value/range. - The 3 values can be selected according to following steps: - Select one set SNR/CNR values @ 10% BLER from companies based on RAN1 LLS (refer to latest RAN1 LLS summary) - Set the SNR/CNR range by removing the highest and lowest value from the set in step 1 - Pick 3 values from the range in step 2, e.g. highest, lowest, middle Note: R2D CNR needs to be transformed to SINR

The values of SINR for D2R and R2D to be used in RAN4 are highlighted in green as following.

|  | D1T1 D2R |
| --- | --- |
| min | -7.8 |
| max | 0.23 |
| average | -4.3 |

|  | TDL-A device1 CNR | R2D SNR for device 1 |
| --- | --- | --- |
| min | 2.6 | -14.8 |
| max | 26.7 | 9.3 |
| average | 10.9 | -6.5 |

According to the simulation cases and the values of SINR for performance evaluation, our simulation results and conclusion as follow.

Table1: simulation results and conclusions

| Deployment scenario No | Case No | SINR degradation@5% in dB | SINR degradation@50% in dB | Throughput degradation @5% | Throughput degradation @5% | Outage percentage consider SINR level with [10%] BLER | Tentative Conclusion for co-existence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | a | 0 | 0 | 0 | 0 | / | No SINR and throughput degradation |
| 1-1 | b | 0 | 0 | / | / | Min SINR: 7% Max SINR: 34% Average SINR: 14% | There is no SINR degradation for inter-system. For intra-system, outage percentage is large than 5%. |
| 1-1 | c | 0.0091 | 0.0048 | 0.0007 | 0.0008 | / | SINR and throughput degradation can be negligible |
| 1-1 | d | 0.0327 | 0.0695 | / | / | Min SINR: 0% Max SINR: 49% Average SINR: 0% | SINR degradation is less than 1dB, inter-system interference is negligible. For intra-system, outage percentage is 0% when use min and average SINR. Outage percentage is 49% when use max SINR. |
| 1-2(10% indoor UE) | a | 0 | 0 | 0 | 0 | / | No SINR and throughput degradation. |
| 1-2(10% indoor UE) | b | 0 | 0 | / | / | Min SINR: 6% Max SINR: 37% Average SINR: 15% | There is no SINR degradation for inter-system. For intra-system, outage percentage is larger than 5%. |
| 1-2(10% indoor UE) | c | 0.3891 | 0.126 | 0.0265 | 0.02 | / | SINR degradation is less than 1dB. Throughput degradation is 2.65% for cell edge and 2% for average throughput. |
| 1-2(10% indoor UE) | d | 0.0304 | 0.0709 | / | / | Min SINR: 0% Max SINR: 48% Average SINR: 0% | SINR degradation is less than 1dB. For inter-system. For intra-system interference, outage percentage is 0% when SINR is min value. Outage percentage is 0% when SINR is average value. Outage percentage is 48% when SINR is max value. |
| 1-2(100% indoor UE) | a | 0.016 | 0.01 | 0 | 0.0013 | / | SINR degradation is less than 1dB, throughput degradation can be negligible. |
| 1-2(100% indoor UE) | b | 0 | 0 | / | / | Min SINR: 5% Max SINR: 33% Average SINR: 17% | No SINR degradation for inter-system. For intra-system interference, outage percentage is larger than 5%. |
| 1-2(100% indoor UE) | c | 21.4071 | 15.4807 | 0 | 0.8209 | / | SINR degradation is larger than 1dB.Throughput degradation is 82.9%. |
| 1-2(100% indoor UE) | d | 0.0355 | 0.0731 | / | / | Min SINR: 0% Max SINR: 48% Average SINR: 0% | SINR degradation is less than 1dB for inter-system. For intra-system interference, outage percentage is 0% when SINR are min value and average value. Outage percentage is 48% when SINR is max value. |
| 2-1 | e | 0 | 0 | 0 | 0 | / | No SINR and throughput degradation |
| 2-1 | f | 0.2277 | 0.0532 | / | / | Min SINR: 39% Max SINR: 70% Average SINR: 53% | SINR degradation is less than 1dB for inter-system. For intra-system interference, outage percentage is larger than 39%. |
| 2-1 | c | 0.0193 | 0.0042 | 0.0015 | 0.0007 | / | SINR and throughput degradation can be negligible |
| 2-1 | d | 0.0333 | 0.0673 | / | / | Min SINR: 0% Max SINR: 49% Average SINR: 0% | SINR degradation is less than 1dB for inter-system. For intra-system interference, outage percentage is 0% when SINR are min value and average value. Outage percentage is larger than 49% when max SNR. |
| 2-2(10% indoor UE) | e | 0 | 0 | 0 | 0 | / | No SINR and throughput degradation |
| 2-2(10% indoor UE) | f | 13.3715 | 6.0655 | / | / | Min SINR: 62% Max SINR: 83% Average SINR: 72% | SINR degradation is larger than 1dB for inter-system. For intra-system interference, outage percentage is large than 62%. |
| 2-2(10% indoor UE) | c | 0.3133 | 0.1643 | 0.0209 | 0.0261 | / | SINR and throughput degradation can be negligible |
| 2-2(10% indoor UE) | d | 0.0323 | 0.074 | / | / | Min SINR: 0% Max SINR: 48% Average SINR: 0% | SINR degradation is larger than 1dB for inter-system. For intra-system interference, outage percentage is 0% when SINR are min value and average value. Outage percentage is 48% when use max SINR. |
| 2-2(100% indoor0 UE) | e | 0 | 0 | 0 | 0 |  | No SINR and throughput degradation |
| 2-2(100% indoor UE) | f | 28.4993 | 24.306 | / | / | Min SINR: 95% Max SINR: 98% Average SINR: 97% | SINR degradation is larger than 1dB for inter-system. For intra-system interference, outage percentage is larger than 95%. |
| 2-2(100% indoor UE) | c | 21.3042 | 15.4753 | 0 | 0.8078 | / | SINR degradation is larger than 1dB and throughput degradation is 80.78% |
| 2-2(100% indoor UE) | d | 0.312 | 0.0685 | / | / | Min SINR: 0% Max SINR: 49% Average SINR: 0% | SINR degradation is less than 1dB for inter-system. For intra-system interference, outage percentage is 0% when SINR are min value and average value. Outage percentage is 49% when use max SINR. |

Observation 1: For D1T1-A2 in scenario 1-1, there is no co-existence impact between A-IoT and NR when Legacy UE is only outdoor.

Observation 2: For D1T1-B in scenario 2-1, A-IoT and NR for in-band deployment can co-exist successfully when legacy UE is only outdoor.

Observation 3: R2D/CW2D/D2R is DL spectrum for D1T1 with better performance compared to R2D is DL spectrum and CW2D/D2R is UL spectrum when some NR UE is indoor.

Proposal 1: Guard RB(s) between NR and A-IoT needs to be considered for co-existence between A-IoT and NR when 10% legacy UE is indoor for D1T1-B and 100% legacy UE is indoor for D1T1-A2.

CW considerations

Agreement: Following is optional for co-existence evaluation: - Use SNR degradation of 1 dB, 3 dB and 10 dB in coexisting study to evaluate the CW cancellation impact for CW inside topology A2 - Assume perfect CW cancellation in SINR calculation, add degradation afterwards in the simulation

Scenario 1-1 and scenario 1-2 are for CW inside topology A2. These two scenarios have four cases, they are case a, b, c, d. During the co-existence study, we assume that only reader is affected by CW interference. Therefore, there is case b needs to evaluate the CW cancellation impact. The evaluation for the CW cancellation impact as follow.

Table1: SINR degradation by CW interference for baseline SINR degradation

| Deployment | Case No | Baseline SINR @5% (dB) for no CW interference | Baseline SINR @5% (dB) for CW interference | Baseline SINR degradation@5% (dB) | Baseline SINR @50% (dB) for no CW interference | Baseline SINR @50% (dB) for CW interference | Baseline SINR degradation@50%(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario 1-1 | b | 5.3 | -9.4 | 14.7 | 16.5 | 3.7 | 12.8 |
| Scenario 1-2 | b | 4.9 | -10.5 | 15.4 | 17.2 | 3.8 | 13.4 |

Table2: SINR degradation by CW interference for inter-system @5%

| Deployment | Case No | Baseline SINR @5% (dB) for no CW interference | ACI SINR@5% (dB) for no CW interference | SINR degradation @5% (dB) for no CW interference | Baseline SINR @5% (dB) for CW interference | ACI SINR@5% (dB) for CW interference | SINR degradation @5% (dB) for CW interference | SINR degradation @5% =no interference CW-CW interference(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario 1-1 | b | 5.3 | 5.3 | 0 | -9.4 | -9.4 | 0 | 0 |
| Scenario 1-2 | b | 4.9 | 4.9 | 0 | -10.5 | -10.5 | 0 | 0 |

Table3: SINR degradation by CW interference for inter-system @50%

| Deployment | Case No | Baseline SINR @5% (dB) for no CW interference | ACI SINR@5% (dB) for no CW interference | SINR degradation @5% (dB) for no CW interference | Baseline SINR @5% (dB) for CW interference | ACI SINR@5% (dB) for CW interference | SINR degradation @5% (dB) for CW interference | SINR degradation @5% =no interference CW-CW interference(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario 1-1 | b | 16.5 | 16.5 | 0 | 3.7 | 3.7 | 0 | 0 |
| Scenario 1-2 | b | 17.2 | 17.2 | 0 | 3.8 | 3.8 | 0 | 0 |

Based on the table 1, we can see that baseline degradation is larger than 10dB.It is observed that CW interference has severe impact for Reader for CW inside topology A2. In addition, we can see that CW interference has no impact for NR system.

Observation 4: Baseline SINR degradation is larger than 10dB compared to no CW interference. It is observed that CW interference has a severe impact on Reader for CW inside topology A2.

Conclusions

In this paper, we provide our views on co-existence between A-IoT and NR/UE, we have made the following observations and proposals.

Observation 1: For D1T1-A2 in scenario 1-1, there is no co-existence impact between A-IoT and NR when Legacy UE is only outdoor.

Observation 2: For D1T1-B in scenario 2-1, A-IoT and NR for in-band deployment can co-exist successfully when legacy UE is only outdoor.

Observation 3: R2D/CW2D/D2R is DL spectrum for D1T1 with better performance compared to R2D is DL spectrum and CW2D/D2R is UL spectrum when some NR UE is indoor.

Proposal 1: Guard RB(s) between NR and A-IoT needs to be considered for co-existence between A-IoT and NR when 10% legacy UE is indoor for D1T1-B and 100% legacy UE is indoor for D1T1-A2.

Observation 4: Baseline SINR degradation is larger than 10dB compared to no CW interference. It is observed that CW interference has a severe impact on Reader for CW inside topology A2.

Reference

- R4-2417196, WF on co-existence evaluation for ambient IoT, CMCC, INC.TSG-RAN WG4#112bis meeting