3GPP TSG-RAN WG4 Meeting#113		R4-2418412

Orlando, United States, Nov 18th -22nd, 2024

Agenda item:	7.23.2.2

Source: 	Spreadtrum Communications

Title: 	Discussion on RX requirements of REFSENS for LP-WUR

Document for:	Discussion

Introduction

In RAN4#112bis meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

REFSENS requirements

For below 6GHz, the REFSENS level can be calculated by the equation below.

Sensitivity (dBm) = -174dBm (KT)+NF+10log (RX BW)-Diversity gain+ SNR+IM

IM value for REFSENS

As we know, we need to consider the Implementation Margin (IM) because handheld UE/CPE and so on terminals have so many antenna combinations. Now existing NR UE use 2.5dB for IM value. In our understanding, LP-WUR also is applied in handheld UE and other form factor terminals but still have so many antenna combinations. Thus, we could reuse 2.5dB IM value for LP-WUR. In addition, there is no obvious difference between OOK-based and OFDM-based receivers with regard to antenna combinations.

Proposal 1: IM value could use 2.5dB for OOK-based and OFDM-based receivers.

NF GAP between LR and MR for REFSENS (assume MR as 9dB)

Agreement: - RAN4 further discuss potential NF for OOK-based and OFDM-based receiver. - Linked to resolution of Issue 2-2-8

As to noise figure (NF), we could refer to NF part in RAN1 evaluation. RAN1’s conclusion in RAN1#116bis about NF is that all three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported. Based on the above information, NF can be define as delta NF on top of MR. I.e., OOK based LP-WUS is + 8 dB and OFDM based LP-WUS is +2 dB on top of MR but NF of MR is 9dB.

Proposal 2: NF can be defined as delta NF on top of MR (e.g., OOK based LP-WUS is + 8 dB and OFDM based LP-WUS is +2 dB on top of MR (assume MR NF is 9dB).

SNR value for LR and MR for REFSENS

According to RAN1’s conclusion, SNR evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, agreements as follow.

Agreement For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

As we all know, under the same conditions, the performance of OFDM-based receiver is better than OOK-based receiver. For MR, just one SNR to derive the RFESENS. Therefore, we can define a set of SNR for OOK-based and OFDM-based receiver based on the worst case. According to the RAN1’s conclusion, in our understanding, we can use SNR=-3dB to derive the REFSENS for OOK-based and OFDM-based LP-WUR.

Proposal 3: To define SNR=-3dB for OOK-based and OFDM-based LP-WUR to derive the REFSENS.

Summary for REFSENS

Based on the above comprehensive analysis for IM, NF and SNR, we can use a table to derive the REFSENS for OOK-based and OFDM-based receiver as follow. RX BW is 5MHz, diversity gain is 0dB.

Proposal 4: Adopt the value of REFSENS in Table1 for OOK-based and OFDM-based LP-WUR.

|  | OOK-based receiver | OFDM-based receiver |
| --- | --- | --- |
| IM(dB) | 2.5 | 2.5 |
| NF(dB) | 17 | 9 |
| SNR(dB) | -3 | -3 |
| RFESENS(dB) | -90.5 | -98.5 |

Table 1: REFSENS for OOK-based and OFDM-based LP-WUR

Conclusions

In this paper, we provide our views on RX requirements of REFSENS, we have made the following proposals.

Proposal 1: IM value could use 2.5dB for OOK-based and OFDM-based receiver.

Proposal 2: NF can be defined as delta NF on top of MR (e.g., OOK based LP-WUS is + 8 dB and OFDM based LP-WUS is +2 dB on top of MR (assume MR NF is 9dB).

Proposal 3: To define SNR=-3dB for OOK-based and OFDM-based LP-WUR to derive the REFSENS.

Proposal 4: Adopt the value of REFSENS in Table1 for OOK-based and OFDM-based LP-WUR.

|  | OOK-based receiver | OFDM-based receiver |
| --- | --- | --- |
| IM(dB) | 2.5 | 2.5 |
| NF(dB) | 17 | 9 |
| SNR(dB) | -3 | -3 |
| RFESENS(dB) | -90.5 | -98.5 |

Table 1: REFSENS for OOK-based and OFDM-based LP-WUR

- Reference

- R4-2417112, WF on NR LP-WUS UE requirements, vivo, TSG-RAN WG4 112bis meeting