3GPP TSG-RAN4 Meeting #113	R4-2418420

# Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0666 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-4 corrections of PDSCH and corresponding HARQ-ACK relationship for 30D4S6U TDD pattern for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Perf |  |  |  |  |  | Date: |  |  | 2024-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For UEs that don’t support outOfOrderOperationDL-r16, which test case to apply is TBD |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the applicability rule to cover the test case for UE that don’t support outOfOrderOperationDL-r16 - Update the timing relationship between PDSCH and corresponding HARQ-ACK regarding the UE capability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ATG spec will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.12, A.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change#1>

#### 5.1.1.12	Applicability of requirements for ATG

The performance requirements in Table 5.1.1.12-1 shall apply for ATG UEs which support optional feature airToGroundNetwork-r18.

Other performance requirements mandatory for UE supporting NR operation defined in Section 5 but not included in table 5.1.1.12-1 should not be considered applicable to ATG UEs.

Table 5.1.1.12-1: Requirements applicability for ATG UEs

| UE capability | Test type |  | Test list | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| ATG UE with 2RX | FR1 FDD | PDSCH | All tests in Clause 5.2.2.1.22 |  |  |
|  |  | PDCCH | Test number 1, 3 and 5 in Table 5.3.2.1.1 Test number 3 in Table 5.3.2.1.2 |  |  |
|  |  | PBCH | Clause 5.4.2.1 |  |  |
|  |  | SDR | Clause 5.5.1 |  |  |
|  | FR1 TDD | PDSCH | Tests 1-1, 1-2, 1-3 in Clause 5.2.2.2.23 | Only if ATG UE does not supports optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |  |
|  |  |  | Tests 1-4, 1-5, 1-6 in Clause 5.2.2.2.23 | Only if ATG UE supports the optional features k1-RangeExtensionATG-r18, and maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |  |
|  |  | PDCCH | All test cases in Table 5.3.2.2.1 All test cases in Table 5.3.2.2.2 |  |  |
|  |  | PBCH | Clause 5.4.2.2 |  |  |
|  |  | SDR | Clause 5.5.1 |  |  |
| ATG UE with 4RX | FR1 FDD | PDSCH | All tests in Clause 5.2.3.1.21 |  |  |
|  |  | PDCCH | Test number 1, 3 and 5 in Table 5.3.3.1.1 Test number 3 in Table 5.3.3.1.2 |  |  |
|  |  | PBCH | Clause 5.4.3.1 |  |  |
|  |  | SDR | Clause 5.5.1 |  |  |
|  | FR1 TDD | PDSCH | Tests 1-1, 1-2, 1-3 in Clause 5.2.3.2.22 | Only if ATG UE does not supports optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |  |
|  |  |  | Tests 1-4, 1-5, 1-6 in Clause 5.2.3.2.22 | Only if ATG UE supports the optional features k1-RangeExtensionATG-r18, or maxHARQ-ProcessNumberATG-r18 with 32 DL processes. |  |
|  |  | PDCCH | All test cases in Table 5.3.3.2.1 All test cases in Table 5.3.3.2.2 |  |  |
|  |  | PBCH | Clause 5.4.3.2 |  |  |
|  |  | SDR | Clause 5.5.1 |  |  |

# <End of change#1>

# <Start of change#2>

A.1.2	TDD UL-DL configurations for FR1

<Unchanged parts skipped>

Table A.1.2-2c: TDD UL-DL configuration for SCS 30 kHz for ATG

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  | FR1.30-8 |
| TDD Slot Configuration pattern (Note 1) |  |  | 30D4S6U |
| Special Slot Configuration (Note 2) |  |  | 14G |
| referenceSubcarrierSpacing |  | kHz | 30 |
| Pattern 1 | dl-UL-TransmissionPeriodicity | ms | 10 |
|  | nrofDownlinkSlots | slot | 20 |
|  | nrofDownlinkSymbols | symbol | 0 |
|  | nrofUplinkSlot | slot | 0 |
|  | nrofUplinkSymbols | symbol | 0 |
| Pattern 2 | dl-UL-TransmissionPeriodicity | ms | 10 |
|  | nrofDownlinkSlots |  | 10 |
|  | nrofDownlinkSymbols |  | 0 |
|  | nrofUplinkSlot |  | 6 |
|  | nrofUplinkSymbols |  | 0 |
| The number of slots between PDSCH and corresponding HARQ-ACK information for UE supporting outOfOrderOperationDL-r16 (Note 3) |  |  | 34 if mod(i,40) = 0, 1, 2, 3, 4, 5 28 if mod(i,40) = 6, 7, 8, 9, 10, 11 22 if mod(i,40) = 12, 13, 14, 15, 16, 17 16 if mod(i,40) = 18, 19, 20, 21, 22, 23 10 if mod(i,40) = 24, 25, 26, 27, 28, 29 |
| The number of slots between PDSCH and corresponding HARQ-ACK information for UE not supporting outOfOrderOperationDL-r16 (Note 3) |  |  | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 10 if mod(i,40) = 29 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index of all slots in every 20ms, i = {0,…,39} |  |  |  |

# <End of change#2>