3GPP TSG-RAN WG4 Meeting # 113		R4-2418421

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	5.21.3

Source: 	CMCC

Title: 			 Discussion on core maintenance for NTN enhancement

Document for:	Discussion

- Introduction

In past several meetings, RAN4 focused on the Rel-18 NTN maintenance issues, the latest progress has been agreed in the WF [1]. In this contribution, we further discuss on the leftovers.

- Discussion

Issue 4-1: TN to NTN cell reselection

Agreement (online):

- For NTN incapable UE, if both TN and NTN carriers are broadcasted for neighbour cells measurement in IDLE/Inactive mode, the existing TN-to-TN cell reselection requirements apply.

FFS:

- For TN to NTN cell re-selection requirement when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if UE is configured by network to have at least one high priority carrier which contains NTN cells, the requirements for GNSS ON shall be applied

For UE camping on TN cell which perform TN to NTN cell re-selection, the requirement includes T_GNSS, i.e., TTFF(Time To First Fix) value, which is left undefined in RRM spec. If UE GNSS has been switched ON, T_GNSS can be assumed zero.

Company proposed that the requirement for UE GNSS has been switched ON (T_GNSS=0) shall be applied if UE is configured by network to have at least on high priority carrier which contains NTN cells. Generally, we think it is a common UE implementation especially when UE is required to measure per high priority layer within 60s, considering TTFF = 20s as defined in TS 38.171. However, if UE is required to measure per high priority layer with relaxation, i.e., within 1h, such UE implementation assumption seems not valid anymore.

Observation 1: For the TN to NTN cell re-selection case that UE is configured by network to have at least one high priority layer which contains NTN cells, if UE is required to measure per high priority layer within 60s, GNSS module keeping ON is a common UE implementation, if UE is required to measure per high priority layer with relaxation, i.e., within 1h, GNSS module keeping ON is not a common UE implementation.

Based on the observation above, we think it is reasonable to apply the TN to NTN cell re-selection requirement for GNSS ON when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if UE is configured by network to have at least one high priority layer which contains NTN cells and UE is required to measure per high priority carrier within 60s.

Proposal 1: For TN to NTN cell re-selection requirement when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the requirements for GNSS ON shall be applied if UE is configured by network to have at least one high priority layer which contains NTN cells and UE is required to measure per high priority carrier within 60s.

Issue 4-2: NTN to TN cell reselection

Currently, the NTN to TN cell re-selection delay requirement had been defined in the spec. However, the maximum interruption requirement in paging reception when target cell with TN carrier is still missing, which should be also defined. This interruption including the delay for system information and the delay for fine time and frequency synchronization, due to the target cell is TN cell, we think the legacy requirements can be reused.

At inter-frequency cell re-selection, for target cell with TN carrier, the interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms, and K = 2.

At inter-RAT cell re-selection, for target cell with TN carrier , the interruption time must not exceed TSI-EUTRA + 55 ms.

Proposal 2: Define maximum interruption requirement in paging reception when target cell with TN carrier:

- At inter-frequency cell re-selection, for target cell with TN carrier, the interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms, and K = 2.

- At inter-RAT cell re-selection, for target cell with TN carrier , the interruption time must not exceed TSI-EUTRA + 55 ms.

### Issue 5-2-S: Soft’ Satellite switch

FFS:

- RAN4 to not consider PDD reporting between serving and target satellites involved in the satellite switching without PCI change.

FFS:

- To enhance scheduling restriction in soft satellite switching, consider modifying the capability rule such that: if UE indicates to support soft satellite switching without PCI change, it also means UE supports both simultaneousRxDataSSB-DiffNumerology and parallelMeasurementWithoutRestriction-r17.

We prefer not to consider PDD reporting and introduce UE capability bundling with following reasons:

- The network is still workable without any PDD assistance information from UE and UE capability bundling. The duration of the scheduling restriction from network side depends on the maximum propagation delay difference between source cell and target cell, which further depends on the practical network scenario. The worst case is that the source cell could not schedule any data transmission during the whole period between T-seviceStart and T-service.

- The feature will be less attractive if UE capability bundling is considered

- RAN2 has agreed not to consider PDD reporting between serving and target satellites involved in the satellite switching without PCI change.

Proposal 3: Don’t consider PDD reporting and UE capability bundling for satellite switching with re-synchronization.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Observation 1: For the TN to NTN cell re-selection case that UE is configured by network to have at least one high priority layer which contains NTN cells, if UE is required to measure per high priority layer within 60s, GNSS module keeping ON is a common UE implementation, if UE is required to measure per high priority layer with relaxation, i.e., within 1h, GNSS module keeping ON is not a common UE implementation.

Proposal 1: For TN to NTN cell re-selection requirement when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the requirements for GNSS ON shall be applied if UE is configured by network to have at least one high priority layer which contains NTN cells and UE is required to measure per high priority carrier within 60s.

Proposal 2: Define maximum interruption requirement in paging reception when target cell with TN carrier:

- At inter-frequency cell re-selection, for target cell with TN carrier, the interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms, and K = 2.

- At inter-RAT cell re-selection, for target cell with TN carrier , the interruption time must not exceed TSI-EUTRA + 55 ms.

Proposal 3: Don’t consider PDD reporting and UE capability bundling for satellite switching with re-synchronization.

- Reference

- R4-2414100, WF on RRM requirements for NR_NTN_enh, Moderator (Qualcomm Incorporated).