3GPP TSG-RAN WG4 Meeting #113     	R4-2418429

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.16.3

Source: 	CMCC

Title: 			 Simulation results for BS interference suppressing with MMSE-IRC

Document for:	Discussion

- Introduction

Based on the simulation assumption approved in last meeting[1], we give our simulation results.

- Simulation results

PUSCH requirements with interference

|  |  | SNR for HomoNet (dB) | SNR for HetNet (dB) |
| --- | --- | --- | --- |
| 1T2R (1 interference) | MMSE-IRC | 1.3 | 2.7 |
|  | MMSE-MRC | 4.7 | 9.3 |
|  | Gain | 3.4 | 6.6 |
| 1T2R (2 interference) | MMSE-IRC | 3 | 4.6 |
|  | MMSE-MRC | 7.2 | 12 |
|  | Gain | 4.2 | 7.4 |
| 1T4R | MMSE-IRC | 5.5 | 4.1 |
|  | MMSE-MRC | 10.1 | 14.8 |
|  | Gain | 4.6 | 10.7 |
| 1T8R | MMSE-IRC | 10.8 | 10.1 |
|  | MMSE-MRC | 16.2 | 20 |
|  | Gain | 5.4 | 9.9 |

- Conclusion

In this contribution, we provides some initial simulation results for BS MMSE-IRC receiver.

- Simulation assumption

PUSCH requirements with interference

Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD, FDD |
| TDD pattern |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  | TDD: 30kHz/10MHz FDD: 15kHz/5MHz |
| Transmit precoding |  | Disabled |
| Antenna configuration |  | 1x2 (1/2 interferes) 1x4 (2 interferes) 1x8 (2 interferes) |
| Propagation condition |  | For both serving and interference cell TDLC300-100 for HomNet TDLA30-10 for HetNet |
| MCS |  | 2R: MCS 5(QPSK, R=379/1024) 4R: MCS 13(16QAM, R=490/1024) 8R: MCS 24(64QAM, R=772/1024) |
| PUSCH configuration | Mapping type | Type A/B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | DM-RS sequence generation | nSCID =0 for all cells Different NID0 for serving and interference cells |
| Maximum HARQ transmissions |  | 4 |
| Redundancy version coding sequence |  | {0,2,3,1} |
| Test metric |  | SNR@70% max TP |

| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID 1 for interferer 1, cell ID 2 for interferer 2 |
| --- | --- |
| INR values for Homogeneous deployment | INRs 5.35 dB in case of 1 interference cells INRs 7.28, -2.52 dB in case of 2 interference cells |
| INR values for Heterogeneous deployment | INRs 9.82 dB in case of 1 interference cells INRs 12.38, -0.97 dB in case of 2 interference cells |
| Modulation order of interfering PUSCH | 16QAM randomly modulated symbols |
| Interference cell modeling | Interfering cell has same channel model as for serving cell and it is independently generated for each cell. |

- Reference

[1] R4-2416555, WF for [112bis][320] NR_demod_Ph5_Part2_UE, Moderator (Nokia)