# 3GPP TSG-RAN WG4 Meeting #113                              R4-2418440

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.19.3.1

Source: 	CMCC

Title: 			 Discussion on RRM requirements for event-driven beam management for NR MIMO Phase 5

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR MIMO Phase 5 was approved [1], the objectives are duplicated as following.

RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last meeting, there was initial discussion on NR MIMO Phase 5, and a WF was agreed [2]. This contribution provides discussion on RRM requirements for event-driven beam management.

- Discussion

Issue 1-1: How to define Event triggered measurement reporting delay? < Agreement> - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air. - Ending point is the point when the UE transmits a [first PUCCH] triggered by the event. - “[first PUCCH]” can be updated based on further RAN1/2 agreement. Issue 1-2: How to define measurement period in Event triggered measurement reporting delay? <Way forward > - Option 1: - If multiple beams are measured with different measurement period, RAN4 to discuss how to consider the total measurement time. - Option 2: - It is proposed to take measurement period defend for Rel-18 LTM as baseline - Option 3: - RAN4 should investigate whether those measurement periods can be reused for defining measurement reporting delay requirements for UE-initiated/event-driven beam management - Option 4: - Use legacy measurement period for TL1-RSRP_Measurement_Period_SSB for SSB based intra-cell , inter-cell, TL1-RSRP_Measurement_Period_CSI-RS for CSI RS based. - Option 4a: - Remove Treport or set to 0 in legacy requirements. - Option 4b: - Remove considerations for HST enhancements/ scenarios - From legacy measurement period, for new beam report further discuss if M is needed or can be removed/assumed 1. - From legacy measurement period, for new beam report further discuss if P can be the same or modified for simplification. - Option 4c: - For FR2, the T_L1_measurement_period is scaled by N. - Other options are not precluded.

It was agreed in last meeting that for UE-initiated/event-driven beam reporting, the starting point is the point that the event is met over the air. The ending point is the point when the UE transmits a [first PUCCH] triggered by the event.

For legacy L3 based event triggered measurement report, as duplicated as following, the measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. It can be seen that the definition of L1 based event triggered measurement reporting delay is similar as the definition for L3 based event triggered measurement reporting delay.

TS 38.133 9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

As duplicated as above, the L3 event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify, within which UE shall be able to identify a new detectable cell. Since inter-cell measurement is considered, it can be considered to use Rel-18 LTM measurement delay as baseline to define measurement reporting delay for SSB bsed UE-initiated/event-driven beam reporting.

For Rel-18 LTM, the measurement period is defined as TL1-RSRP_Measurement_Period if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period + TSSB_time_index, with the assumption that the target cell is known. Similar as event triggered reporting for L3 measurement, it can be considered that the event triggered L1 measurement reporting delay, measured without filtering is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index. When filtering is used, an additional delay can be expected.

Proposal 1: for reporting delay for SSB based UE-initiated/event-driven beam reporting, it is proposed to take measurement period defind for Rel-18 LTM as baseline

- in detail, the reporting delay for SSB bsed UE-initiated/event-driven beam reporting is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index , which are specified for L1-RSRP measurement for Rel-18 LTM .

According to RAN1 agreements, on UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management. And in previous meeting, RAN4 also agreed that L1-RSRP is used as measurement metrics for beam report, and agreed to define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS. For CSI-RS based beam report, legacy measurement delay requirements only apply to CSI-RS resources of the serving cell. While according to RAN1 design, measurement on beam from neighbour cell are also considered, it is necessary to discuss whether to reuse legacy L1 measurement delay requirements or to define new delay requirements to cover neighbour cell. And the details can be further discussed.

Proposal 2: for reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused.

Issue 1-4: How to consider the time window of triggering event determination in Event triggered measurement reporting? <Way forward > - Option 1: - RAN4 needs to consider the impact of triggering window defined in RAN1. - Option 2: - It is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M). - It is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M) - Option 3: - No additional time-window-based filtering or other filtering should be applied for the L1-RSRP measurements - Option 4: - Discuss the necessity of filtering based on the two trigger-event detection solutions approved in RAN1. - For the basic solution, one-shot measurement is defined by RAN1. No room to consider filtering. - For the optional solution, multi-shot measurement with at least M instance is defined by RAN1. Filtering is feasible. - Option 5: - RAN4 to discuss how to define one Event-2 instance for following Event-2 case: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam - Option 6: - Wait further RAN1 progress. - Other options are not precluded.

According to RAN1 design, except basic feature (i.e. once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs), a filtering with timer and counter M subject to UE capability is introduced.  Since RAN1 has supported both cases, RAN4 need to define requirements for both cases.

Agreements in RAN1#117 meeting Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

The above time-window-based filtering is about triggering event determination. If the reporting delay is defined as the time between an event that will trigger a beam report and the point when the UE starts to transmit the beam report over the air interface, it seems that time-window-based filtering has no impact on reporting delay. Neither impact on reporting accuracy is observed. Same reporting requirements, including reporting delay and reporting accuracy requirements, can be applied to the case without filtering and the case with filtering.

Proposal 3: for UE-initiated/event-driven beam reporting, it is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M).

Proposal 4: it is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M).

Issue 1-5: How to consider UL resource in Event triggered measurement reporting? <Way forward > - Option 1: - RAN4 needs to consider the impact of UL resource waiting time for L1 event triggered reporting. - Option 2: - The beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. - No need to differentiate mode A and mode B - Option 3: - Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B - Option 4: - To determine how to count for the uncertainty in acquiring UL resource for UEIBR, RAN4 waits further conclusion from RAN1 on whether the first PUCCH is SR or not (for Mode-A). - Other options are not precluded.

It was agreed that event triggered measurement reporting delay requirements are applicable for all events agreed in RAN1/2 and are applicable to both Mode A and B. As for whether to differentiate mode A and mode B, our view is that if we follow the approach for L3 measurement report delay to add clarification that this beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, it seems same reporting delay requierments can be applied.

Proposal 5: it is proposed to follow the approach for L3 measurement report delay to add clarification in the spec that the beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. In this case, no need to differentiate mode A and mode B.

Issue 1-12: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward > - Option 1: - The existing accuracy requirements for L1 measurements can be applied. No new accuracy requirements. - Option 2: - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements. - Option 3: - Discuss the accuracy for the two trigger-event solutions respectively. - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement; - For the optional trigger-event detection, reusethe existing L1-RSRP accuracy requirement. - Other options are not precluded.

Except reporting delay requirements, reporting accuracy requirements also need to be defind. Similar as L3 based measurement report, for UE-initiated/event-driven beam reporting, the reported L1 measurement contained in event triggered beam reports shall meet the accuracy requirements.

According to RAN1 agreements, on UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management. And in last meeting, RAN4 also agreed that L1-RSRP is used as measurement metrics for beam report, and agreed to define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS.

For SSB based beam report, accuracy requirements defined for Rel-18 LTM can be considered. Rel-18 LTM specify intra-frequency and inter-frequency L1-RSRP measurement accuracy requirements. For both cases, absolute and relative accuracy are specified with legacy side condition SNR= -3dB. And  reuse legacy absolute and relative accuracy requirements for intra-frequency LTM L1-RSRP measurement. For inter-frequency L1-RSRP measurement accuracy, absolute and relative accuracy are the same, which equal to intra-frequency absolute accuracy. It is necessary to discuss whether LTM measurement accuracy can be reused for  event triggered L1 measurement reporting. In our view, the accuracy requirements for Rel-18 LTM can be used as baseline. We are also open to update accuracy requirements based on new scenarios/conditions if needed.

For CSI-RS based beam report, legacy accuracy requirements only apply to CSI-RS resources of the serving cell. While according to RAN1 design, measurement on beam from neighbour cell are also considered, it is necessary to define new accuracy requirements to cover neighbour cell.

Proposal 6: for UE-initiated/event-driven beam reporting, except reporting delay requirements, it is proposed to define reporting accuracy requirements.

- The reporting accuracy requirement is that the reported L1 measurement contained in event triggered beam reports shall meet the accuracy requirements

- For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

- For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account.

- Conclusion

This contribution provides discussion on RRM requirements for event-driven beam management for NR MIMO Phase 5. The proposals are:

Proposal 1: for reporting delay for SSB based UE-initiated/event-driven beam reporting, it is proposed to take measurement period defind for Rel-18 LTM as baseline

- in detail, the reporting delay for SSB bsed UE-initiated/event-driven beam reporting is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index , which are specified for L1-RSRP measurement for Rel-18 LTM .

Proposal 2: for reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused.

Proposal 3: for UE-initiated/event-driven beam reporting, it is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M).

Proposal 4: it is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M).

Proposal 5: it is proposed to follow the approach for L3 measurement report delay to add clarification in the spec that the beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. In this case, no need to differentiate mode A and mode B.

Proposal 6: for UE-initiated/event-driven beam reporting, except reporting delay requirements, it is proposed to define reporting accuracy requirements.

- The reporting accuracy requirement is that the reported L1 measurement contained in event triggered beam reports shall meet the accuracy requirements

- For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

- For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account.

- Reference

- RP-234007, New WID: NR MIMO Phase 5

- RP-242394, WID revision: NR MIMO Phase 5

- R4-2416856, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, Samsung