# 3GPP TSG-RAN WG4 Meeting #113                             R4-2418449

# Orlando, US, 18th - 22nd November, 2024

Agenda Item:	7.15.2.1

Source: 	CMCC

Title: 			 Discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN #103 meeting, the new WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is FR2-1 SSB based L3 measurement delay reduction for connected mode, the details are duplicated as following.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

There is discussion on FR2-1 SSB based L3 measurement delay reduction for connected mode in last meeting, and a WF was approved [2]. This contribution provides discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor.

- Discussion

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF: Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment. Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” : - Option 1 (CATT, LGE, OPPO, MTK): use UE indication of Multi-Rx preference for L3 measurement and further discuss whether to reuse the existing signaling. - Option 1a(LGE, NTT DCM, OPPO): For power saving for operating multi-Rx simultaneous reception, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 1b(OPPO): Excepting network configuration for FBS, UE assistance information can be beneficial, and new UAI indication is feasible and it should be up to RAN2 decision. - Option 1c(MTK): RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 2 (Samsung): - Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off - Dynamic indication; Semi-static configuration - Option 3 (Samsung, Ericsson): enable FBS in a time period after FBS is triggered by the condition in issue 1-1-1 - Option 3a (Samsung): Introduce validity timer configuration with certain ranges via dedicated RRC signalling - Option 3b (Ericsson): NW indicates UE enabling FBS through L3 or lower layers signaling. To save power consumption, after receiving the indication, UE may enable FBS in a predefined time period or for predefined measurement samples. After that, the UE is allowed to disable FBS or fall back to normal beam sweeping. - Option 4 (Samsung): - Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event - Option 5 (Samsung): - UE autonomously fallback based on measurement - Option 6 (HW): - For scenario 1/2/3/4, UE can switch back to normal measurement (i.e. single panel activated) when UE has reported qualified results. - Option 7 (Intel, QC): - Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced. (Intel) - Other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed. (QC)

In last meeting, it was agreed that RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions, and the criteria/threshold(s) are configured by RRC signaling.

As for the details of the triggering condition, firstly, UE location can be considered. When UE in the cell edge, if the L3 measurement delay is reduced by activating fast beam sweeping, the L3 measurement will be reported timely, and better experience of handover is expected. When UE in the cell center, the mobility demand is not urgent, legacy L3 measurement with nornal beam sweeping is enough, and fast beam sweeping can be deactivated which is better for power saving. Network can configure RSRP threshold, when the measured RSRP is lower than the threashold, UE need to activate the FBS. Otherwise, the FBS can be deactivated.

Proposal 1: it is proposed to consider UE location/link quality as the triggering condition

- When the measured RSRP is lower than a network configured threshold, FBS is activated

- Otherwise, FBS is deactivated

We do not think UE mobility can be considered as the triggering condition. The motivation to introduce fast beam sweeping is to reduce measurement delay, which is to improve mobility performance. As mentioned above, if UE is in cell edge, no matter its velocity, it is expected that FBS can be activated to report measurement timely.

Propasal 2: it is proposed not to consider UE mobility as triggering condition for activation/deactivation of FBS.

Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX - Option 1 (Xiaomi): The SSB based L3 measurement delay reduction based on FBS can be dependent with DRX configuration. - Option 2 (CATT, CMCC, Samsung, CTC, NTT DCM, Nokia): L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case. - Option 3 (Ericsson): - L3 measurement enhancement is applicable to UE configured with DRX. It can be studied if some DRX cases really impact performance. - When L3 measurement enhancement is enabled, DRX can be ignored during the fast L3 measurement from tech. perspective.

According to the proponent of option 1, the motivation to deprioritize DRX case is that in case of measurement under DRX, the power consumption may be more crucial than the measurement delay itself when the basic mobility management performance can be guaranteed. We have different view and do not see the necessaity to deprioritize DRX case. Firstly, depriorotize DRX case will limit the application of this feature, which is not preferred. Secondly, as discussed above, activation/deactivation of L3 measurement delay reduction based on UE Rx BSF optimization can be considered to solve the power consumption issue.

Proposal 3: L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX, no need to prioritize DRX case.

- Issue 1-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF - Option 1 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Option 2 (CATT, vivo, CMCC, Apple, ZTE, CTC, NTT DCM, Nokia, OPPO, MTK): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability - Option 2a (CATT, CMCC, ZTE, CTC): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. - Option 2b (vivo): Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4. - Option 2c (Apple, CTC): For UE supporting R19 multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Option 2d (Nokia): At least support reduction of N from 8 to 4 in Rel-19. Support of other values such as N = 2, 6 to be discussed, and UE capability signalling to be introduced, if more than one value for reduced N is supported. - Option 2e (OPPO): Discuss the reduced scaling factors due to Rx BSF, considering both UE capabilities and applicable conditions. - Option 3 (vivo, Samsung, LGE): Introduce the UE capability to indicate the reduced M value (M=sample*BSF), - Option 3a (vivo): e.g., M=12, 14, 16, etc. - Option 3b (Samsung): suggest to take N=4, M=20 as the baseline. Whether to/How to define the other values can be based on SLS results (if necessary). - Option 3c (LGE): 3 or 4 Rx BSF value or additional scaling factor < 1 to reduce M value - Option 4 (vivo, LGE): - Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc. - Option 5 (Ericsson): - In order to cover all possible directions of neighbor cells, RX beam sweep doesn’t dynamically change, the factor can be defined as a single number, e.g., [4]. - Option 6 (Intel): - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Beam sweeping factor/total sample number of measurements can be cut half as a baseline while a margin can be added to the cut factor according to different SSB/SMTC configurations.

Rel-18 multi-Rx chain WI mainly focus on the L1 related measurement. According to the discussion in Rel-18 multi-Rx chain WI, it was agreed to introduce reduced beam sweeping factor for SSB-based L1-RSRP measurement if the UE is capable of multi-Rx operation [4], which are applied to L1-RSRP/RLM/BFD. The candidate values for reduced beam sweeping factor are: {2,4,6} for FR2-1. In our view, for UE capable of multi-Rx operation, we do not see issues to extend the Rel-18 RX beam sweeping factor reduction to L3 measurement. It is proposed to reduce L3 measurement delay by reducing RX beam sweeping factor, and the Rel-18 RX beam sweeping factor reduction can be used as baseline.

For option 2 and option 3, the main differnece is that the UE capability is based on the BSF or based on the M value. If we look at the legacy L3 delay requirements, BSF is relected by the value of M. M is related with the number of samples and BSF. However, the number of samples are differeent for different cases, for example, the sample number is 5 for intra-frequency measurement and 8 for inter-frequency measurement. If we go wih option 3, there are will be more candidate valus, and we may also need to differentiate which values are for intra-frequency measurement and which values are for inter-frequency measurement. If we go with option 2, we can simply update the M value in the spec.

Proposal 4: it is proposed to reduce L3 measurement delay by reducing RX beam sweeping factor, and the Rel-18 RX beam sweeping factor reduction, i.e. N= {2,4,6}, can be used as baseline.

- Conclusion

This contribution provides discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor. The proposals are:

Proposal 1: it is proposed to consider UE location/link quality as the triggering condition

- When the measured RSRP is lower than a network configured threshold, FBS is activated

- Otherwise, FBS is deactivated

Propasal 2: it is proposed not to consider UE mobility as triggering condition for activation/deactivation of FBS.

Proposal 3: L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX, no need to prioritize DRX case.

Proposal 4: it is proposed to reduce L3 measurement delay by reducing RX beam sweeping factor, and the Rel-18 RX beam sweeping factor reduction, i.e. N= {2,4,6}, can be used as baseline.

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2416853, WF for NR_RRM_Ph5_Part1, Apple

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple

- RP-240817, Moderator's summary for discussion on RRM enhancements