# 3GPP TSG-RAN WG4 Meeting #113                             R4-2418451

# Orlando, US, 18th - 22nd November, 2024

Agenda Item:	7.15.3

Source: 	CMCC

Title: 			 Discussion on fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In RAN #103 meeting, the new WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is fast SCell activation for UE supporting Rel-18 EMR, the details are duplicated as following.

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In last meetin, there was discussion on fast SCell activation for UE supporting Rel-18 EMR, an a WF was agreed [2]. This contribution provides discussion on fast SCell activation for UE supporting Rel-18 EM.

- Discussion

- Issue 1-1-1: Applicability of fast SCell activation delay requirements Proposals: - Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK) - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured. - Option 2: (OPPO, Nokia, Samsung, Ericsson) - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR. - Option 2a: (OPPO) - Consider the following two approaches to support the case: - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration. - Option 2b: (Samsung) - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured

Rel-18 eEMR (i.e. fast CA/DC setup) mainly focus on the validity check in order to report the valid measurement results. Measurement validation and report based on both EMR measurement and reselection measurements are supported, i.e. UE supporting  measValidationReportEMR-r18 or UE supporting measValidationReportReselectionMeasurements-r18 are considered.

As for the FFS part whether to include  the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured, our peference is no need to consider. The reason is that according to the WID [1], the objective is to reduce the SCell activation delay with valid EMR reporting. While according TS 38.133,  as highlighted in blue as following, if measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured, UE is not required to perform validity check. UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance, but it is not sure whether the reported measurement results are valid or not. From this point of view, no need to include the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured since this case is not within the WID scope.

TS 38.133 - 4.7.3 Measurement Report Requirements A UE shall perform validity check and report valid measurement results: - if the UE supports measValidationReportEMR-r18 and measIdleValidityDuration-r18 is configured for carriers in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, - if the UEsupports measValidationReportReselectionMeasurements-r18 and measReselectionValidityDuration-r18 is configured for carriers in measReselectionCarrierListNR-r18 The measurement results are considered valid if the following conditions are met for the validity check: - the measurements are performed before msg1 transmission for RRC resume/setup request within the last: - measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or - measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18, - the measurement results satisfy measurement accuracy requirement at the measurement instance. Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured. If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. RSRP, RSRQ measurements contained in the measurement reports shall meet the corresponding accuracy requirements at the measurement instance specified in the clauses 10.1.2B, 10.1.3B, 10.1.4B, 10.1.5B, 10.1.7B, 10.1.8B, 10.1.9B, 10.1.10B.

Observation 1: According to TS 38.133, if measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured, UE is not required to perform validity check. UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance, but it is not sure whether the reported measurement results are valid or not.

Proposal 1: for applicability of fast SCell activation delay requirements, no need to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured, since UE is not required to perform validity check for this case based on current spec, while according to the WID, the objective is to reduce the SCell activation delay with valid EMR reporting.

- Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting Agreement: - RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception. - For FR1, - [Y] equals to [5s] for direct SCell activation - FFS: Y for other SCell activation case. - For FR2, - Reuse the existing value of Y in direct SCell activation - FFS: Y for other SCell activation case. Agreement: - the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting). - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. For continued discussion in next meeting: - For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured: - For FR1: - use existing SSB based known SCell activation delay requirement with [2] SSB samples. - For FR2: - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X] - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW) - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion. - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1 - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured) - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT) - K = 0 or 1 if when the “validityduration” is smaller than X - K = 1 or 2 otherwise - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung) - K = 0 or 1 if when the “validityduration” is applied with smaller than X - K = 1 or 2 otherwise - FFS how to test Other proposals: - Proposal 6: For a UE supporting measValidationReportEMR or measValidationReportReselectionMeasurements, the legacy known conditions with deactivated SCell L3 measurement still apply. - Proposal 7: - In case the SCell has more than one actually transmitted SSB, the report should be based on beam level measurement result, i.e. MeasIdleConfig > MeasIdleCarrierNR > BeamMeasConfigIdle-NR > includeBeamMeasurements = true. - Proposal 8: It is feasible to introduce a new dedicated eEMR based fast SCell activation indication - The corresponding MAC-CE command could be provided together with MAC-CE for SCell activation - [Rel-18 EMR based SCell fast activation command] is received [after L3-RSRP reporting via Rel-18 EMR] and no later than the time when UE receives MAC-CE command for TCI activation

In last meeting, it was agreed that RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception. And [Y] equals to [5s] for direct SCell activation and FFS for other SCell activation case. For now, except direct SCell activation, MAC CE based SCell Activation is also supported. According to legacy known condition for MAC CE based SCell Activation, period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command. However, measCycleSCell is configured in connected mode, while current target scenario is that UE perform EMR measurement and send Rel-18 EMR report immediately after entering connected mode, in this case, measCycleSCell is not in use. And we do not see issue to use same value of Y for MAC CE based SCell Activation.

Proposal 2: for FR1, it is proposed that [Y] equals to [5s] for MAC CE based SCell Activation.

- Conclusion

This contribution provides discussion on fast SCell activation for UE supporting Rel-18 EMR. The proposals are:

Observation 1: According to TS 38.133, if measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured, UE is not required to perform validity check. UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance, but it is not sure whether the reported measurement results are valid or not.

Proposal 1: for applicability of fast SCell activation delay requirements, no need to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured, since UE is not required to perform validity check for this case based on current spec, while according to the WID, the objective is to reduce the SCell activation delay with valid EMR reporting.

Proposal 2: for FR1, it is proposed that [Y] equals to [5s] for MAC CE based SCell Activation.

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2416854, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2414041, WF on NR_RRM_Ph5_Part2 - fast SCell activation, CATT