# 3GPP TSG-RAN WG4 Meeting #113	R4-2418466

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.15.2.1

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN4 #112bis meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor issues captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1] and summary [2]:

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” : Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656. - Option 1 (CATT, LGE, OPPO, MTK): use UE indication of Multi-Rx preference for L3 measurement and further discuss whether to reuse the existing signaling. - Option 1a(LGE, NTT DCM, OPPO): For power saving for operating multi-Rx simultaneous reception, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 1b(OPPO): Excepting network configuration for FBS, UE assistance information can be beneficial, and new UAI indication is feasible and it should be up to RAN2 decision. - Option 1c(MTK): RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 2 (Samsung): - Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off - Dynamic indication; Semi-static configuration - Option 3 (Samsung, Ericsson): enable FBS in a time period after FBS is triggered by the condition in issue 1-1-1 - Option 3a (Samsung): Introduce validity timer configuration with certain ranges via dedicated RRC signalling - Option 3b (Ericsson): NW indicates UE enabling FBS through L3 or lower layers signaling. To save power consumption, after receiving the indication, UE may enable FBS in a predefined time period or for predefined measurement samples. After that, the UE is allowed to disable FBS or fall back to normal beam sweeping. - Option 4 (Samsung): - Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event - Option 5 (Samsung): - UE autonomously fallback based on measurement - Option 6 (HW): - For scenario 1/2/3/4, UE can switch back to normal measurement (i.e. single panel activated) when UE has reported qualified results. - Option 7 (Intel, QC): - Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced. (Intel) - Other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed. (QC) [Moderator]: try to discuss the following options(wording can be revised/polished) and companies can add more details if needed. For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”: - For Scenario 1/2/3/4, - Option 1 (CATT, LGE, OPPO, MTK): use UE indication of Multi-Rx preference for L3 measurement and further discuss whether to reuse the existing signaling. - FFS on option 1a/1b/1c. - Option 3 (Samsung, Ericsson): enable FBS in a time period after FBS is triggered by the condition in issue 1-1-1 - FFS on option 3a/3b. - Option 5 (Samsung): UE autonomously fallback based on measurement - For Scenario 5/7, - FFS: disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed.

For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”, since it was agreed that deactivating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions, and the criteria/threshold(s) are configured by RRC signaling. In order to assist to NW’s better decision and scheduling, it’s proposed to introduce UE indication, and a new UE assistance information is preferred.

Proposal 1: It’s proposed to introduce UE indication to assist to NW’s decision, and a new UE assistance information is preferred.

Status in the WF [1] and summary [2]:

Issue 1-1-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656. - Option 1 (CATT, Apple, Samsung, ZTE, QC, NTT DCM): For Multi-Rx based L3 measurement, 1 searcher is assumed for single carrier. - Option 2 (vivo, LGE, OPPO): For Multi-Rx based L3 measurement, 2 searcher is assumed for single carrier. - Option 3 (CTC): Searcher occupied by single carrier is depended on UE implementation, 1 searcher or 2 searchers can be occupied by the single carrier. - Option 4 (HW): RAN4 to skip the discussion about the searcher assumption to support FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. And whether UE can support this feature can be considered with UE capability discussion. [Moderator]: FFS on option 1 and option 2.

When CA mode is configured, one searcher is for PCell in PCC while another searcher is for SCells or other cells in SCCs, thus one searcher is considered for single carrier. Since the target scenario for the BSF topic is that UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC, if UE has two searchers, it’s not precluded to consider the two searchers used for the single carrier. However, in order to consider forward compatibility to extend the feature to CA mode in the future, it’s slightly preferred that one searcher is considered for single carrier.

Proposal 2: For Multi-Rx based L3 measurement, one searcher is assumed for single carrier.

Status in the WF [1] and summary [2]:

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656. - Option 1 (CATT, Apple, CTC, Nokia): if option 1 is adopted in issue 1-1-3, no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity - Option 1a (CTC, Intel): UE baseband processing capabilities need to be considered. - Option 1b (Nokia): - If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms - Option 2 (vivo): No need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity. - Option 3 (NTT DCM):SSB processing delay/time for this BSF optimization should be same as the legacy requirements because the BSF optimization should be achieved by simultaneously measurement of two spatial signal quality for one SSB. - Option 4 (QC): Proposal 3: In the handover delay equation, SSB processing time (2 ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - With above proposal, FR2 – FR2 HO timeline takes the following form (changes from legacy HO timeline is shown in blue): - Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + M * Tmargin ms - Where, - M is the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Tmargin is time for SSB post-processing. Tmargin can be up to 2ms. - Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is a known cell, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)* Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)*3* Trs ms. N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. Proposal 4: SSB processing time does not need to be introduced to handle optimized Rx BSF in following scenarios. - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection. - Option 5 (Ericsson): - Reflecting different assumptions of searcher number, processing time is specified differently for such two configurations. - For single carrier configuration in multi-Rx reception, extra processing time=0. - For CA/DC configuration in multi-Rx reception, if applicable, extra processing time could be reviewed after completing single carrier requirements. - For single carrier configuration, no extra processing time. For CA/DC configuration, if applicable, RAN4 to check the necessity of extra processing time for handover interruption time. - No extra processing time for cell identification delay. [Moderator]: FFS the following summarized options. (CA/DC is low priority, and therefore CA/DC based options are not reflected below): - For scenario 1/2/3/4 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms - Option 2: no need to add additional processing time in the delay requirement. - For scenario 5 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity - Option 2: In the handover delay equation, SSB processing time (2 ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - For scenario 7 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity - Option 2: no need to add additional processing time in the delay requirement.

For SSB processing delay/time for processing multiple beams received in a SMTC, if one searcher is occupied by the single carrier when UE supporting multi-Rx simultaneous reception for L3 delay enhancement, we support to consider an additional processing delay after the last SMTC periodicity.

Proposal 3: If one searcher is occupied by the single carrier when UE supporting multi-Rx simultaneous reception for L3 delay enhancement, it’s supported to consider an additional processing delay after the last SMTC periodicity.

Status in the WF [1] and summary [2]:

Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656. - Option 1 (Xiaomi): The SSB based L3 measurement delay reduction based on FBS can be dependent with DRX configuration. - Option 2 (CATT, CMCC, Samsung, CTC, NTT DCM, Nokia): L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case. - Option 3 (Ericsson): - L3 measurement enhancement is applicable to UE configured with DRX. It can be studied if some DRX cases really impact performance. - When L3 measurement enhancement is enabled, DRX can be ignored during the fast L3 measurement from tech. perspective. [Moderator]: Is option 2 agreeable?

For DRX issue, different ranges of DRX cycle are considered in L3 measurement requirements, and M value related to beam sweeping factor is defined for each DRX cycle range, therefore, it’s no need to prioritize/ deprioritize DRX case.

Proposal 4: L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case.

Status in the WF [1] and summary [2]:

Issue 1-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656. - Option 1 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Option 2 (CATT, vivo, CMCC, Apple, ZTE, CTC, NTT DCM, Nokia, OPPO, MTK): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability - Option 2a (CATT, CMCC, ZTE, CTC): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. - Option 2b (vivo): Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4. - Option 2c (Apple, CTC): For UE supporting R19 multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Option 2d (Nokia): At least support reduction of N from 8 to 4 in Rel-19. Support of other values such as N = 2, 6 to be discussed, and UE capability signalling to be introduced, if more than one value for reduced N is supported. - Option 2e (OPPO): Discuss the reduced scaling factors due to Rx BSF, considering both UE capabilities and applicable conditions. - Option 3 (vivo, Samsung, LGE): Introduce the UE capability to indicate the reduced M value (M=sample*BSF), - Option 3a (vivo): e.g., M=12, 14, 16, etc. - Option 3b (Samsung): suggest to take N=4, M=20 as the baseline. Whether to/How to define the other values can be based on SLS results (if necessary). - Option 3c (LGE): 3 or 4 Rx BSF value or additional scaling factor < 1 to reduce M value - Option 4 (vivo, LGE): - Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc. - Option 5 (Ericsson): - In order to cover all possible directions of neighbor cells, RX beam sweep doesn’t dynamically change, the factor can be defined as a single number, e.g., [4]. - Option 6 (Intel): - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Beam sweeping factor/total sample number of measurements can be cut half as a baseline while a margin can be added to the cut factor according to different SSB/SMTC configurations. - Recommended WF - [Moderator]: discuss option 2 and option 3 together with the scenarios 1/2/3/4/5/7.

In Rel-18 multi-Rx WI, fast beam sweeping method was introduced for FR2 L1 measurement delay reduction, which can be considered as baseline for FR2-1 SSB based L3 measurement delay reduction by optimizing Rx BSF. For UE supporting BSF reduction feature for L3 measurement, the Rx BSF can be reduced to 2/4/6 according to the UE capability.

Proposal 5: For UE supporting BSF reduction feature for L3 measurement, the Rx BSF can be reduced to 2/4/6 according to the UE capability.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, with the following proposals:

Proposal 1: It’s proposed to introduce UE indication to assist to NW’s decision, and a new UE assistance information is preferred.

Proposal 2: For Multi-Rx based L3 measurement, one searcher is assumed for single carrier.

Proposal 3: If one searcher is occupied by the single carrier when UE supporting multi-Rx simultaneous reception for L3 delay enhancement, it’s supported to consider an additional processing delay after the last SMTC periodicity.

Proposal 4: L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case.

Proposal 5: For UE supporting BSF reduction feature for L3 measurement, the Rx BSF can be reduced to 2/4/6 according to the UE capability.

- References

- R4-2416853, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.

- R4-2415656, Topic summary for [112bis][209] NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.