# 3GPP TSG-RAN WG4 Meeting #113	R4-2418468

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.15.3

Source:	China Telecom

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In RAN4 #112bis meeting, fast SCell activation for UE supporting Rel-18 EMR was discussed and the WF was approved [1].

In this paper, we present our views on fast SCell activation for UE supporting Rel-18 EMR issues captured in the WF [1].

- Discussion

Status in the WF [1]:

#### Issue 1-1-1: Applicability of fast SCell activation delay requirements Proposals: - Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK) - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured. - Option 2: (OPPO, Nokia, Samsung, Ericsson) - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR. - Option 2a: (OPPO) - Consider the following two approaches to support the case: - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration. - Option 2b: (Samsung) - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured

Considering the limited TU for this part, it’s preferred that RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured.

Proposal 1: Considering the limited TU, it’s preferred that RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured.

Status in the WF [1]:

#### Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting Agreement: - RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception. - For FR1, - [Y] equals to [5s] for direct SCell activation - FFS: Y for other SCell activation case. - For FR2, - Reuse the existing value of Y in direct SCell activation - FFS: Y for other SCell activation case. Agreement: - the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting). - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. For continued discussion in next meeting: - For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured: - For FR1: - use existing SSB based known SCell activation delay requirement with [2] SSB samples. - For FR2: - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X] - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW) - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion. - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1 - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured) - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT) - K = 0 or 1 if when the “validityduration” is smaller than X - K = 1 or 2 otherwise - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung) - K = 0 or 1 if when the “validityduration” is applied with smaller than X - K = 1 or 2 otherwise - FFS how to test Other proposals: - Proposal 6: For a UE supporting measValidationReportEMR or measValidationReportReselectionMeasurements, the legacy known conditions with deactivated SCell L3 measurement still apply. - Proposal 7: - In case the SCell has more than one actually transmitted SSB, the report should be based on beam level measurement result, i.e. MeasIdleConfig > MeasIdleCarrierNR > BeamMeasConfigIdle-NR > includeBeamMeasurements = true. - Proposal 8: It is feasible to introduce a new dedicated eEMR based fast SCell activation indication - The corresponding MAC-CE command could be provided together with MAC-CE for SCell activation - [Rel-18 EMR based SCell fast activation command] is received [after L3-RSRP reporting via Rel-18 EMR] and no later than the time when UE receives MAC-CE command for TCI activation

It was agreed that RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception, where the values of [Y] for direct SCell activation were achieved agreement, and the values of [Y] for other SCell activation case is FFS. It was also agreed that on top of single direct SCell activation at SCell addition, MAC CE based single SCell activation is also supported. For the values of [Y] for MAC CE based single SCell activation, it’s proposed that [Y] equals to max(5*measCycleSCell, 5*DRX cycles) for FR1, and [Y] equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 for FR2.

Proposal 2: For the values of [Y] for MAC CE based single SCell activation, it’s proposed that [Y] equals to max(5*measCycleSCell, 5*DRX cycles) for FR1, and [Y] equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 for FR2.

For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/ measReselectionValidityDuration-r18 is configured, it’s proposed to use existing SSB based known SCell activation delay requirement with 2 SSB samples for FR1, and to use existing SSB based known SCell activation delay requirement with 2 SSB samples and no need additional side condition for FR2.

Proposal 3: For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/ measReselectionValidityDuration-r18 is configured, it’s proposed to use existing SSB based known SCell activation delay requirement with 2 SSB samples for FR1, and to use existing SSB based known SCell activation delay requirement with 2 SSB samples and no need additional side condition for FR2.

Status in the WF [1]:

#### Issue 1-2-3: Consideration on measurement period condition of FR1 known SCell activation Proposals - Option 1: (CATT) - No need to update the measurement period condition for FR1 known SCell activation. - Option 2a: (Apple) - For fast SCell activation based on eEMR, time for AGC is always needed since measurement period in IDLE/INACTIVE mode is longer than 2400ms in most cases. - Option 2b: (Qualcomm, Nokia) - In any case, the SCell activation requirement should allow UE to receive at least one SSB sample based on [SSB period] or [configured SMTC periodicity]. - Option 3: (Samsung) - If the SCell is known based on the eEMR based known condition and belongs to FR1, 2400 ms can be modified to Y ms - The basic components to define Y and the subsequent impact on AGC training gate for activation shall be discussed.

For consideration on measurement period condition of FR1 known SCell activation, no need to update the measurement period condition for FR1 known SCell activation and legacy measurement period condition for FR1 known SCell activation can be reused.

Proposal 4: No need to update the measurement period condition for FR1 known SCell activation.

Status in the WF [1]:

#### Issue 1-2-4: Whether the indication to network is needed Proposals - Option 1: (CATT, Apple, Samsung) - Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation. - Option 2: (Ericsson, Nokia) - RAN4 to introduce a UE indication to the network. The fast or not definition can refer to the known/unknown condition that will be defined for fast Scell activation via EMR.

For whether the indication to network is needed, since the legacy SCell activation requirements don’t need additional indication to the network, we support that no need to introduce additional indication to the network for fast or slow SCell activation.

Proposal 5: Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation.

- Conclusion

The contribution presents our views on fast SCell activation for UE supporting Rel-18 EMR, with the following proposals:

Proposal 1: Considering the limited TU, it’s preferred that RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured.

Proposal 2: For the values of [Y] for MAC CE based single SCell activation, it’s proposed that [Y] equals to max(5*measCycleSCell,  5*DRX cycles) for FR1, and [Y] equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 for FR2.

Proposal 3: For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/ measReselectionValidityDuration-r18 is configured, it’s proposed to use existing SSB based known SCell activation delay requirement with 2 SSB samples for FR1, and to use existing SSB based known SCell activation delay requirement with 2 SSB samples and no need additional side condition for FR2.

Proposal 4: No need to update the measurement period condition for FR1 known SCell activation.

Proposal 5: Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation.

- References

- R4-2416854, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.