# 3GPP TSG-RAN WG4 Meeting #113                                                       R4-2418480

# Orlando, USA, 18th – 22rd November, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for RedCap NTN enhancement

Agenda item:	7.26.4.1

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN phase 3 [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. As this is the first meeting for RAN4 RRM to work on this WI, we will provide our initial views on potential impacts on RRM requirements.

Discussion

The objectives of Rel-19 NTN phase 3 enhancement are described in [1] including:

2.1 Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective. The objective will be divided into several aspects:

  - 2.1 Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

RAN4 shall define the RRM requirements based on the latest revised WID as below:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In above WID description, we know that RAN4 shall combine with R17/R18 RedCap WI and R17/R18 NR NTN WI to discuss this objective. Thus, we shall study the features introduced in RedCap which will impact the definition of requirements.

2.1.1 RRM requirements for (e)RedCap UE with FR1-NTN

In the last meeting, we have discussed the potential requirements one by one and three general cases shall be considered:

- Unique requirements for RedCap

- Unique requirements for NR NTN

- Shared requirements for RedCap and NR NTN

The concrete requirements can be seen as following list:

Table 1 Potential RRM requirements for RedCap UE in NTN scenario

| Unique requirements for RedCap | - Configured Grant based Small Data Transmission (CG-SDT) - Random access based Small Data Transmission (RA-SDT) |  |
| --- | --- | --- |
| Unique requirements for NR NTN | - NR SAN Satellite switching with re-synchronization - Pathloss reference signal switching delay - Network verified UE location - R17/18 Conditional Handover - RACH-less Handover - FFS: - Minimization of Derive Tests (MDT) in RRC Idle state and RRC Inactive state |  |
| Shared requirements for RedCap and NR NTN | RRC Idle/Inactive mobility | Cell re-selection |
|  | RRC Connection mobility control | - RACH based handover - RRC Re-establishment - Random Access - RRC connection release with redirection |
|  | Timing | - UE transmit timing - UE timer accuracy - Timing advance adjustment accuracy |
|  | Singnalling characteristics | - Radio Link Monitoring - Link Recovery Procedures - Active BWP switch delay - Active TCI state switching delay - UE-specific CBW change |
|  | Measurement procedure | - General measurement requirement - NR intra-frequency measurement - NR inter-frequency measurement - L1-RSRP measurement for Reporting |

Another point shall be considered also which are the enhanced features for RedCap UE and the following agreements concluded in then last meeting can be seen:

Issue 6-3-1: The principle for defining the requirements for 2Rx/1Rx (e)Redcap UE with FR1-NTN Online agreement <Agreement> - For RRM requirements, the number of Rx considered for (e)Redcap UE with FR1-NTN bands support include both 1Rx and 2Rx. - RAN4 should define separate sets of RRM requirements for 1Rx and 2 Rx (e)Redcap UE, if needed. - As baseline, follow the same principle for (e)RedCap UE in TN network. For 2Rx (e)RedCap UEs with FR1-NTN: Reuse the existing requirements for NTN as a baseline. Issue 6-4-1: The specific impact of HD-FDD for (e)Redcap UE with FR1-NTN Online agreement <Agreement> - RAN4 to consider the impact of HD-FDD for (e)Redcap UE with FR1-NTN. - The TN HD-FDD related requirements and applicable conditions defined for (e)RedCap UE can be used as a starting point, and RAN4 take into account NTN specific aspects if any.

Based on the above discussion, we shall study each requirements one by one with the RedCap UE feature and the details will be provided in this paper.

2.1.1.1 Potential impact on RRM requirements for RedCap UEs (1Rx and 2Rx)

First of all, we would like to analyze the potential RRM requirements with the reduced UE receiving branches. And the details can be seen as the following sections.

2.1.1.1.1 Shared requirements for RedCap and NTN

2.1.1.1.1.1 RRC Idle/Inactive mobility

Based on the RRM requirements for legacy normal UE, the potential R19 RedCap UEs in NR NTN mobility requirements in IDLE/INACTIVE may be:

Table 2 Potential cell re-selection requirements

| Scenario |  | Requirements | Has impact or not |
| --- | --- | --- | --- |
| TN-NTN | NR-NR | Inter-frequency cell re-selection | Yes |
|  | LTE-NR | Inter-RAT cell re-selection | RAN2 is discussing |
| NTN-TN | NR-NR | Inter-frequency cell re-selection and Inter-RAT cell re-selection | Yes |
|  | NR-LTE |  | Yes |
| NTN-NTN (earth-fixed) |  | Intra-frequency cell re-selection | Yes |
|  |  | Inter-frequency cell re-selection | Yes |
| NTN-NTN (earth-moving) |  | Intra-frequency cell re-selection | Yes |
|  |  | Inter-frequency cell re-selection | Yes |

Based on WID description “UEs with NR NTN operating in FR1-NTN bands”. We deem that the above scenarios shall be considered. Combined with discussion in RedCap WI, we shall specify the requirements on 1Rx and 2Rx. First of all, we shall focus on measurement capability on cell re-selection requirements for 1Rx and 2Rx UE in NTN.

- UE measurement capability for 1Rx and 2Rx RedCap

For legacy normal UE, the UE measurement capability can be seen :

For idle mode cell re-selection purposes, and for UE supporting idleInactiveNR-MeasReport-r16, idleInactiveEUTRA-MeasReport-r16 or idleInactiveNR-MeasReport-r17, for NR CA and MR-DC measurement purpose, the UE shall be capable of monitoring at least: - Intra-frequency carrier, and - Depending on UE capability, 7 NR inter-frequency carriers, and - Depending on UE capability, 7 FDD E-UTRA inter-RAT carriers, and - Depending on UE capability, 7 TDD E-UTRA inter-RAT carriers. In addition to the requirements defined above, a UE supporting E-UTRA measurements in RRC_IDLE state shall be capable of monitoring a total of at least 14 carrier frequency layers, which includes serving layer, comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD and NR layers.

For legacy RedCap 1Rx UE, the UE measurement capability can be seen:

For idle mode cell re-selection purposes, and for UE supporting IdleInactiveMeasurements-r16 or idleInactiveEUTRA-MeasReport-r16, the UE shall be capable of monitoring at least: - Intra-frequency carrier, and - Depending on UE capability, 6 NR inter-frequency carriers, and - Depending on UE capability, 6 FDD E-UTRA inter-RAT carriers, and - Depending on UE capability, 6 TDD E-UTRA inter-RAT carriers. In addition to the requirements defined above, a UE supporting E-UTRA measurements in RRC_IDLE state shall be capable of monitoring a total of at least 11 carrier frequency layers, which includes serving layer, comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD and NR layers.

For legacy RedCap 2Rx UE, the UE measurement capability is the same as legacy normal UE in TN.

For normal UE in legacy NTN, the UE measurement capability can be seen:

For idle mode cell re-selection purposes, the UE shall be capable of monitoring at least: - Intra-frequency carrier, and - Depending on UE capability, 7 NR inter-frequency carriers, and

We find that the legacy NTN UE and RedCap 2Rx UE measurement capability are the same as legacy TN UE measurement capability. From my perspective, 1Rx RedCap UE measurement capability in NTN shall be the same as legacy RedCap 1Rx UE., 2Rx RedCap UE measurement capability in NTN shall be same as legacy normal UE in NTN.

Proposal 1: 1Rx RedCap UE measurement capability in NTN shall be the same as legacy RedCap 1Rx UE.

Proposal 2: 2Rx redcap UE measurement capability in NTN shall be same as legacy normal UE in NTN.

- Measurements of  TN-NTN and NTN-TN for 1Rx and 2Rx RedCap UE

| TN-NTN | NR-NR | Inter-frequency cell re-selection | Yes |
| --- | --- | --- | --- |
|  | LTE-NR | Inter-RAT cell re-selection | RAN2 is discussing |
| NTN-TN | NR-NR | Inter-frequency cell re-selection and Inter-RAT cell re-selection | Yes |
|  | NR-LTE |  | Yes |

In R18 NR NTN WI, we define the cell re-selection requirements between TN and NTN including inter-RAT and inter-frequency measurements requirements. Thus, RAN4 shall define related requirements on TN-NTN (NR-NR) and NTN-TN(NR-NR/NR-LTE) for 1Rx and 2Rx RedCap UE. As for TN-NTN (LTE-NR), RAN2 is under discussion and there has been no RAN4 RRM impact so far.

Observation 1: RAN2 is discussing TN to NTN (LTE to NR) cell re-selection and there has been not RAN4 RRM impact so far.

Observation 2: RAN4 has already defined the inter-RAT and inter-frequency measurement requirements between TN and NTN in R18 NTN.

For measurement requirements in inter-frequency and inter-RAT, we mainly define the Tdetect/measure/evaluate, and the legacy measurement requirements can be seen as below:

TN to NTN (NR to NR): Table 4.2.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for TN DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). FR1; FR2-1Note1; FR2-2 Note2. 0.32; 1; 8; 12; 11.52 x N1 x 1.5 (36 x N1 x 1.5); 1.28 x N1 x 1.5 (4 x N1 x 1.5); 5.12 x N1 x 1.5 (16 x N1 x 1.5). 0.64; 5; 8; 17.92x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 6; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 5; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length.. Table 4.2C.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for NTN DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). FR1. 0.32; 1; 11.52 x N1 x 1.5 (36 x N1 x 1.5); 1.28 x N1 x 1.5 (4 x N1 x 1.5); 5.12 x N1 x 1.5 (16 x N1 x 1.5). 0.64; 17.92x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: UE is not required to fulfil the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.. Table 4.2C.2.4-2: Tdetect,NR_Inter_enh, Tmeasure,NR_Inter_enh and Tevaluate,NR_Inter_enh for NTN DRX cycle length [s]; Tdetect,NR_Inter_enh [s] (number of DRX cycles); Tmeasure,NR_Inter_enh [s] (number of DRX cycles); Tevaluate,NR_Inter_enh [s] (number of DRX cycles). 0.32; [3.2 x M2 (10 x M2)] Note 1; [0.32 x M3 ([1] x M3)] Note 1; 0.96 x M4 (3 x M4) Note 1. 0.64; [6.4 (10)]; [0.64 (1)]; 1.92 (3). 1.28; [10.24 (8)]; 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. NTN to TN (NR to NR): Same as TN to NTN. NTN to TN (NR to LTE): Table 4.2.2.5-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN DRX cycle length [s]; Tdetect,EUTRAN [s] (number of DRX cycles); Tmeasure,EUTRAN [s] (number of DRX cycles); Tevaluate,EUTRAN [s] (number of DRX cycles). 0.32; 11.52 (36); 1.28 (4); 5.12 (16). 0.64; 17.92 (28); 1.28 (2); 5.12 (8). 1.28; 32(25); 1.28 (1); 6.4 (5). 2.56; 58.88 (23); 2.56 (1); 7.68 (3).

From my perspective, the cell re-selection measurement requirements in legacy NTN can be reused for 2Rx and 1Rx RedCap UEs. However, RAN4 shall consider one point is the accuracy margin for 1Rx RedCap UE.

Based on the following table, we find that the extral 1dB for 1Rx RedCap UE is needed in inter-frequency measurements compared to legacy conditions. The main issues for this change is the  transmission rate and accuracy shift. Compared to 2Rx, 1Rx has lower transmission rate and the accuracy shift  is serious vary time, and RAN4 deems that the margin or threshold for 1Rx shall be higher than 2Rx and this can limit that two problems.

For inter-RAT measurement, we find that the accuracy margin for 1Rx and 2Rx is the same as each other. In the existing specification we find :

- The 1 Rx RedCap UE applies absThreshSS-BlocksConsolidation as the signaled value of absThreshSS-BlocksConsolidation [2] + 1 dB. - For inter-frequency in legacy RedCap TN The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS 38.304 [1] within Kcarrier_RedCap * Tdetect,NR_Inter_RedCap if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met For 2 Rx RedCap by a margin of at least [same as legacy TN/NTN] 5 dB in FR1 or 6.5 dB in FR2 for reselections based on ranking or 6 dB in FR1 or 7.5 dB in FR2 for SS-RSRP reselections based on absolute priorities or 4 dB in FR1 and 4 dB in FR2 for SS-RSRQ reselections based on absolute priorities For 1 Rx RedCap by a margin of at least [extra 1dB compared to 2Rx] 6 dB in FR1 for reselections based on ranking or 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities. - For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [1] within Kcarrier * Tevaluate,NR_Inter_RedCap when Treselection = 0 provided that the reselection criteria is met by - the condition when performing equal priority reselection and - when rangeToBestCell is not configured: - the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2 for 2 Rx RedCap. - the cell is at least 6dB better ranked in FR1 for 1 Rx RedCap. - when rangeToBestCell is configured: - the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value defined in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell. - if there are multiple such cells, the cell has the highest rank among them - the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2 if the current serving cell is among them, or 6dB in FR1 or 7.5dB in FR2 for SS-RSRP reselections based on absolute priorities for 2 Rx RedCap or 4dB in FR1 or 4dB in FR2 for SS-RSRQ reselections based on absolute priorities for 2 Rx RedCap. - the cell is at least 6dB better ranked in FR1 if the current serving cell is among them, or 7dB in FR1 for SS-RSRP reselections based on absolute priorities or 5dB in FR1 for SS-RSRQ reselections based on absolute priorities for 1 Rx RedCap. - For inter-RAT in legacy RedCap TN - The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS38.304 [1] within (NEUTRA_carrier_RedCap) * Tdetect,EUTRAN_RedCap when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria are met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 1 Rx RedCap. For a cell that has been already detected, but has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [1] within (NEUTRA_carrier_RedCap) * Tevaluate,EUTRAN_RedCap when Treselection = 0 provided that the reselection criteria are met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

Although the measurement requirements are same, the accuracy margin in inter-frequency shall add 1dB extra margin. For inter-RAT measurement, the accuracy margin shall be maintained.

Observation 3: Although the measurement requirements are same, the accuracy margin in inter-frequency measurement, 1dB margin is needed for 1Rx UE compared to legacy condition. For inter-RAT measurement, the accuracy margin is the same as original ones.

Proposal 3: The measurement requirements for inter-frequency and inter-RAT measurement requirements in legacy NTN shall be reused for 2Rx and 1Rx RedCap UEs when considering TN to  NTN (NR-NR) and NTN to TN (NR-NR).

Proposal 4: For inter-frequency measurement, RAN4 shall reuse the extra 1dB offset for 1Rx RedCap UE in NR NTN if the legacy requirements are reused.

Proposal 5: For inter-RAT measurement, the orginal accuracy margin shall be maintained for 1Rx RedCap UE in NR NTN if the legacy requirements are reused.

- Measurements of NTN-NTN for 1Rx and 2Rx RedCap UE

| NTN-NTN (earth-fixed) | Intra-frequency cell re-selection | Yes |
| --- | --- | --- |
|  | Inter-frequency cell re-selection | Yes |
| NTN-NTN (earth-moving) | Intra-frequency cell re-selection | Yes |
|  | Inter-frequency cell re-selection | Yes |

In R17, we mainly introduced the time-based and location-based condition for NTN to NTN cell re-selection. In R18, we just reuse the legacy NTN requirements and only introduced movingReferenceLocation and location-based margin from 50m to 80m based on RAN2 agreements because of earth-moving scenario. Thus, the legacy requirements shall be reused for 1Rx and 2Rx RedCap UEs.

Also, the accuracy margin shall also be studied for 1Rx and 2Rx RedCap UE when we considering to reuse the legacy NTN requirements. From my perspective,  the concrete conditions for 1Rx UE in inter-frequency measurement shall need 1dB extra margin based on legacy conditions in NTN and the legacy conditions in NTN shall be reused.

Proposal 6: The cell re-selection intra-frequency and inter-frequency measurement requirements in legacy NTN can be reused for 2Rx and 1Rx RedCap UEs when considering NTN to NTN cell re-selection.

Proposal 7: RAN4 shall define the concrete accuracy margin for 1Rx and 2Rx UE in NTN-NTN scenario. For 1Rx RedCap UE, additional 1dB is needed based on legacy conditions in NTN. For 2Rx RedCap UE, the legacy  conditions in NTN shall be reused.

2.1.1.1.1.2 RRC Connected mobility

Based on R17/R18 discussion, R19 NR NTN mobility requirements in CONNECTED state can be seen :

| NTN-NTN | RHCH-based handover |
| --- | --- |
|  | RRC Re-establishment |
|  | Random Access |
|  | RRC connection release with redirection |

As for mobility requirements in RRC CONNECTED state, we list the potential RRM requirements and we shall study them one by one.

- RACH-based handover

For RACH-based handover, RAN4 has already defined the FR1-FR1 handover delay for RedCap UE in TN and also NTN respectively. We can see the following table to see the difference:

Legacy RedCap RACH-based interruption time: The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay. When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where, Tsearch is the time required to search the target cell which depends on whether the target cell is already known when the handover command is received by the UE. - If the target cell is a known intra-frequency cell, Tsearch = 0ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 1*Trs ms if UE is capable of 2 Rx antennas; Tsearch = 2*Trs ms if UE is only required to support 1 Rx antenna. - If the target cell is a known inter-frequency cell, then if the measured SSB is the target SSB for HO of the target cell, Tsearch = 0ms; if the measured SSB and the target SSB for HO belong to the same NR target cell and fulfil the following conditions, Tsearch = Trs ms: - CD-SSB in DL BWP is the measured SSB and NCD-SSB in RedCap specific DL BWP is the target SSB for HO - NCD-SSB in RedCap specific DL BWP is the measured SSB and CD-SSB in DL BWP is the target SSB for HO - Both measured SSB and the target SSB for HO are NCD-SSBs within different RedCap specific DL BWPs Otherwise, the target cell is an unknown inter-frequency cell. In this case, if the target cell Es/Iot≥-2 dB, then Tsearch = 3* Trs ms if the UE is operating with 2 Rx antennas; Tsearch = 5*Trs ms if UE is operating with only 1 Rx antenna. Legacy NR NTN interruption time: When intra-frequency or inter-frequency handover to NR SAN cell is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Otherwise, no interruption time requirement is applied. Where, Tsearch is the time required to search the target NR SAN cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

For handover between NTN and NTN, we defined  the handover delay and specify the concrete parameters which are the same as legacy normal UE in TN. We also find that RAN4 has already defined the handover delay for RedCap UEs in TN scenario. As can be seen by comparison, the value of Tsearch is impacted by reduced UE Rx branches and the main difference is the values for 1Rx.  For 2Rx RedCap UEs, the values are same. From my perspective, the legacy Tsearch values for 1Rx UE shall be reused when we considering to define the handover delay for 1Rx RedCap UE in NTN. That is,

- If the target cell is a known intra-frequency cell, Tsearch = 0ms.

- If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 1*Trs ms if UE is capable of 2 Rx antennas; Tsearch = 2*Trs ms if UE is only required to support 1 Rx antenna.

- If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 3*Trs ms if UE is capable of 2 Rx antennas; Tsearch = 5*Trs ms if UE is only required to support 1 Rx antenna.

Observation 4: By comparison interruption time between legacy RedCap UEs in TN and normal UE in NTN, the  Tsearch is impacted by reduced UE Rx branches and the main difference is the values for 1Rx. For 2Rx RedCap UEs, the  Tsearch is same.

Proposal 8: RAN4 shall reuse the legacy 1Rx RedCap Tsearch when considering to define the handover delay for 1Rx RedCap UE in NTN:

- If the target cell is a known intra-frequency cell, Tsearch = 0ms.

- If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 1*Trs ms if UE is capable of 2 Rx antennas; Tsearch = 2*Trs ms if UE is only required to support 1 Rx antenna.

- If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 3*Trs ms if UE is capable of 2 Rx antennas; Tsearch = 5*Trs ms if UE is only required to support 1 Rx antenna.

- RRC Re-establishment

For RRC re-establishment requirements, we mainly focus on time to identify the target intra-frequency NR cell Tidentify_intra_NR and target inter-frequency NR cell Tidentify_inter_NR,i.  We can see the specific requirements as below:

Legacy normal UE in TN: Tidentify_intra_NR: It is the time to identify the target intra-frequency NR cell and it depends on whether the target NR cell is known cell or unknown cell and on the FR of the target NR cell. If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in Table 6.2.1.2.1-1 when highSpeedMeasFlagFR2-r17 is not configured or UE is not capable of FR2 power class 6 and Table 6.2.1.2.1-3 when highSpeedMeasFlagFR2-r17 is configured and UE is capable of FR2 power class 6. Tidentify_inter_NR,i: It is the time to identify the target inter-frequency NR cell on inter-frequency carrier i configured for RRC re-establishment and it depends on whether the target NR cell is known cell or unknown cell and on the FR of the target NR cell. Tidentify_inter_NR,i shall not exceed the values defined in Table 6.2.1.2.1-2. Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Legacy RedCap UE in TN: The requirements in clause 6.2.1 shall apply when RedCap UE is capable of 2 Rx. When UE is only required to support 1 Rx antenna, the requirements defined in clause 6.2.1 shall apply except that: - Tidentify_intra_NR as specified in Table 6.2.1B.2-1. - Tidentify_inter_NR, i as specified in Table 6.2.1B.2-2. Table 6.2.1B.2-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC); MAX (800 ms, [11] x TSMTC). < -8; FR1; N/A; 800Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1B.2-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 7 x TSMTC, i); MAX (800 ms, [14] x TSMTC, i). < -8; FR1; N/A; 800Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Legacy NR NTN: Tidentify_intra_NR: It is the time to identify the target intra-frequency NR cell and it depends on whether the target NR cell is known cell or unknown cell. If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in Table 6.2C.1.2.1-1. Tidentify_inter_NR,i: It is the time to identify the target inter-frequency NR cell on inter-frequency carrier i configured for RRC re-establishment and it depends on whether the target NR cell is known cell or unknown cell. Tidentify_inter_NR,i shall not exceed the values defined in Table 6.2C.1.2.1-2. Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); Kmulti_SMTC * MAX (800 ms, 10 x TSMTC). < -8; FR1; N/A; k * 800 ms Note1, 3. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: Kmulti_SMTC is defined in clause 9.2C.5.1. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt.. Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); K_satellite * MAX (800 ms, 13 x TSMTC, i). < -8; FR1; N/A; k * 800 ms Note1, 3. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: K_satellite is defined in clause 9.3C.4. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt..

Based on the above table, we find that the time to identify target NR intra-frequency and inter frequency cell for 2Rx is the same as legacy normal UE in TN and the time is also same for legacy NTN when  Ês/Iot ≥-8 dB. The main difference is the time for 1Rx RedCap UE. By comparison, we know that only the number of samples is increased (one sample increased). Thus, we deem that the legacy NTN time to identify target intra-frequency and inter-frequency cell shall be reused for 2Rx RedCap UE in NTN scenario. The legacy RedCap UE time to identify target intra-frequency and inter-frequency cell shall be reused for 1Rx RedCap UE in NTN scenario.

Observation 5: For 2Rx RedCap UE, the time to identify target NR intra-frequency and inter frequency cell is the same as legacy normal UE in TN and also for legacy NTN

Observation 6: For 1Rx RedCap UE, the time to identify target NR intra-frequency and inter frequency cell is different from 2Rx and only one measurement sample is increased.

Proposal 9: For 2Rx RedCap UE, RAN4 shall reuse legacy NTN requirements on time to identify intra-frequency and inter-frequency.

Proposal 10: For 1Rx RedCap UE, RAN4 shall reuse legacy RedCap UE requirements on time to identify intra-frequency and inter-frequency.

- RRC Connection Release with redirection

The UE shall be capable of performing the RRC connection release with redirection to the target NR cell within Tconnection_release_redirect_NR including  Tidentify-NR whcih  is the time to identify the target NR cell and depends on the FR of the target NR cell. The following table can be seen:

Legacy normal UE in TN: Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. Legacy RedCap UE in TN: The requirements in clause 6.2.3.2.1 shall apply when RedCap UE is capable of 2 Rx. When UE is only required to support 1 Rx antenna, the requirements defined in clause 6.2.3.2.1 shall apply except that: - Tidentify-NR as specified in Table 6.2.3A.2.1-1. Table 6.2.3A.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, [12] x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. Legacy NR NTN: Table 6.2C.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; K_satellite * MAX (680 ms, 11 x Trs). Note 1: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. Note 2: K_satellite is defined in clause 9.3C.4..

Actually, the same principle for RRC re-establishment can be considered when defining the requirements for RedCap UEs in NTN.

Proposal 11: For 2Rx UE in NTN, the time to identify target NR cell for RRC connection release with redirection to NR  in legacy NTN shall be reused.

Proposal 12: For 1Rx UE in NTN, the time to identify target NR cell for RRC connection release with redirection to NR  in legacy NTN shall be the baseline and one measurement sample shall be increased.

- Random Access

For random access, RAN4 mainly focus on 4-step and 2-step RA step with contention-based and non-contention based random access. Based on the analysis, we find that:

The 1 Rx RedCap UE for performing the random access procedure defined in clause 5.1 [7] applies: - rsrp-ThresholdSSB as the signaled value of rsrp-ThresholdSSB [2] + 1 dB. - msgA-RSRP-ThresholdSSB as the signaled value of msgA-RSRP-ThresholdSSB [2] + 1 dB. - msgA-RSRP-Threshold as the signaled value of msgA-RSRP-Threshold [2] + 1 dB. - rsrp-ThresholdMsg3 as the signaled value of rsrp-ThresholdMsg3 [2] - 1 dB.

We know that the requirements on RA for 1Rx RedCap UE is the same as 2Rx RedCap UEs or legacy normal UE. However, the conditions for performing the random access procedure are different. To my understanding, the legacy requirements in NR NTN shall be reused for 1Rx and  2Rx UE in NTN, the conditions for performing the random access procedure for 1Rx in legacy RedCap TN shall be used for 1Rx UE in NTN.

Proposal 13: The legacy requirements in NR NTN shall be reused for 1Rx and  2Rx UE in NTN, the conditions for performing the random access procedure for 1Rx in legacy RedCap TN shall be used for 1Rx UE in NTN.

2.1.1.1.1.3 Timing

Based on R17/R18 NR NTN discussion for normal UE, the timing requirements can be seen as below:

| Te, UE transmit timing | Yes |
| --- | --- |
| Gradual timing | Yes |
| Timing advance adjustment accuracy | Yes |
| Timing advance adjustment delay | Yes |

In R17, RAN4 mainly focused on the timing related issues because of the special NTN scenario, such as long propagation delay and frequency error etc. And UE TA pre-compensation has been discussed. However, the RedCap UE timing reused legacy timing requirements whatever the Te or TA adjustment delay or accuracy in RedCap WI. Thus, we deem that the timing requirements for redcap UE in NTN shall reuse the legacy timing requirements defined in NTN since the timing requirements for NTN is impacted by the time difference and the Doppler shift instead of the reduced UE receiving branches.

Proposal 14: The legacy NR NTN timing requirements shall be resued when considering to define the requirements on RedCap UEs in NTN scenario.

2.1.1.1.1.4 Signalling characteristics

- Radio Link Monitoring

Based on R17/R18 NR NTN discussion for normal UE, the RLM requirements can be seen as below:

| Signalling characteristics | Radio link monitoring (RLM) | Evaluation period |
| --- | --- | --- |

For RLM, the configured RLM-RSs can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. The configured RLM-RSs can be from the same or different TRPs. UE is not required to perform RLM outside the active DL active BWP in general. We shall focus on minimum requirement, UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB [ms] evaluation period.

For the evaluation period, the legacy requirements for normal UE in FR1 can be seen as below:

Table 8.1C.2.2-1: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1 Configuration; TEvaluate_out_SSB (ms); TEvaluate_in_SSB (ms). no DRX; Max(200, Ceil(10 ´ P) ´ TSSB); Max(100, Ceil(5 ´ P) ´ TSSB). DRX cycle≤320ms; Max(200, Ceil(15 ´ P) ´ Max(TDRX,TSSB)); Max(100, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)). DRX cycle>320ms; Ceil(10 ´ P) ´ TDRX; Ceil(5 ´ P) ´ TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length..

For legacy RedCap UE , the requirements can be seen as below:

Table 8.1B.2.2-1: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 2Rx RedCap UE Configuration; TEvaluate_out_SSB,RedCap (ms); TEvaluate_in_SSB,RedCap (ms). no DRX; Max(200, Ceil(10 P) TSSB); Max(100, Ceil(5 P) TSSB). DRX cycle≤320ms; Max(200, Ceil(15 P) Max(TDRX,TSSB)); Max(100, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle>320ms; Ceil(10 P) TDRX; Ceil(5 P) TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length.. Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE Configuration; TEvaluate_out_SSB,RedCap (ms); TEvaluate_in_SSB,RedCap (ms). no DRX; Max(400, Ceil(20 P) TSSB); Max(100, Ceil(5 P) TSSB). DRX cycle≤320ms; Max(400, Ceil(30 P) Max(TDRX,TSSB)); Max(100, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle>320ms; Ceil(20 P) TDRX; Ceil(5 P) TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length..

For legacy NR NTN, the minimum requirements can be seen as below:

Table 8.1C.2.2-1: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1 Configuration; TEvaluate_out_SSB (ms); TEvaluate_in_SSB (ms). no DRX; Max(200, Ceil(10 ´ P) ´ TSSB); Max(100, Ceil(5 ´ P) ´ TSSB). DRX cycle≤320ms; Max(200, Ceil(15 ´ P) ´ Max(TDRX,TSSB)); Max(100, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)). DRX cycle>320ms; Ceil(10 ´ P) ´ TDRX; Ceil(5 ´ P) ´ TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length..

We find that the legacy evaluation period for FR1in NTN is the same as the legacy TN in FR1.

Also in RedCap WI there is a agreement as below:

SSB-based RLM : evaluation period for Qout - The evaluation period of SSB based SINR is not extended for RLM Qout for RedCap UE with 2 Rx. SSB-based RLM : evaluation period for Qin - The evaluation period of SSB based SINR is not extended for RLM Qin for RedCap UE with 2 Rx. SSB-based RLM : evaluation period for Qout and Qin - Double the evaluation period for Qout - keep the legacy evaluation period for Qin CSI-RS-based RLM : evaluation period for Qout - The evaluation period of CSI-RS based SINR is not extended for RLM Qout for RedCap UE with 2 Rx. CSI-RS-based RLM : evaluation period for Qin - The evaluation period of CSI-RS based SINR is not extended for RLM Qin for RedCap UE with 2 Rx. CSI-RS-based RLM : evaluation period for Qout and Qin Follow the agreement from corresponding issue for SSB-based RLM evaluation period. Whether to extend the lower bound in RLM evaluation period: - lower bound in RLM evaluation period is extended. The lower bound for SSB-based and CSI-RS based RLM Qout evaluation period for FR1 is extended by factor 2. The lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 is not extended.

In RedCap WI, it also discussed the SSB based and CSI-RS RLM requirements about two evaluation period for 1Rx and 2Rx. For 2Rx, the evaluation period is not extended whatever evaluation period for Qout or Qin. For 1Rx, RAN4 doubles the evaluation period for Qout and keep the legacy evaluation period for Qin. We can also find this with highlight in the table above. Thus, when considering defining the RLM requirements for RedCap UE in NTN scenario, we shall also keep the evaluation period as legacy for 2Rx and the lower bound shall be extended for 1Rx. To my understanding, the extended factor can be reuse the legacy factor which is 2.

Observation 7: The legacy NR NTN evaluation period is the same as legacy TN evaluation period.

Observation 8: In RedCap WI, the evaluation period will not change for 2Rx and the evaluation period of 1Rx is extended by factor 2.

Proposal 15: RAN4 shall define the Qout requirements for RedCap UEs in FR1-NTN bands. The evaluation period for 2Rx shall be reused and the evaluation period shall be extended for 1Rx by factor 2. Also, Qin shall be reused.

- Link Recovery Procedures

Based on R17/R18 NR NTN discussion for normal UE, the BFD/CBD requirements can be seen as below:

| Signalling characteristics | Link Recovery Procedures | Evaluation period for BFD and CBD |
| --- | --- | --- |

Same as the analysis of RLM, RAN4 has already analyzed requirements of SSB-based and CSI-based BFD/CBD, we can see the concrete legacy SSB-based BFD/CBD requirements as below (SSB-based can be an example):

Legacy normal UE in TN: BFD: Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1 Configuration; TEvaluate_BFD_SSB (ms). no DRX; Max(50, Ceil(5 P) TSSB). DRX cycle ≤ 320ms; Max(50, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle > 320ms; Ceil(5 P) TDRX. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. CBD: Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1 Configuration; TEvaluate_CBD_SSB (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(3 P PCBD) TSSB). DRX cycle > 320ms; Ceil(3 P PCBD) TDRX. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. Legacy RedCap UE in TN: BFD: Table 8.5B.2.2-1: Evaluation period TEvaluate_BFD_SSB_Redcap for FR1 Configuration; TEvaluate_BFD_SSB_Redcap (ms) for Redcap UE with 2 Rx; TEvaluate_BFD_SSB_Redcap (ms) for 1 Rx Redcap. no DRX; Max(50, Ceil(5 P) TSSB); [ Max(50, Ceil(10 P) TSSB)]. DRX cycle ≤ 320ms; Max(50, Ceil(7.5 P) Max(TDRX,TSSB)); [ Max(50, Ceil(15 P) Max(TDRX,TSSB)) ]. DRX cycle > 320ms; Ceil(5 P) TDRX; [ Ceil(10 P) TDRX]. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. CBD: Table 8.5B.5.2-1: Evaluation period TEvaluate_CBD_SSB_Redcap for FR1 Configuration; TEvaluate_CBD_SSB_Redcap (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(3 P PCBD) TSSB). DRX cycle > 320ms; Ceil(3 P PCBD) TDRX. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. Legacy normal UE in NTN: BFD: Table 8.5C.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1 Configuration; TEvaluate_BFD_SSB (ms). no DRX; Max(50, Ceil(5 ´ P) ´ TSSB). DRX cycle ≤ 320ms; Max(50, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)). DRX cycle > 320ms; Ceil(5 ´ P) ´ TDRX. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. CBD: Table 8.5C.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1 Configuration; TEvaluate_CBD_SSB (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(3 ´ P ´ PCBD) ´ TSSB). DRX cycle > 320ms; Ceil(3 ´ P ´ PCBD) ´ TDRX. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length..

We find that the scaling factor is also 2 which is same as RLM for 1Rx UE BFD evaluation period. And the same BFD evaluation period for 2Rx UE as legacy. However, when defining the CBD evaluation period, there is no difference between 1Rx and 2Rx RedCap UEs.

Observation 9: When defining the CBD evaluation period, there is no difference between 1Rx and 2Rx RedCap UEs.

Proposal 16: The evaluation period of BFD in legacy RedCap for 2Rx shall be reused and the evaluation period of BFD in legacy RedCap shall be extended for 1Rx by factor 2.

Proposal 17: The evaluation period of CBD in legacy NTN shall be used without any change.

- Active BWP switch delay

For active BWP switching which applies for a UE configured with more than one BWP on PCell. RAN4 has already defined the related active BWP switching delay for RedCap UEs in TN and normal UEs in NTN. We know that there are three methods to perform the active BWP switching, which are DCI-based switch, timer-based switching and RRC-based switching. Different methods own different active BWP switching delay.

However, what we need to do is that how to define the requirements for RedCap UEs in NTN scenario. We list the different legacy requirements as below:

Legacy RedCap UEs in TN: DCI-based BWP switching delay: TBWPswitchDelay + Y which starts from the beginning of DL slot n Timer-based BWP switching delay: TBWPswitchDelay Table 8.6A.2-1: BWP switch delay NR Slot length; BWP switch delay TBWPswitchDelay (slots). (ms); Type 1Note 1; Type 2Note 1. 0; 1; 1; 3. 1; 0.5; 2; 5. 2; 0.25; 3; 9. 3; 0.125; 6; 18. Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch.. RRC-based BWP switching delay: TRRCprocessingDelay+TBWPswitcℎDelayRRCNR Slot lengtℎ slots which begins from the beginning of DL slot n Legacy normal UEs in NTN: DCI-based BWP switching delay: TBWPswitchDelay + Y which starts from the beginning of DL slot n Timer-based BWP switching delay: TBWPswitchDelay Table 8.6C.2-1: BWP switch delay NR Slot length; BWP switch delay TBWPswitchDelay (slots). (ms); Type 1Note 1; Type 2Note 1. 0; 1; 1; 3. 1; 0.5; 2; 5. 2; 0.25; 3; 9. Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch.. RRC-based BWP switching delay: TRRCprocessingDelay+TBWPswitcℎDelayRRCNR Slot lengtℎ slots which begins from the beginning of DL slot n

We find that the active BWP switching delay will not be impacted by reduced UE receiving branches and only depends on UE capability or different NR slot length. For NTN scenario, the maximum slot length is 0.25ms refers to SCS=60kHz in FR1. For slot length is 0.125ms for RedCap UEs in TN, the SCS is 120kHz belongs to FR2 which is out of scope when we define the requirements for RedCap UEs in NTN scenario. To my understanding, legacy BWP switching delay requirements in NTN shall be reused for RedCap UEs in NTN scenario. By the way, RAN4 shall only study active BWP switch delay on a single CC instead of multiple CCs since CA is not supported in NTN scenario.

Observation 10:  Active BWP switching delay will not be impacted by reduced UE receiving branches and only depends on UE capability or different NR slot length.

Observation 11: For NTN scenario, the maximum slot length is 0.25ms refers to SCS=60kHz in FR1. For slot length is 0.125ms for RedCap UEs in TN, the SCS is 120kHz belongs to FR2 which is out of scope when we define the requirements for RedCap UEs in NTN scenario.

Proposal 18: Legacy BWP switching delay requirements in NTN shall be reused for RedCap UEs in NTN scenario.

Proposal 19: RAN4 shall only study active BWP switch delay on a single CC instead of multiple CCs since CA is not supported in NTN scenario.

- Active TCI state switch delay and UE-specific CBW change

We find that the active TCI state switch delay and UE specific CBW change will not change due to the reduced UE receiving branches. Thus, we deem that the legacy NTN requirements shall be reused for RedCap UEs in NTN scenario when we considering to define the active TCI state switch delay and UE specific CBW change.

Proposal 20:  For active TCI state switch delay and UE specific CBW change, RAN4 shall reuse the legacy NTN requirements for RedCap UEs in NTN scenario.

2.1.1.1.1.5 Measurement Procedure

Based on R17/R18 NR NTN discussion for normal UE, the measurement related requirements can be seen as below:

| Measurement | Intra-frequency measurement | Intra-frequency cell identification and measurement period |
| --- | --- | --- |
|  | Inter-frequency measurement | Inter-frequency cell identification and measurement period |
| Scenario | Multiple SMTC | Based on time and frequency difference |

For defining measurement requirements, we mainly focus on the intra-frequency and inter-frequency measurement especially for cell identification and measurement period. However, RAN4 shall clarify the basic scenario for redcap UE in NTN before studying the related requirements.

Multiple SMTCs

In legacy, RAN2 introduced multiple SMTC based on time and frequency difference. Although we are studying the redcap UE in NTN scenario, the multiple SMTCs shall also be reused since in my opinion, this is related to the NTN scenario not the types of UE. For legacy NTN, 2 SMTCs are mandatory supported and 4 SMTCs are optionally supported based on the UE capability. However, considering the complexity limitation of RedCap UEs, we deem that more SMTCs maybe are not proper for them. Thus, to my understanding, 2 SMTCs are enough for RedCap UEs.

Observation 12: RAN2 introduced the multiple SMTCs based on time and frequency difference in NTN in R17.

Proposal 21: Multiple SMTCs (2 SMTCs) shall be reused for RedCap UEs in NTN scenario.

Cell identification

In this section, we mainly discuss the inter-frequency measurement without measurement gaps as an example.

PSS/SSS detection and time index detection

UE performs cell identification including the time period of PSS/SSS detection and time index detection. The requirements are defined as below:

Legacy normal UE： Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra. DRX cycle≤ 320ms; max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(5 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1.. Table 9.2.5.1-3: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; Ceil(3 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. RedCap UE: Table 9.2B.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) for 2Rx RedCap UE DRX cycle; TPSS/SSS_sync_intra_RedCap. No DRX; max(600ms, ceil(5 x Kp) x SMTC period )Note 1 x CSSFintra_RedCap. DRX cycle≤ 320ms; max(600ms, ceil(1.5 x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap. DRX cycle>320ms; ceil(5 x Kp) x DRX cycle x CSSFintra_RedCap. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.2B.5.1-3: Time period for PSS/SSS detection, (Frequency range FR1) for 1Rx RedCap UE DRX cycle; TPSS/SSS_sync_intra_RedCap. No DRX; max(600ms, ceil( 7 x Kp) x SMTC period )Note 1 x CSSFintra_RedCap. DRX cycle≤ 320ms; max(600ms, ceil(1.5 x 7 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap. DRX cycle>320ms; ceil(7 x Kp) x DRX cycle x CSSFintra_RedCap. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.2B.5.1-4: Time period for time index detection (FR1) for 2Rx RedCap UE DRX cycle; TSSB_time_index_intra_RedCap. No DRX; max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra_RedCap. DRX cycle≤ 320ms; max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap. DRX cycle>320ms; Ceil(3 x Kp) x DRX cycle x CSSFintra_RedCap. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.2B.5.1-5: Time period for time index detection (FR1) for 1Rx RedCap UE DRX cycle; TSSB_time_index_intra_RedCap. No DRX; max(160ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFintra_RedCap. DRX cycle≤ 320ms; max(160ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap. DRX cycle>320ms; Ceil(6 x Kp) x DRX cycle x CSSFintra_RedCap. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. NTN scenario： Table 9.2C.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; max( 600ms, ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period )Note 1 x CSSFintra. DRX cycle≤ 320ms; max( 600ms, ceil(1.5 x 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.2C.5.1-2: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil (1.5 x 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; Ceil(3 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

Based on the table in above, we find that the legacy NR NTN requirements and the legacy  requirements for 2Rx RedCap UE are the same as legacy requirements. For 1Rx RedCap UE, the time period of PSS/SSS detection and time index detection are relaxed and highlight can be seen in the table.

From my perspective, the time period of PSS/SSS detection and time index detection can be reused for 2Rx RedCap UE in NTN. For 1Rx RedCap UE in NTN, the legacy time period for 1Rx RedCap UE can be baseline.

Observation 13: The legacy NR NTN requirements and the legacy requirements for  2Rx RedCap UE are the same as legacy requirements. For 1Rx RedCap UE, the time period are relaxed.

Proposal 22: RAN4 shall define time period of PSS/SSS detection and time index detection. The requirements for 1Rx and 2Rx RedCap UE in NTN scenario shall reuse the legacy RedCap requirements.

Measurement period (intra-frequency measurement without measurement gaps)

Based on R17/R18 discussion, we also discussed the legacy TN/ legacy RedCap UE/legacy NR NTN measurement requirements, and we find that the requirements are same in TS 38.133. Thus, to my understanding, the RedCap UEs with 1Rx and 2Rx in NTN shall own the same measurement period requirements as legacy RedCap UE.

Observation 14：The measurement period requirements for legacy TN/ legacy RedCap UE/legacy NR NTN are same in TS 38.133.

Proposal 23: The RedCap UEs with 1Rx and 2Rx in NTN shall own the same measurement period requirements as legacy RedCap UE.

2.1.1.1.2 Unique requirements for NTN

In legacy, RAN4 has already studied several unique requirements for NTN based on its special characteristics. In the last meeting, we also agreed several potential RRM requirements shall be studied for RedCap UEs in NTN scenario. And the following potential requirements agreed in last meeting can be seen as below:

| Satellite switch with re-synchronization | Hard / Soft switching delay |
| --- | --- |
| Pathloss reference signal switching delay | MAC-CE based pathloss reference signal switch delay |

Also, the following aspects in the below table shall firstly study the feasibility and then study the related RRM requirements:

FFS: - R17/18 Conditional Handover - RACH-less Handover - Minimization of Derive Tests (MDT) in RRC Idle state and RRC Inactive state

Next, we will also study each requirements one by one.

2.1.2.1.2.1 Satellite switch with re-synchronization

For satellite switch, this is NTN specific requirement that we have hard satellite switch and soft satellite switch.

The purpose of NR SAN Satellite switching with re-synchronization is to switch satellite in the same SSB frequency and same gNB without PCI changing. The requirements in this clause are applicable to SA NR SAN. The requirements in this clause are applicable to both hard and soft switch over in quasi-earth fixed scenario from NR SAN FR1 cell to NR SAN FR1 cell.

For hard satellite switch, when the UE switches from the current serving satellite to a new satellite, it first disconnects from the original satellite before establishing a connection with the new one.

For soft satellite switch, it allows the UE to maintain connections with multiple satellites simultaneously. During the handover process, the UE first establishes a communication link with the target satellite and only disconnects from the original satellite after ensuring the new connection is stable.

- Hard satellite switching delay

For hard satellite switch delay, the main delay can be seen as below:

Figure1  Hard satellite switch delay

The interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

First of all, we find that the interruption time in hard satellite switch delay is nearly same as the RedCap UEs handover requirements. We can see the each component of interruption time in legacy hard satellite switch delay defined in existing spec:

Legacy Satellite hard switching interruption time: the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Otherwise, no interruption time requirement is applied. Where: - Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when UE starts synchronizing with target satellite. If the target cell Es/Iot ≥ -2 dB, then Tsearch = [Tfirst_SSB] ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - T∆ is same as the one defined in section 6.1C.2.2.2.1. - Tprocessing is time for UE processing. Tprocessing can be up to 10 ms. - Tmargin is time for SSB post-processing. Tmargin can be up to 2ms. - TIU is the interruption uncertainty in acquiring the first UL transmission resource, which can be a configured grant based PUSCH, dynamic grant based PUSCH, SR on PUCCH, according to NW configuration and scheduling, or PRACH if TA timer is not running and there is no PUCCH SR - Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE.

We know that the Tsearch will be impacted by reduced UE receiving branches. From my perspective, if the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2dB, then Tsearch =1*Tfirst_ssb ms if UE is capable of  2Rx antennas; Tsearch=2*Tfirst_ssb ms if UE is only required to support 1Rx antenna.

Observation 15: The hard satellite switching delay will be impacted by reduced UE receiving branches.

Proposal 24: Considering hard satellite switching delay, Tsearch will not be changed which is 1*Tfirst_ssb ms for 2Rx RedCap UEs. For 1Rx RedCap UEs, Tsearch will be 2*Tfirst_ssb ms.

- Soft satellite switching delay

For soft satellite switch delay, the main delay can be seen as below:

Figure 2 Soft satellite switch delay

The soft satellite switch time shall be less than Tsoft_switch

Tsoft_switch = max(t-service-t-seviceStart, Tsearch + T∆ + Tmargin) + TIU + Tprocessing  ms

We know that the soft satellite switch delay includes two parts: Sync to target satellite and the interruption. For the first part, it includes full or partial Tsearch + T∆ + Tmargin or t-service-t-serviceStart. For Tsearch + T∆ + Tmargin, if the delay of Tsearch + T∆ + Tmargin  is larger than t-service-t-serviceStart, the interruption will also contain partial Tsearch + T∆ + Tmargin  and the soft satellite switch delay will be much longer than the case which the delay of Tsearch + T∆ + Tmargin  is smaller than t-service-t-serviceStart. Tsearch is also involved in soft satellite switch delay. To my understanding, the same method in hard satellite switch shall be used.

Proposal 25: Considering soft satellite switching delay, Tsearch will not be changed which is 1*Tfirst_ssb ms for 2Rx RedCap UEs. For 1Rx RedCap UEs, Tsearch will be 2*Tfirst_ssb ms.

2.1.1.1.2.2 Pathloss reference signal switching delay

For pathloss reference signal switch, this is the RS designated by the network for the UE to update its channel pathloss. For example, when a UE moves from the edge of current cell to a new cell, the power of signal and quality will change.To ensure the UE can adapt the new channel condition, the network will send the MAC-CE activation message to notify UE that the RS shall be switched. The UE is required to complete the switch within a specific window after receiving this message and commence the pathloss estimation using the new reference signal. However, the specific window is actually the pathloss reference signal switching delay. If the target pathloss reference signal is known, upon receiving PDSCH carrying MAC-CE activation in slot n, UE shall be able to apply the target pathloss reference signal of the serving cell on which pathloss reference signal switch occurs no later than the slot n +  THARQ+3Nslotsubframe,µ +  NM∗  5∗Ttarget_PL−RS + 2msNR slot lengtℎ.  The UE shall be able to apply old pathloss reference signals until the slot n + THARQ+ 3Nslotsubframe,µ.

However, we think there is no impact on RedCap UEs in NTN scenario since each component is not impacted by reduced UE receiving branches.

Proposal 26: RAN4 shall reuse the legacy passloss reference signal switching delay in NTN for 1Rx and 2Rx RedCap UEs in NTN scenario.

2.1.1.1.2.3 RACH-less handover

For RACH-less handover compared to RACH-based handover, the main difference is the end point of the handover delay. For RACH-based handover delay, the end point is to start the transmission of the new uplink PRACH. For RACH-less handover delay, the end point is to start the transmission of the new uplink PUSCH. The interruption time is the same as each other. Thus, we deem that the RACH-less handover delay is impacted by reduced UE receiving branches.

Observation 16: For RACH-less handover compared to RACH-based handover, the main difference is the end point of the handover delay. For RACH-based handover delay, the end point is to start the transmission of the new uplink PRACH. For RACH-less handover delay, the end point is to start the transmission of the new uplink PUSCH. The interruption time is the same as each other.

Proposal 27: If RAN4 defines the RACH-based handover requirements for RedCap UEs in NTN, RAN4 shall reuse the same requirements for RACH-less handover requirements for RedCap UEs in NTN.

2.1.1.1.2.4 Conditional handover

For conditional handover with and without L3 measurement, we find that RAN4 has not defined the requirements for RedCap UEs in TN in legacy. RAN4 has already defined the conditional handover requirements for normal UEs in NTN scenario. The condition can be CondEvent D1 and CondEvent T1 etc. However, RAN4 is discussing how to define the requirements for RedCap UEs in NTN scenario. We shall based on the following one or two aspects to analyze this requirement.

How to define the conditional handover for RedCap UEs in NTN

Firstly, the legacy conditional handover with L3 measurement requirements in NTN can be the baseline. Also, we shall consider the reduction of the UE receiving branches. Combined with these two aspects, the concrete requirements can be decided.

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Based on the existing conditional handover delay, only Tmeasure will be impacted by reduced UE receiving branches. Tmeasure is the measurement time delay for identifying a new detectable intra-/inter-frequency cell which is defined in intra-/inter-frequency measurement. Thus, we deem that the legacy time period for PSS/SSS detection and time period for time index detection requirements for 1Rx and 2Rx RedCap UEs shall be considered.

Observation 17: Only Tmeasure is impacted by reduced UE receiving branches.

Proposal 28: For conditional handover with L3 measurement, the legacy time period for PSS/SSS detection and time period for time index detection requirements for 1Rx and 2Rx RedCap UEs shall be considered.

For conditional handover without L3 measurement, we do not see the updates and the legacy requirements can be considered.

Proposal 29: For conditional handover without L3 measurement, the legacy NTN requirements could be considered.

2.1.1.1.2.5 Minimization of Derive Tests (MDT) in RRC Idle state and RRC Inactive state

For minimization of derive tests, RAN4 mainly focus on the logged MDT which is different legacy derive tests. The general procedure can be seen as below:

Figure  MDT procedure

And for this part there are three parts shall be considered:

- Logging measurements in RRC_IDLE state, reporting the logged measurement;

- Logging of RRC connection establishment failure, reporting the logged failure;

- Logging of radio link failure and handover failure, reporting the logged failure.

For the logged MDT requirements and RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirements, among them the relative time stamp has different time duration which is values in seconds.

To my  understanding, RAN4 shall define the related timing accuracy requirements. However, the requirements will not be impacted by the reduced UE receiving branches since the accuracy of the relative time stamping is the drift of the time stamping. Whatever it’s the 1Rx UE or 2Rx UE, if the drift of time stamping for each measurement data collection is not more than the seconds in legacy, the requirements can be applied. Since the differences between 1Rx UE and 2Rx UE in terms of timestamp recording accuracy are primarily determined by their internal hardware configurations, not directly by the number of antennas.

However, we see majority companies do not have strong view to define the related requirements for MDT since the RedCap UEs have low complexity and low cost. Few companies deem that the cell size is large enough for NTN and MDT requirements have been defined in legacy NTN. Thus, MDT shall be defined. However, several companies think that normal UEs can also perform MDT and it is not necessary to study MDT for RedCap UEs. We do not have the strong view on this.

Observation 18: For the logged MDT requirements and RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirements, among them the relative time stamp has different time duration which is values in seconds.

Observation 19: Majority companies do not have strong view to define the related requirements for MDT since the RedCap UEs have low complexity and low cost. Few companies deem that the cell size is large enough for NTN and MDT requirements have been defined in legacy NTN. Thus, MDT shall be defined. However, several companies think that normal UEs can also perform MDT and it is not necessary to study MDT for RedCap UEs. We do not have the strong view on this.

Proposal 30: RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

2.1.1.1.3 Unique requirements for RedCap

In the last meeting, we just discussed the requirements only for RedCap which can be seen as below:

- Configured Grant based Small Data Transmission (CG-SDT) - Random Access based Small Data Transmission (RA-SDT)

For unique requirements for RedCap, we will analyze its feasibility for RedCap UEs in NTN scenario and then decide to whether define the related RRM requirements or not.

2.1.1.1.3.1 Configured Grant based Small Data Transmission (CG-SDT)

For CG-SDT, it likes the LTE PUR which does not need the PRACH process in order to reduce the overhead of the uplink resources and reduce the power consumption especially for the small data transmission.

Based on the existing specification, the UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the small data transmission. Thus, the most important thing is the timing synchronization. RAN4 has already defined the requirements to perform the TA validation based on the RSRP change. When UE is configured with the CG-SDT parameters, UE will store the original RSRP value which is the measurement reference RSRP1. Then UE will select SSB which is above the cg-SDT-RSRP-ThresholdSSB or has the same SSB index as the SSB selected for initial transmission for CG-SDT. Based on the selected SSB, UE will measure the current RSRP value of the downlink pathloss reference and than the change between the stored RSRP and the RSRP for current DL pathloss reference will be calculated. If the change is within the threshold, the TA is valid; otherwise, UE will drop the CG-SDT. The current RedCap UEs’ requirement is as below:

Table 5.5.3-1 Valid measurement for FR1 for UE not configurd with eDRX cycle

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)) |
| RSRP2 | (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2 |

We see that the SDT maybe also apply for the NTN scenario which is not the corner case. However, if we plan to define the requirements in NTN, GSO and NGSO shall be distinguished.

For NTN GSO scenario, if UE receives the CG-SDT configuration, UE shall store the latest RSRP value, then UE will select the proper SSB to measure the current DL RSRP, the RSRP change will be smaller since the position of GSO satellite is stable from UE perspective and UE would not have the large mobility. The legacy RedCap UEs’ RSRP measurement can be the baseline.

For NTN NGSO scenario, the main difference compared to GSO is the velocity of the satellite. In theory, the RSRP change will be very large for NGSO and the TA will be invalid based on RSRP change which will exceed the RSRP change threshold.  However, based on RAN2’s understanding we find the threshold of RSRP change can be up to 34dB:

We think this threshold is relaxed enough and also there are 5bits spared. Thus, we support the CG-SDT shall be studied in NGSO scenario. The  valid measurement for FR1 in NGSO shall be the same as in GSO.

The next point shall be also considered in NTN scenario which is the SDT timer, the TA validation shall be verified before the timer expiration.

Above all, the final valid measurement for FR1 can be:

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)) |
| RSRP2 | (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2 |
| Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |

For 1Rx and 2Rx RedCap UEs, the requirements are same only the accuracy margin is different:

The 1 Rx RedCap UE for determining whether to perform SDT procedure defined in clause 5.27 [7] applies: - sdt-RSRP-Threshold-r17 as the signaled value of sdt-RSRP-Threshold-r17 [2] + 1 dB. sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated

We shall also reuse the margin as legacy.

Observation 20: The UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission.

Observation 21: For 1Rx and 2Rx RedCap UEs, the requirements are same only the accuracy margin is different.

Observation 22: Based on RAN2’s understanding,  the threshold of RSRP change can be up to 34dB.

Proposal 31: CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be:

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)) |
| RSRP2 | (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2 |
| Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |

Proposal 32: Compared to 2Rx UE, extra 1dB margin shall be needed for 1Rx RedCap UEs.

2.1.1.1.3.2 Random access based Small Data Transmission (RA-SDT)

For random access based small data transmission, we find the legacy SDT requirements is the same as the legacy RA requirements for 1Rx and 2Rx UEs. Compared to 2Rx UE, extra 1dB margin shall be needed for 1Rx RedCap UEs. Thus, if the random access for 2Rx and 1Rx RedCap requirements are defined, the RA based SDT requirements shall be reused. And the accuracy margin for 1Rx can be reused.

Observation 23: The legacy SDT requirements is the same as the legacy RA requirements for 1Rx and 2Rx UEs

Proposal 33: If the random access for 2Rx and 1Rx RedCap requirements are defined, the RA based SDT requirements shall be reused. The extra 1dB accuracy margin for 1Rx can be reused.

2.1.1.2 Potential impact on RRM requirements for RedCap UE (HD-FDD UE)

The second feature is half-duplex FDD operation. Compared to FD-FDD UE, HD-FDD operates by using separate frequency bands for uplink and downlink communication, but with a UE only being able to either transmit or receive at any given time – not both simultaneously. In RedCap WI, they discussed the several requirements for HD-FDD UE and define lots of applicable conditions not the new requirements. We can see the handover requirements as below:

Handover: Legacy RedCap HD-UE： For RedCap UE with HD-FDD, the handover requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU.

We find that the legacy handover requirements can be met for HD-FDD UE, however, the measurement applicable conditions may be defined for HD-FDD operation. Thus, when we considering defining the requirements for RedCap HD-FDD UE in NTN scenario, the legacy requirements can be the baseline and the applicable condition can refer to the legacy conditions.

Proposal 34: The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario.

2.1.1.3 Other features

For this section, RAN4 is still discussing whether to consider or not on:

- eDRX for LEO

- Relaxation measurement in Idle/Inactive state

- Concurrent gap

We will study the above features one by one in following contents.

2.1.1.3.1 eDRX for LEO

In the last meeting, for eDRX configuration RAN4 reached the following agreements:

<Agreement>: - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO. For LEO scenario (in chairman notes): - For LEO earth moving: - Option 1: Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN, and further discuss the applicable eDRX cycle. (E///) - Option 2: Not define eDRX related requirements in RRC_Idle / RRC_Inactive mode (ZTE, CATT, LGE, vivo, Xiaomi, Samsung, Apple) - For LEO earth fixed: - Option 1: Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN, and further discuss the applicable eDRX cycle. (E///, vivo, Xiaomi, Samsung, CMCC, Apple, LGE) - Option 2: Not define eDRX related requirements in RRC_Idle / RRC_Inactive mode (ZTE, CATT)

In order to reach the power saving gain, RAN4 firstly agreed the eDRX can be configured in GSO scenario. However, the LEO scenario can be divided into two parts: LEO earth-moving and LEO quasi earth-fixed.

For LEO earth-moving cell, we reached one conclusion that 2.56s DRX is not suitable for earth-moving scenario in R17. Thus, the eDRX shall not be configured for earth-moving scenario.

Observation 24: In legacy, 2.56s DRX is not suitable for earth-moving scenario.

Proposal 35: For LEO earth-moving scenario, eDRX shall not be configured for LEO earth-moving cell.

For LEO quasi earth-fixed scenario,  we obtain the information that the LEO quasi earth-fixed serving time can be up to 10min based on the lowest evaluation angle LEO 30°. Some companies provide the other values like 5min,7min etc. Thus, the serving time may be from 5min to 10min. From my perspective, the eRDX could be configured for LEO quasi earth-fixed cell and the value can be up to 10.24s (Tdetect=235.52s).

Observation 25: The serving time may be from 5min to 10min for  LEO quasi earth-fixed cell.

Proposal 36: The eRDX could be configured for LEO quasi earth-fixed cell and the value can be up to 10.24s (Tdetect=235.52s).

2.1.1.3.2 Relaxation measurement in Idle/Inactive state

In the last meeting, RAN4 discussed the relaxation measurement for RedCap UE in FR1 NTN scenario and the following agreements can be seen:

Online agreement <Agreement>: - FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline. - If defined, the requirements apply provided that target cell is GSO

For the first part, whether the R16 relaxations shall be introduced or not. To my understanding, this part shall be introduced since R16 relaxation in GEO have been introduced in NR NTN for normal UE, so it is reasonable to apply for (e)RedCap in NTN.

For the second part, whether the R17 RedCap relaxed measurement requirements shall be introduced or not.

2.1.1.3.3 Concurrent gap

Concurrent gap is defined for normal UE in NTN scenario and there is no RRM impact for RedCap UEs. From my perspective, RedCap UEs have low complexity and low capability, if network configures the concurrent gap for RedCap UE, it has large probability will not support more than two gaps. Some companies mentioned that the concurrent gaps is optional not the mandatory. However, we do not think it has significance for defining such requirements based on low capability even if RAN4 defines the related requirements.

Proposal 38: RAN4 had better not consider concurrent gaps for RedCap UEs in NTN scenario.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR RedCap NTN .

Proposal 1: 1Rx RedCap UE measurement capability in NTN shall be the same as legacy RedCap 1Rx UE.

Proposal 2: 2Rx redcap UE measurement capability in NTN shall be same as legacy normal UE in NTN.

Observation 1: RAN2 is discussing TN to NTN (LTE to NR) cell re-selection and there has been not RAN4 RRM impact so far.

Observation 2: RAN4 has already defined the inter-RAT and inter-frequency measurement requirements between TN and NTN in R18 NTN.

Observation 3: Although the measurement requirements are same, the accuracy margin in inter-frequency measurement, 1dB margin is needed for 1Rx UE compared to legacy condition. For inter-RAT measurement, the accuracy margin is the same as original ones.

Proposal 3: The measurement requirements for inter-frequency and inter-RAT measurement requirements in legacy NTN shall be reused for 2Rx and 1Rx RedCap UEs when considering TN to  NTN (NR-NR) and NTN to TN (NR-NR).

Proposal 4: For inter-frequency measurement, RAN4 shall reuse the extra 1dB offset for 1Rx RedCap UE in NR NTN if the legacy requirements are reused.

Proposal 5: For inter-RAT measurement, the orginal accuracy margin shall be maintained for 1Rx RedCap UE in NR NTN if the legacy requirements are reused.

Proposal 6: The cell re-selection intra-frequency and inter-frequency measurement requirements in legacy NTN can be reused for 2Rx and 1Rx RedCap UEs when considering NTN to NTN cell re-selection.

Proposal 7: RAN4 shall define the concrete accuracy margin for 1Rx and 2Rx UE in NTN-NTN scenario. For 1Rx RedCap UE, additional 1dB is needed based on legacy conditions in NTN. For 2Rx RedCap UE, the legacy  conditions in NTN shall be reused.

Observation 4: By comparison interruption time between legacy RedCap UEs in TN and normal UE in NTN, the  Tsearch is impacted by reduced UE Rx branches and the main difference is the values for 1Rx. For 2Rx RedCap UEs, the  Tsearch is same.

Proposal 8: RAN4 shall reuse the legacy 1Rx RedCap Tsearch when considering to define the handover delay for 1Rx RedCap UE in NTN:

- If the target cell is a known intra-frequency cell, Tsearch = 0ms.

- If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 1*Trs ms if UE is capable of 2 Rx antennas; Tsearch = 2*Trs ms if UE is only required to support 1 Rx antenna.

If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 3*Trs ms if UE is capable of 2 Rx antennas; Tsearch = 5*Trs ms if UE is only required to support 1 Rx antenna.

Observation 5: For 2Rx RedCap UE, the time to identify target NR intra-frequency and inter frequency cell is the same as legacy normal UE in TN and also for legacy NTN

Observation 6: For 1Rx RedCap UE, the time to identify target NR intra-frequency and inter frequency cell is different from 2Rx and only one measurement sample is increased.

Proposal 9: For 2Rx RedCap UE, RAN4 shall reuse legacy NTN requirements on time to identify intra-frequency and inter-frequency.

Proposal 10: For 1Rx RedCap UE, RAN4 shall reuse legacy RedCap UE requirements on time to identify intra-frequency and inter-frequency.

Proposal 11: For 2Rx UE in NTN, the time to identify target NR cell for RRC connection release with redirection to NR  in legacy NTN shall be reused.

Proposal 12: For 1Rx UE in NTN, the time to identify target NR cell for RRC connection release with redirection to NR  in legacy NTN shall be the baseline and one measurement sample shall be increased.

Proposal 13: The legacy requirements in NR NTN shall be reused for 1Rx and  2Rx UE in NTN, the conditions for performing the random access procedure for 1Rx in legacy RedCap TN shall be used for 1Rx UE in NTN.

Proposal 14: The legacy NR NTN timing requirements shall be resued when considering to define the requirements on RedCap UEs in NTN scenario.

Observation 7: The legacy NR NTN evaluation period is the same as legacy TN evaluation period.

Observation 8: In RedCap WI, the evaluation period will not change for 2Rx and the evaluation period of 1Rx is extended by factor 2.

Proposal 15: RAN4 shall define the Qout requirements for RedCap UEs in FR1-NTN bands. The evaluation period for 2Rx shall be reused and the evaluation period shall be extended for 1Rx by factor 2. Also, Qin shall be reused.

Observation 9: When defining the CBD evaluation period, there is no difference between 1Rx and 2Rx RedCap UEs.

Proposal 16: The evaluation period of BFD in legacy RedCap for 2Rx shall be reused and the evaluation period of BFD in legacy RedCap shall be extended for 1Rx by factor 2.

Proposal 17: The evaluation period of CBD in legacy NTN shall be used without any change.

Observation 10:  Active BWP switching delay will not be impacted by reduced UE receiving branches and only depends on UE capability or different NR slot length.

Observation 11: For NTN scenario, the maximum slot length is 0.25ms refers to SCS=60kHz in FR1. For slot length is 0.125ms for RedCap UEs in TN, the SCS is 120kHz belongs to FR2 which is out of scope when we define the requirements for RedCap UEs in NTN scenario.

Proposal 18: Legacy BWP switching delay requirements in NTN shall be reused for RedCap UEs in NTN scenario.

Proposal 19: RAN4 shall only study active BWP switch delay on a single CC instead of multiple CCs since CA is not supported in NTN scenario.

Proposal 20:  For active TCI state switch delay and UE specific CBW change, RAN4 shall reuse the legacy NTN requirements for RedCap UEs in NTN scenario.

Observation 12: RAN2 introduced the multiple SMTCs based on time and frequency difference in NTN in R17.

Proposal 21: Multiple SMTCs (2 SMTCs) shall be reused for RedCap UEs in NTN scenario.

Observation 13: The legacy NR NTN requirements and the legacy requirements for  2Rx RedCap UE are the same as legacy requirements. For 1Rx RedCap UE, the time period are relaxed.

Proposal 22: RAN4 shall define time period of PSS/SSS detection and time index detection. The requirements for 1Rx and 2Rx RedCap UE in NTN scenario shall reuse the legacy RedCap requirements.

Observation 14：The measurement period requirements for legacy TN/ legacy RedCap UE/legacy NR NTN are same in TS 38.133.

Proposal 23: The RedCap UEs with 1Rx and 2Rx in NTN shall own the same measurement period requirements as legacy RedCap UE.

Observation 15: The hard satellite switching delay will be impacted by reduced UE receiving branches.

Proposal 24: Considering hard satellite switching delay, Tsearch will not be changed which is 1*Tfirst_ssb ms for 2Rx RedCap UEs. For 1Rx RedCap UEs, Tsearch will be 2*Tfirst_ssb ms.

Proposal 25: Considering soft satellite switching delay, Tsearch will not be changed which is 1*Tfirst_ssb ms for 2Rx RedCap UEs. For 1Rx RedCap UEs, Tsearch will be 2*Tfirst_ssb ms.

Proposal 26: RAN4 shall reuse the legacy passloss reference signal switching delay in NTN for 1Rx and 2Rx RedCap UEs in NTN scenario.

Observation 16: For RACH-less handover compared to RACH-based handover, the main difference is the end point of the handover delay. For RACH-based handover delay, the end point is to start the transmission of the new uplink PRACH. For RACH-less handover delay, the end point is to start the transmission of the new uplink PUSCH. The interruption time is the same as each other.

Proposal 27: If RAN4 defines the RACH-based handover requirements for RedCap UEs in NTN, RAN4 shall reuse the same requirements for RACH-less handover requirements for RedCap UEs in NTN.

Observation 17: Only Tmeasure is impacted by reduced UE receiving branches.

Proposal 28: For conditional handover with L3 measurement, the legacy time period for PSS/SSS detection and time period for time index detection requirements for 1Rx and 2Rx RedCap UEs shall be considered.

Proposal 29: For conditional handover without L3 measurement, the legacy NTN requirements could be considered.

Observation 18: For the logged MDT requirements and RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirements, among them the relative time stamp has different time duration which is values in seconds.

Observation 19: Majority companies do not have strong view to define the related requirements for MDT since the RedCap UEs have low complexity and low cost. Few companies deem that the cell size is large enough for NTN and MDT requirements have been defined in legacy NTN. Thus, MDT shall be defined. However, several companies think that normal UEs can also perform MDT and it is not necessary to study MDT for RedCap UEs. We do not have the strong view on this.

Proposal 30: RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

Observation 20: The UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission.

Observation 21: For 1Rx and 2Rx RedCap UEs, the requirements are same only the accuracy margin is different.

Observation 22: Based on RAN2’s understanding,  the threshold of RSRP change can be up to 34dB.

Proposal 31: CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be:

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)) |
| RSRP2 | (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2 |
| Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |

Proposal 32: Compared to 2Rx UE, extra 1dB margin shall be needed for 1Rx RedCap UEs.

Observation 23: The legacy SDT requirements is the same as the legacy RA requirements for 1Rx and 2Rx UEs

Proposal 33: If the random access for 2Rx and 1Rx RedCap requirements are defined, the RA based SDT requirements shall be reused. The extra 1dB accuracy margin for 1Rx can be reused.

Proposal 34: The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario.

Observation 24: In legacy, 2.56s DRX is not suitable for earth-moving scenario.

Proposal 35: For LEO earth-moving scenario, eDRX shall not be configured for LEO earth-moving cell.

Observation 25: The serving time may be from 5min to 10min for  LEO quasi earth-fixed cell.

Proposal 36: The eRDX could be configured for LEO quasi earth-fixed cell and the value can be up to 10.24s (Tdetect=235.52s).

Proposal 38: RAN4 had better not consider concurrent gaps for RedCap UEs in NTN scenario.

- References

[1] RP-241667，Rel-19 WID NR NTN phase 3.

[2] TS38.133

[3] TS38.304

[4] TS38.331