# 3GPP TSG-RAN WG4 Meeting #113                             R4-2418475

# Orlando, USA, 18th – 22rd November, 2024

Source:  ZTE Corporation, Sanechips

Title:  Discussion on the RRM requirements of AI/ML Beam management

Agenda Item:	7.17.4.2

Document for:	Approval

- Introduction

In RAN plenary #94 meeting, a new SID[1] for Rel-18 was approved to study the AI/ML for NR air interface. The objectives for RAN4 of this SID are listed as follows.

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we will provide some initial views on RRM requirements for beam management from the general requirements framework perspective.

- Discussion

Based on the progresses in the previous meeting, there were several typical use cases incorporated in [2], which are listed below.

Agreement: - Beam management - Spatial-domain DL beam prediction - Temporal DL beam prediction

Beam management case 1 is the spatial-domain DL beam prediction for set A of beams based on measurement result of set B beams which the set B beams is the subset beam of set A, the best beams shall be estimated through AI/ML based BM methods with L1 measurements on a beam subset as the figure shown:

Figure 1 AI beam prediction (Spatial domain)

Another sub use case is beam management case 2 which is the time-domain DL beam prediction for set A of beams based on the historic measurement results of set B of beams. The best beam in the future can be predicted through AI/ML based BM methods based on the beam quality at the current and historical time as the figure shown:

Figure 2 AI beam prediction (Time domain)

In this paper, we mainly provide the analysis on the RRM requirements for model inference, model monitoring other LCM procedures based on RAN1 and RAN2’s progress and agreements for BM case1 and case 2.

2.1 Model inference

For beam prediction, we know that NW-side model and UE-side model shall be both considered, the following figure can be seen [3]:

Figure 3 NW-side model

Figure 4 UE-side model

Based on the above figures, we know that the main difference between NW-side model and UE-side model is that the model inference is located at NW side or UE side.

For NW-side model, the model inference is located at NW-side. gNB sends the downlink beam, UE performs the beam sweeping and receives the related beam. Then, the UE will report the measurement result to gNB which is the model inference input at NW-side model. From my perspective, the time delay from gNB send the downlink beams to UE measurement and report shall not be considered. Since the model inference will be up to gNB implementation. From the following figure the timeline for NW-side model, whatever the cost time for UE to report the measurement to gNB, the gNB shall decide when to start to perform the model inference.

Figure 5 the timeline for NW-side model

We can also see the following RAN2’s agreement:

- There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

It is observed that there is no RAN2’s specification impact associated to gNB-side model inference. Thus, we deem that for NW-side model, the model inference is up to network and there is no RRM impact.

Observation 1: Whatever the cost time for UE to report the measurement to gNB, the gNB shall decide when to start to perform the model inference since the model inference will be up to gNB implementation.

Observation 2: It is observed that there is no RAN2’s specification impact associated to gNB-side model inference.

Proposal 1: From RAN4 perspective, there is no RRM impact for model inference on NW-side and no need to define RRM requirements.

For UE-side model, the model inference is located at UE side. gNB send the downlink beams and UE shall perform the beam sweeping and measurement. Then, the measurement result will directly be the input for model inference. RAN1 is discussing the report contents of inference results for UE-side model and the initial agreements can be seen:

RAN1 agreement: For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output

Thus, we know that after performing the inference, UE shall report the model output (predicted results) to gNB. To my understanding, the timeline from UE measurement to report predicted results to gNB shall be considered as below:

Figure 6 timeline for UE-side model

Two delays shall be considered: (1) model inference delay; (2) UE report delay. If model inference delay is too long, we are not sure the accuracy of model output especially for BM case2 which have the strong relation with timing. If the UE report delay is too long, we are not sure the reliability of the final model output. Thus, the RRM requirements for model inference of UE-side model shall be considered especially the delay requirements.

Observation 3: After performing the inference, UE shall report the model output (predicted results) to gNB.

Observation 4: Two delays shall be considered: (1) model inference delay; (2) UE report delay.

Proposal 2: The RRM requirements for model inference of UE-side model shall be considered especially the delay requirements in order to guarantee the accuracy of inference results.

2.2 Model monitoring

For performance monitoring, two types of models shall also be considered.

For NW-side model, RAN2 has the related agreements as below:

For the network-side model, required network side additional condition is left up to the network implementation

RAN2 will wait for RAN1 for any required UE side additional conditions.

For network-sided model for BM use case, RAN2 confirms that UE inputs for inference at network-sided model will rely on L1 signaling, RAN2 will not further spend time on this aspect.

The gNB is responsible for monitoring its own performance.  RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any

To my understanding, the performance monitoring shall also be up to gNB which is similar to the inference for NW-side model. Since RAN2 deems that the gNB is responsible for monitoring its own performance, which means the gNB-implementation. However, RAN2 is still discussing specifications enhancements associated to gNB-side model monitoring, maybe RAN4 can wait for the progress of RAN1 and RAN2, then we can decide whether the RRM requirements shall be defined or not.

Observation 5: RAN2 is still discussing specifications enhancements associated to gNB-side model monitoring.

Observation 6: RAN2 deems that the gNB is responsible for monitoring its own performance, which means the gNB-implementation.

Proposal 3: RAN4 can wait for the progress of RAN1 and RAN2, then we can decide whether the RRM requirements shall be defined or not.

For UE-side model, RAN1 defines two types of performance monitoring as below:

For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Type 1 performance monitoring: - - Configuration/Signalling from gNB to UE for measurement and/or reporting - - UE may have different operations - - Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - - Indication from NW for UE to do LCM operations - - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type 2 performance monitoring: - - Indication/request/report from UE to gNB for performance monitoring - - Note: The indication/request/report may be not needed in some case(s) - - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - - If it is for UE side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Mechanism that facilitates the UE to detect whether the functionality/model is suitable or no longer suitable

For Type 2 performance monitoring, RAN1 deems that the indication/request/report from UE to gNB for performance monitoring is not needed, that is, the performance monitoring will be up to UE implementation and there is no RRM impact on Type 2 performance monitoring.

For Type 1 performance monitoring, there are two options: (1) NW-side performance monitoring, UE sends the report to gNB; (2) UE-assisted performance monitoring, UE calculates the performance metrics and reports it to gNB. To my understanding, option 1 and option 2 are similar to the inference on UE-side model since this will impact the next steps after performance monitoring, such as fine-tuning.

Observation 7: For Type 2 performance monitoring, RAN1 deems that the indication/request/report from UE to gNB for performance monitoring is not needed.

Observation 8: For Type 1 performance monitoring, there are two options: (1) NW-side performance monitoring, UE sends the report to gNB; (2) UE-assisted performance monitoring, UE calculates the performance metrics and reports it to gNB.

Observation 9: For Type 1 performance monitoring, option 1 and option 2 are similar to the inference on UE-side model.

Proposal 4: The performance monitoring will be up to UE implementation and there is no RRM impact on Type 2 performance monitoring.

Proposal 5: RAN4 shall consider to define the RRM requirements for Type 1 performance monitoring like inference on UE-side if it has.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML:

Observation 1: Whatever the cost time for UE to report the measurement to gNB, the gNB shall decide when to start to perform the model inference since the model inference will be up to gNB implementation.

Observation 2: It is observed that there is no RAN2’s specification impact associated to gNB-side model inference.

Proposal 1: From RAN4 perspective, there is no RRM impact for model inference on NW-side and no need to define RRM requirements.

Observation 3: After performing the inference, UE shall report the model output (predicted results) to gNB.

Observation 4: Two delays shall be considered: (1) model inference delay; (2) UE report delay.

Proposal 2: The RRM requirements for model inference of UE-side model shall be considered especially the delay requirements in order to guarantee the accuracy of inference results.

Observation 5: RAN2 is still discussing specifications enhancements associated to gNB-side model monitoring.

Observation 6: RAN2 deems that the gNB is responsible for monitoring its own performance, which means the gNB-implementation.

Proposal 3: RAN4 can wait for the progress of RAN1 and RAN2, then we can decide whether the RRM requirements shall be defined or not.

Observation 7: For Type 2 performance monitoring, RAN1 deems that the indication/request/report from UE to gNB for performance monitoring is not needed.

Observation 8: For Type 1 performance monitoring, there are two options: (1) NW-side performance monitoring, UE sends the report to gNB; (2) UE-assisted performance monitoring, UE calculates the performance metrics and reports it to gNB.

Observation 9: For Type 1 performance monitoring, option 1 and option 2 are similar to the inference on UE-side model.

Proposal 4: The performance monitoring will be up to UE implementation and there is no RRM impact on Type 2 performance monitoring.

Proposal 5: RAN4 shall consider to define the RRM requirements for Type 1 performance monitoring like inference on UE-side if it has.

- Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- R4-2401570, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated

- R4-2411635，Samsung