3GPP TSG-RAN WG4 Meeting #113	R4-2418499

# Orlando, FL, USA, 18 – 22 November 2024

Source: 	KDDI

Title: 	Discussion on UE Capability and BS Signaling for non-collocated Type 4a4b

Agenda Item:	7.5.2.2

Document for:	Discussion

- 1. Introduction

In RAN#105 meeting, the revised WID for intra-band non-collocated EN-DC/NR-CA is approved [1]. The WI’s core part is focusing on specifying UE RF/RRM requirements for Type-4 UE capability for FWA and clarifying the UE behaviour for Type-4 supported UE. Some discussions were organized in the last RAN4#112bis meeting and the agreements were captured in [2]. We try to solve remaining issues in this contribution.

- 2. Discussion

- 2.1 Working Assumption

According to WID [1], RAN4 need to specify Type-4a/4b capability for FWA. Based on the working assumption previously agreed as WF in RAN4#106 [4], in RAN#103 we propose the following working assumption in Table-1 same as [3].

Table-1. Working assumption for UE architecture

Observation 1: The working assumption of RP-240101 can be reused in RAN4.

- 2.2 Reference UE Architecture

In RAN4#104-bis, UE reference architecture for Type 4b was proposed as Rel-18 in Figure-1 [5], and also it can be reused in Rel-19.

Figure-1. Reference UE architecture

- 2.3 New UE Capability for Type 4a/4b

Regarding UE capability, RAN4 conclude UE Capability for Type 4 in the last meeting as follows.

< Issue 2-1: How to differentiate UE Capability Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC > Agreement: - Legacy capabilities, such as MIMO-CapabilityDL-r10 on E-UTRA carriers < Issue 2-2: UE Capability Type 4a(6Rx/UE) for NR-CA > Agreement: - Do not consider Type 4a for NR-CA

Proposal 1: Specific UE Capability’s design up to RAN2

- 2.4 New BS signaling for Type 4a/4b

In Rel-18, RAN2 designed BS signaling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC in a case of maxMIMO-Layers with value less than or equal to 2. The following description on Rel-18 BS signaling and UE behavior between Type 1 and Type 2 for NR-CA is excerpted from TS 38.331 v18.2.0 section 6.3.2.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

The above Rel-18 BS signaling nonCollocatedTypeNR-CA (nonCollocatedTypeMRDC for EN-DC) to switch between Type 1 and Type 2 is not mentioned for a case with maxMIMO-layers equal to 4, but also in our understanding the case of maxMIMO-layers equal to 4 is collocated scenario Type 1.

Observation 1: Rel-18 BS signaling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC to switch between Type 1 and Type 2 is specified for the case with maxMIMO-layers less than or equal to 2.

Observation 2: In Rel-18, the case of maxMIMO-layers larger than 2 is collocated Type1.

Based on the above Rel-18 BS signaling, we can follow a same direction on Rel-19 BS signaling to switch between Type 1 and Type 4a/4b. So specifically, Rel-19 BS signaling to switch between Type 1 and Type 4a/4b is needed to specify for the case with maxMIMO-layers larger than 2 and less than or equal to 4. Additionally, the case of maxMIMO-layers larger than 4 is collocated scenario Type1.

Proposal 2: Specify Rel-19 BS signaling to switch between Type 1 and Type 4a/4b for the case with maxMIMO-layers larger than 2 and less than or equal to 4. Additionally, the case of maxMIMO-layers larger than 4 is collocated Type 1.

- 2.5 UE behavior/transition between single CC, Type 1, Type 2 and Type 4

Based on the previous observation 1, 2 and the proposal 2, in a case of NR-CA, we can show UE behaviors/transitions between single CC operation, Type 1, Type 2 and Type 4 (4b) with using Rel-19 new BS signaling and maxMIMO-layers as shown in the following table. In the previous RAN4 meetings, companies tried to solve this issue from aspects of each specific values of new/legacy BS signaling, UE capabilities and related RF requirements. On the other hand, in this contribution, firstly we try to promptly show overall from aspects of UE behaviors/transitions between each UE types as follows.

Table 1: UE behavior between single CC, Type 1, Type 2 and Type 4

| UE behavior/transition From→To |  | To |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ①Type 4_Non-colloc_2cc_4L/cc | ②Type 2_Non-colloc_2cc_2L/cc | ③Type 1_Colloc_2cc_4L/cc | ④Type 1_Colloc_2cc_2L/cc | ⑤Single 1cc_4L/cc |
| From | ①Type 4_Non-colloc_2cc_4L/cc | N/A | maxNumberMIMO-Layers=4→2 | Rel-19 new BS signaling=1 | Rel-19 new BS signaling=1 +maxNumberMIMO-Layers=4→2 | RRC Release Scell |
|  | ②Type 2_Non-colloc_2cc_2L/cc | maxNumberMIMO-Layers=2→4 | N/A | Rel-18 NonCollocatedTypeNR-CA +maxNumberMIMO-Layers=2→4 | Rel-18 NonCollocatedTypeNR-CA | RRC Release Scell +maxNumberMIMO-Layers=2→4 |
|  | ③Type 1_Colloc_2cc_4L/cc | Rel-19 new BS signaling=0 | Rel-18 NonCollocatedTypeNR-CA +maxNumberMIMO-Layers=4→2 | N/A | maxNumberMIMO-Layers=4→2 | RRC Release Scell |
|  | ④Type 1_Colloc_2cc_2L/cc | Rel-19 new BS signaling=0 +maxNumberMIMO-Layers=2→4 | Rel-18 NonCollocatedTypeNR-CA | maxNumberMIMO-Layers=2→4 | N/A | RRC Release Scell +maxNumberMIMO-Layers=2→4 |
|  | ⑤Single 1cc_4L/cc | RRC Add Scell=0 +Rel-19 new BS signaling | RRC Add Scell +Rel-18 NonCollocatedTypeNR-CA +maxNumberMIMO-Layers=4→2 | RRC Add Scell | RRC Add Scell +maxNumberMIMO-Layers=4→2 | N/A |

We try to describe details on the above table. First, regarding behaviors between Type 1(collocated) and Type 2(non-collocated), it has been already specified with using nonCollocatedTypeNR-CA and maxMIMO-layers in Rel-18.

Next, regarding behaviors between Type 2 and Type 4 (both non-collocated), we agreed in the RAN4#112 meeting as follows.

Issue 2-3-3: New BS Signaling to switch between Type 4a/4b and Type 2 Agreement: - No new NW signaling needed from Type-4 to Type-2 switch and maxMIMO-layer can be used.

And then, regarding behaviors between Type 1(collocated) and Type 4(non-collocated), based on the previous proposal 2, this transition needs Rel-19 new BS signaling with/without maxMIMO-layers.

Finally, regarding behaviors between single CC and Type 4(non-collocated), new BS signaling is not needed, because Scell Add/Release via RRC can be reused same as a legacy operation. Actually, RAN4 conclude the WF[2] in the last meeting as follows.

Issue 2-5: Switch between Type 4a/4b and single CC operation Agreement: - No new NW signaling needed from Type 4 to single carrier fallback.

- 2.6 UE behavior on initial access

Based on the previous observation 1, 2, UE behavior on initial access for Rel-18 is as follows.

- The case of maxMIMO-layers larger than 2 is Type 1.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

We also think that same direction might be adopted for Rel-19 as follows.

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present is Type 1.

  - Rel-19 New BS signaling is absent is Type 4.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

On the other hand, we need to consider the following two cases raised during offline discussion after the last RAN4 meeting.

- Type 4 capable Rel-19 UE behavior operate under Rel-18 BS

- Only Type 2 capable Rel-19 UE behavior under Rel-19 BS

Especially, regarding above case A, assuming Rel-19 UE indicates both Type 4 and Type 2 capability, then Rel-18 BS only understand Type 2 and use maxMIMO-layers=4 to control UE work under collocated as Type 1 if following above Rel-18 based new BS signaling design, but Rel-19 UE would see "new Rel-19 BS signaling=absent + maxMIMO-layers=4", then this UE misthink Type4. Eventually, discrepancy between Rel-19 UE and Rel-18 BS happens.

So, we propose new BS signaling to control “present with having value 0/1” and “absent” adding to only “present/absent” of Rel-18 BS signaling as follows, so that new BS signaling can solve the discrepancy as above mentioned.

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present with value 0 is Type 4.

  - Rel-19 New BS signaling is present with value 1 is Type 1.

  - Rel-19 New BS signaling is absent is that UE follows Rel-18 specification.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

Proposal 3: Confirm the following understanding on maxMIMO-layers and Rel-18/19 BS signaling:

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present with value 0 is Type 4.

  - Rel-19 New BS signaling is present with value 1 is Type 1.

  - Rel-19 New BS signaling is absent is that UE follows Rel-18 specification.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

- 3. Conclusion

Observation 1: Rel-18 BS signaling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC to switch between Type 1 and Type 2 is specified for the case with maxMIMO-layers less than or equal to 2.

Observation 2: In Rel-18, the case of maxMIMO-layers larger than 2 is collocated Type1.

Proposal 1: Specific UE Capability’s design up to RAN2.

Proposal 2: Specify Rel-19 BS signaling to switch between Type 1 and Type 4a/4b for the case with maxMIMO-layers larger than 2 and less than or equal to 4. Additionally, the case of maxMIMO-layers larger than 4 is collocated Type 1.
Proposal 3: Confirm the following understanding on maxMIMO-layers and Rel-18/19 BS signaling:

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present with value 0 is Type 4.

  - Rel-19 New BS signaling is present with value 1 is Type 1.

  - Rel-19 New BS signaling is absent is that UE follows Rel-18 specification.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

- 4. Reference

[1] RP-230838 Revised WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), KDDI, RAN#105

[2] R4-2417115 WF on rest issues for intra-band non-collocated EN-DC/NR-CA, KDDI, RAN4#112bis

[3] RP-240101 Motivation on Non-collocated ENDC,NRCA in Release 19, KDDI, Docomo, KT, LG Uplus, SoftBank, Telecom Italia, Samsung, RAN#103

[4] R4-2303696 WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI, RAN4#106

[5] R4-2215736, Views on UE RF aspect for non-collocated EN-DC, NR-CA deployment, Samsung, RAN4#104-bis-e

[6] R4-2306605 WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI, RAN4#106-bis-e