3GPP TSG-RAN WG4 Meeting #113		R4-2418535

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.20.3.2

Source:	Apple

Title:   	On UE RRM requirement for SBFD

Document for:	Discussion

# 1	Introduction

In the latest R19 SBFD WI [1], there is RAN4 objective on UE CLI measurement and reporting, as shown below:

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

At the last RAN4 meeting, a WF was agreed [2]. In this contribution, we continue to share our views on the open issues.

# 2	Discussion

Requirements for SSB based measurement

#### Issue 1-2-1: Requirements for SSB based measurement - Agreements (from online session) - Down-select one of the two options: - Option 1: - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements. - Option 2: - For inter-frequency measurement without gap, UL transmission is prioritized. - Follow option 1 in other cases.

As shown above, the main difference between Option 1 and Option 2 is whether inter-frequency measurement without gap should be prioritized over UL transmission or not. To consider how to address this difference, we can turn to the approach in the current specification.

The current scheduling restriction requirement is shown below:

We think for SBFD, the same restriction should apply. In other words, SSB measurement should be prioritized.

Proposal 1: It is proposed to agree that as baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbor cell measurement, i.e. SSB measurement is prioritized over UL transmission.

Requirements for CSI-RS based measurement

#### Issue 1-2-2: Requirements for CSI-RS based measurement - Agreements (from online session) - RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations. - CSI-RS colliding with dynamically scheduled UL transmission - CSI-RS puncturing in frequency domain in SBFD symbols - CSI-RS distributed over SBFD and non-SBFD symbols - Align the understanding whether above SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI. - RAN4 to discuss whether and how L3 CSI-RS measurement requirements would be impacted by SBFD operations, and further RAN1/2 conclusions can be considered.

In case of CSI-RS colliding with dynamically scheduled UL transmission, there is previous RAN1 agreements that says “If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal.” After that, there seems to be no further agreement on the collision handling if CSI-RS resources are aperiodic.

Therefore, before there is further RAN1 agreement, we can agree that in case of non-aperiodic CSI-RS resources colliding with dynamic UL transmission, the measurement occasion can be dropped. To address the dropped measurement occasion, the measurement period should be extended. This is consistent with the approach RAN4 used to address the missing SSB transmission in LBT for unlicensed spectrum.

Proposal 2: In case of non-aperiodic CSI-RS colliding with dynamically scheduled UL transmission, RAN4 can adopt the similar approach as used in unlicensed spectrum, i.e., the measurement occasion is dropped and the measurement period should be extended to account for the dropped measurement occasions.

In case of CSI-RS puncturing in frequency domain in SBFD symbols, RAN4 may need to study if there is any performance difference compared to the case of contiguous CSI-RS resources. For the time being, we need to ensure that at least the same CSI-RS bandwidth as used before should stay unchanged. This is a reasonable assumption since SBFD is usually targeted for TDD band with large channel bandwidth.

Proposal 3: In case of CSI-RS resources puncturing on REs outside DL usable PRBs, it is assumed that CSI-RS bandwidth is 48 PRBs as SBFD is usually targeted for TDD bands with large CBW. FFS if 24 PRBs need to be considered.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: It is proposed to agree that as baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbor cell measurement, i.e. SSB measurement is prioritized over UL transmission.

Proposal 2: In case of non-aperiodic CSI-RS colliding with dynamically scheduled UL transmission, RAN4 can adopt the similar approach as used in unlicensed spectrum, i.e., the measurement occasion is dropped and the measurement period should be extended to account for the dropped measurement occasions.

Proposal 3: In case of CSI-RS resources puncturing on REs outside DL usable PRBs, it is assumed that CSI-RS bandwidth is 48 PRBs as SBFD is usually targeted for TDD bands with large CBW. FFS if 24 PRBs need to be considered.

# 4	References

RP-241614, "Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)," Huawei

R4-2416858, "WF on RRM requirements for NR_duplex_evo," Huawei, HiSilicon