3GPP TSG-RAN WG4 Meeting #113		R4-2418537

Orlando, US, 18th – 22nd November, 2024

Agenda item:	5.10.1

Source:	Apple

Title:   	On RRM core requirement maintenance for NR FR2 multi-Rx chain DL reception

Document for:	Discussion

# 1	Introduction

At RAN4#112, we discussed the issue of when is UE considered to be in multi-RX operation and there was no conclusion. In this contribution, we continue the discussion.

# 2	Discussion

As captured in the moderator summary [1]:

Issue 1-1-1: When is UE considered to be in multi-rx operation - Proposals - Proposal 1: Keep the agreed definition - Given UE indication of its preference of multi-RX/single-RX operation has been allowed, it is OK to take the current definition of multi-RX operation - Proposal 2: Add following additional condition - P2a: - UE sent a recent valid Rel-17 group-based beam reporting (GBBR). - If UE recently reported ‘Not valid’ for one of the RSRP for a beam pair, this means UE is allow to fallback to single panel for the later reception QCL-ed to that beam pair. - P2b: Rel-17 group-based beam reporting (GBBR) is activated/triggered by the network.

There was discussion on whether the previously agreed definition of multi-RX operation should be revised. In the end, the definition was kept unchanged due to the lack of consensus to change. As it stands, the definition is as follows:

- For fast beam sweeping, the UE is in multi-Rx operation if following condition is met:

  - UE is configured with group-based beam reporting (GBBR) report.

We are OK with the current definition. As a result, when the CR was drafted for 38.133 on applicability of requirements multi-Rx operation in FR2-1, the following wording was used (copied below)

To have accurate specification text, we think the above wording needs to be improved for the following reason. RAN4 has agreed to use UAI to allow the UE to indicate its preference of multi-RX or single-RX operation. Furthermore, neither the network behavior nor UE behavior after UAI indication to the network has been defined. Therefore, it is at least unclear if the requirement would apply to the case where the network configures GBBR to the UE and then the UE indicates via UAI that it prefers to fall back from multi-RX operation to single-RX operation.

Observation 1: Neither the network behavior nor UE behavior after UAI indication to the network has been defined. Therefore, it is at least unclear if the requirement would apply to the case where the network configures GBBR to the UE and then the UE indicates via UAI that it prefers to fall back from multi-RX operation to single-RX operation.

As such, we propose to modify the text a bit to ensure there is no misunderstanding about the requirement applicability as Option 1.

Option 1: In “3.6.19       Applicability of requirements for multi-Rx operation in FR2-1”, change the current wording from “The requirements related to the support of [reducedRxBeamNum] is applicable when the network configures the UE with a CSI report containing groupBasedBeamReporting-v1710.” To “The requirements related to the support of [reducedRxBeamNum] is applicable when the network has configured the UE with a CSI report containing groupBasedBeamReporting-v1710 and the UE has not sent UAI indicating its preference of single-RX operation since it was most recently configured with such CSI reporting.”

At the same time, we understand there is concern on the proposed change because there could be complexity associated with testing/verifying when the UE has sent UAI indication. As a result, we propose Option 2:

Option 2: In “3.6.19       Applicability of requirements for multi-Rx operation in FR2-1”, add a note “Note: The requirements related to the support of [reducedRxBeamNum] is not applicable when the UE has sent UAI indicating its preference of single-RX operation since it was most recently configured with such CSI reporting.”

Compared to Option 1, Option 2 should not impact the current requirement nor testing/verification. As the note is not in a table, it is considered informative.

Proposal 1: It is proposed to adopt Option 2, i.e., in “3.6.19       Applicability of requirements for multi-Rx operation in FR2-1”, add a note “Note: The requirements related to the support of [reducedRxBeamNum] is not applicable when the UE has sent UAI indicating its preference of single-RX operation since it was most recently configured with such CSI reporting.”

Option 2 is implemented in the CR [2].

# 3	Conclusions

In this contribution, we make the following proposals.

Observation 1: Neither the network behavior nor UE behavior after UAI indication to the network has been defined. Therefore, it is at least unclear if the requirement would apply to the case where the network configures GBBR to the UE and then the UE indicates via UAI that it prefers to fall back from multi-RX operation to single-RX operation.

Proposal 1: It is proposed to adopt Option 2, i.e., in “3.6.19       Applicability of requirements for multi-Rx operation in FR2-1”, add a note “Note: The requirements related to the support of [reducedRxBeamNum] is not applicable when the UE has sent UAI indicating its preference of single-RX operation since it was most recently configured with such CSI reporting.”

# 4	References

R4-2408000, "Topic summary for [111][203] FR2_multiRx_part1," Moderator (Ericsson)

R4-2418538, "CR on multi-Rx RRM core requirement maintenance," Apple