# 3GPP TSG-RAN WG4 Meeting #113	R4-2418545

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	5.10.2

Source:	Apple

Title:	On PT-RS config for multi-TRP demod requirements in FR2

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#112 CRs related to multi-RX demod requirements were agreed and the specification now captures the requirements for Multi-RX PDSCH demodulation and PMI reporting requirements. In this contribution we propose an update to the PT-RS configuration.

# 2. Discussion

In TS 38.101-4 the PT-RS for the multi-RX FR2 requirements are configured as following:

For multi-DCI transmission scheme (fully overlapping):

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| PTRS | Frequency density (KPT-RS) |  | 2 | 2 |
|  | Time density (LPT-RS) |  | 1 | 1 |
|  | Resource Element Offset |  | 2 | 0 |

For single DCI transmission scheme:

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| PTRS configuration | Frequency density (KPT-RS) |  | 2 | 2 for Test 1-2, Disabled for Test 1-1 |
|  | Time density (LPT-RS) |  | 1 | 1 for Test 1-2, Disabled for Test 1-1 |
|  | Resource Element Offset |  | 2 | 3 for Test 1-2, N/A for Test 1-1 |

The resource element offset configuration along with the DMRS port determines the actual RE for PT-RS transmission.

- The PT-RS RE offset for RE mapping is determined by DMRS port and resource element offset configuration.

For multi-DCI fully overlapping transmission scheme the DMRS ports are 1000 and 1002 from TRP1 and TRP2 respectively. Configuring the same resource element offset would lead to different RE offset for PT-RS.

- Configuring the same resource element offset configuration for the PDSCH with different DMRS ports would result in different PT-RS RE offset for the 2 PDSCH.

Moreover, the PT-RS configuration is a per BWP configuration and the 2 PDSCH cannot have different PT-RS configuration.

- The PT-RS configuration is a per BWP configuration and cannot be configured differently for 2 PDSCH in multi-DCI transmission scheme.

We propose to update the resource element offset configuration for multi-DCI fully overlapping demodulation requirements to be the same for TRP1 and TRP2.

- For multi-DCI fully overlapping demod requirements update the PT-RS configuration to have the same resource element offset for both TRP.

For single DCI transmission scheme, there is only single PDSCH transmission. The current PTRS configuration for test 1-2 suggests that 2 different PT-RS are transmitted from 2 TRPs. For single DCI SDM transmission scheme, the enhancement is to transmit 2 port PT-RS one from each TRP rather than 1 PT-RS from each TRP.

- For single DCI transmission current configuration for test 1-2 configures PT-RS from each TRP.

- For single DCI transmission only 1 PT_RS can be configured for single PDSCH transmission.

- The enhancement for PT-RS with single DCI SDM transmission is to configure 2 port PT-RS  - one port per TRP.

maxNrofPorts The maximum number of DL PTRS ports specified in TS 38.214 [19] (clause 5.1.6.3). 2 PT-RS ports can only be configured for a DL BWP that is configured, as specified in TS 38.214 [19] clause 5.1, with a mode where a single PDSCH has association between the DM-RS ports and the TCI states as defined in TS 38.214 [19] clause 5.1.6.2.

For the test with PT-RS transmission from each TRP, maxNrofPorts should be set to n2.

For the single DCI demod and PMI reporting requirements the following updates to PT-RS configuration are proposed:

- Only single PT-RS configuration for both TRP

- For test 1-2 with PT-RS transmission from each TRP, maxNrofPorts should be set to n2.

- For single-DCI SDM transmission scheme requirements update the PT-RS configuration as (1) Single PT-RS config for both TRP, (2) For test 1-2 with PT-RS transmission from each TRP, maxNrofPorts should be set to n2.

# 3. Conclusion

In this paper, we propose an update to the PT-RS configuration. Our observations and proposals are captured below:

- The PT-RS RE offset for RE mapping is determined by DMRS port and resource element offset configuration.

- Configuring the same resource element offset configuration for the PDSCH with different DMRS ports would result in different PT-RS RE offset for the 2 PDSCH.

- The PT-RS configuration is a per BWP configuration and cannot be configured differently for 2 PDSCH in multi-DCI transmission scheme.

- For multi-DCI fully overlapping demod requirements update the PT-RS configuration to have the same resource element offset for both TRP.

- For single DCI transmission current configuration for test 1-2 configures PT-RS from each TRP.

- For single DCI transmission only 1 PT_RS can be configured for single PDSCH transmission.

- The enhancement for PT-RS with single DCI SDM transmission is to configure 2 port PT-RS  - one port per TRP.

- For single-DCI SDM transmission scheme requirements update the PT-RS configuration as (1) Single PT-RS config for both TRP, (2) For test 1-2 with PT-RS transmission from each TRP, maxNrofPorts should be set to n2.

# Reference