3GPP TSG-RAN WG4 Meeting #113		R4-2418547

Orlando, US, 18th – 22nd November, 2024

Agenda item:	5.17.2

Source:	Apple

Title:	Clarification on default assumptions for MU-MIMO with advanced receiver

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#111 the performance part of the Rel-18 WI on NR demodulation performance enhancements was completed.  In RAN4#112 we had a contribution [1] and way forward [2] that was not agreed. In this contribution we aim to clarify some default assumptions in RAN4 for advanced receiver for MU-MIMO.

# 2. Discussion

For precoding granularity and resource allocation RAN1 has introduced a limitation since Rel-15 on co-scheduled UE(s) on DMRS ports within the same CDM group when narrowband precoding is configured.

In RAN4 discussion, RAN1 limitation and RAN2 NWA signaling the assumption has only focused on PRG granularity=2,4, but there was no discussion on the assumption when precoding granularity is set to wideband.

There is no default assumption when target UE is configured with wideband precoding.

In RAN4#112 in [2] the following options were captured:

When UE is configured with wideband precoding, for advanced receiver for MU-MIMO

- Option 1: Default assumption is that precoding granularity of co-scheduled UE is also wideband

  - Send LS to RAN2 to add additional NWA to update AdvancedReceiver-MU-MIMO-r18 to cover case for wideband precoding

- Option 2: Up to UE implementation - No requirements for advanced receiver with MU-MIMO

  - FFS whether to send LS to RAN2 to update AdvancedReceiver-MU-MIMO-r18

- Other options not precluded

The WF was not agreed as in some companies’ opinion there is an implicit assumption from Rel-15 that wideband precoder is not used with MU-MIMO, and hence RAN1 limitation is only specified for PRG=2 or 4. On the other hand, a few companies also believe that this is not entirely precluded.

- There is no assumption for wideband precoding in RAN1 spec or RAN2 NWA for MU-MIMO.

- In some companies’ opinion there is an implicit assumption from Rel-15 that wideband precoder is not used with MU-MIMO

- In some other companies’ opinion, it is not precluded to have MU-MIMO with wideband precoding.

During the discussion it was encouraged to seek inputs form infra vendors on the possibility of MU-MIMO with wideband precoding. If it is confirmed that the implicit assumption from Rel-15 is valid, then we can use the same assumption as earlier that UE is not expected to be configured with wideband precoder with MU-MIMO, and no requirements with advanced receiver for MU-MIMO are applicable.

If the input from NW vendors is that this scenario cannot be precluded, RAN4 should discuss the default assumption and update the NWA accordingly.

- Seek feedback from NW vendors if implicit assumption from Rel-15 – wideband precoders not used with MU-MIMO scheduling can be used for advanced receivers with MU-MIMO.

- In case the implicit assumption is not valid, RAN4 to discuss updating the default assumption for advanced receiver with MU-MIMO.

# 3. Conclusion

In this paper, we clarify some default assumptions in RAN4 for advanced receiver for MU-MIMO. Our observations and proposals are captured below:

- There is no assumption for wideband precoding in RAN1 spec or RAN2 NWA for MU-MIMO.

- In some companies’ opinion there is an implicit assumption from Rel-15 that wideband precoder is not used with MU-MIMO

- In some other companies’ opinion, it is not precluded to have MU-MIMO with wideband precoding.

- Seek feedback from NW vendors if implicit assumption from Rel-15 – wideband precoders not used with MU-MIMO scheduling can be used for advanced receivers with MU-MIMO.

- In case the implicit assumption is not valid, RAN4 to discuss updating the default assumption for advanced receiver with MU-MIMO.

# Reference

- R4-2411381, “Clarification on default assumptions for MU-MIMO with advanced receiver”, Apple

- R4-2413601, “Way Forward on extension of RAN4 default assumption for MU-MIMO advanced receiver”, Apple