# 3GPP TSG-RAN WG4 Meeting #113	R4-2418548

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	5.25.1

Source:	Apple

Title:	On gradual timing adjustment for 2TA

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#112 RRM core and performance requirements maintenance for R18 MIMO evolution were discussed and way forward [1] was agreed.  In this contribution we present our views on gradual timing adjustment requirements maintenance for 2TA.

# 2. Discussion

## 2 TA with multi-DCI multi-TRP

We have timing error and gradual timing adjustment requirements defined for multi-DCI multi-TRP operation with 2 TA. The gradual timing adjustment requirement in §7.1.2.1 is defined as:

The gradual timing adjust requirements for 2 TA are applicable for each TAG independently. The UE applies gradual timing adjustment until the transmission timing error between UE and the reference timing is within ±Te for each TAG. For multi-DCI multi-TRP operation with 2TA there is also MTTD requirement applicable to the pair of TAGs.

The gradual timing adjustment requirements are applicable per TAG and the MTTD requirements are applicable for the pair of TAGs.

- For multi-DCI multi-TRP operation with 2TA the UE has gradual timing adjust requirements applicable to each TAG.

- For multi-DCI multi-TRP operation with 2TA the UE has MTTD requirements applicable to pair of TAGs.

The UE is expected to meet both the requirements for gradual timing adjustment and MTTD for the pair of TAGs. The gradual timing adjust is applied until the timing error with respect to reference timing is within ±Te for each TAG. At the same time MTTD requirement shall be met. It is possible that with gradual timing adjustment applied on each TAG independently the TTD between the pair of TAGs exceeds MTTD requirement and could impact the UL transmission.

- While the UE is applying gradual timing adjustment for each TAG, the TTD for the pair of TAGs could exceed the MTTD requirement.

There should be a limit on the gradual timing adjustment to ensure that the MTTD is not exceeded. Without the limit the gradual timing adjustment independently on each TAG might impact UL transmission for multi-DCI multi-TRP with 2TA. Hence, we propose to limit the gradual timing adjustment until the TTD between pair of TAGs is within the MTTD requirement.

- For multi-DCI multi-TRP with 2TA the gradual timing adjustment is applied until the timing error for each TAG is within ±Te or the TTD between pair of TAGs doesn’t exceed the MTTD requirement for 2TAs.

Our proposal is captured in our companion CR [2].

There is a similar issue with legacy gradual timing adjustment requirements and MTTD requirements for CA, which can be discussed in our contribution [3].

- Similar issue with legacy gradual timing adjustment and MTTD for CA also needs to be discussed.

# 3. Conclusion

In this paper, we provide our views on gradual timing adjustment requirements maintenance for 2TA. Our observations and proposals are captured below:

2TA with multi-DCI  multi-TRP

- For multi-DCI multi-TRP operation with 2TA the UE has gradual timing adjust requirements applicable to each TAG.

- For multi-DCI multi-TRP operation with 2TA the UE has MTTD requirements applicable to pair of TAGs.

- While the UE is applying gradual timing adjustment for each TAG, the TTD for the pair of TAGs could exceed the MTTD requirement.

- Similar issue with legacy gradual timing adjustment and MTTD for CA also needs to be discussed.

- For multi-DCI multi-TRP with 2TA the gradual timing adjustment is applied until the timing error for each TAG is within ±Te or the TTD between pair of TAGs doesn’t exceed the MTTD requirement for 2TAs.

# Reference

- R4-2414036, “WF on RRM requirements for NR_MIMO_evo_DL_UL”, Samsung.

- R4-2418559, “CR on gradual timing adjustment for 2TA”, Apple.

- R4-2418540, “(NR_newRAT-core) On gradual timing adjustment for DC and CA”, Apple.